CCNA Uncategorized - Multiple Choice (No Images)

These questions are complete and ready for student use

82 Questions

Topic 1: 6  |  Topic 2: 18  |  Topic 3: 6  |  Topic 4: 30  |  Topic 5: 22

Sources: V7.1: 10  |  V80.2: 72

Question 27  (Topic 3)  [V7.1]

Which two components comprise part of a PKI? (Choose two.)

A. preshared key that authenticates connections

B. RSA token

C. CA that grants certificates

D. clear-text password that authenticates connections

E. one or more CRLs

Answer: BC

Question 32  (Topic 3)  [V7.1]

Which characteristic differentiates the concept of authentication from authorization and accounting?

A. user-activity logging

B. service limitations

C. consumption-based billing

D. identity verification

Answer: D

Question 40  (Topic 1)  [V80.2]

Which level of severity must be set to get informational syslogs?

A. alert

B. critical

C. notice

D. debug

Answer: D

Question 66  (Topic 1)  [V80.2]

In software-defined architecture, which plane handles switching for traffic through a Cisco router?

A. Control

B. Management

C. Data

D. Application

Answer: C

Question 144  (Topic 2)  [V7.1]

What are two reasons for an engineer to configure a floating static route? (Choose two)

A. to automatically route traffic on a secondary path when the primary path goes down

B. to route traffic differently based on the source IP of the packet

C. to enable fallback static routing when the dynamic routing protocol fails

D. to support load balancing via static routing

E. to control the return path of traffic that is sent from the router

Answer: AC

Question 175  (Topic 1)  [V80.2]

Which network allows devices to communicate without the need to access the Internet?

A. 172.9.0.0/16

B. 172.28.0.0/16

C. 192.0.0.0/8

D. 209.165.201.0/24

Answer: B

Question 208  (Topic 1)  [V80.2]

What is a recommended approach to avoid co-channel congestion while installing access points that use the 2.4 GHz frequency?

A. different nonoverlapping channels

B. different overlapping channels

C. one overlapping channel

D. one nonoverlapping channel

Answer: A

Question 223  (Topic 2)  [V7.1]

The SW1 interface g0/1 is in the down/down state. Which two configurations are valid reasons for the interface conditions? (Choose two)

A. There is a duplex mismatch

B. There is a speed mismatch

C. There is a protocol mismatch

D. The interface is shut down

E. The interface is error-disabled

Answer: BE

Question 243  (Topic 2)  [V80.2]

A device detects two stations transmitting frames at the same time. This condition occurs after the first 64 bytes of the frame is received. Which interface counter increments?

A. collision

B. CRC

C. runt

D. late collision

Answer: D

Question 259  (Topic 2)  [V80.2]

If a switch port receives a new frame while it is actively transmitting a previous frame, how does it process the frames?

A. The new frame is delivered first, the previous frame is dropped, and a retransmission request is sent.

B. The previous frame is delivered, the new frame is dropped, and a retransmission request is sent.

C. The new frame is placed in a queue for transmission after the previous frame.

D. The two frames are processed and delivered at the same time.

Answer: B

Question 275  (Topic 2)  [V7.1]

What is the path for traffic sent from one user workstation to another workstation on a separate switch in a three-tier architecture model?

A. access - core - distribution - access

B. access - distribution - distribution - access

C. access - core - access

D. access - distribution - core - distribution - access

Answer: D

Question 281  (Topic 2)  [V7.1]

What is the function of a server?

A. It transmits packets between hosts in the same broadcast domain.

B. It provides shared applications to end users.

C. It routes traffic between Layer 3 devices.

D. It creates security zones between trusted and untrusted networks.

Answer: B

Question 282  (Topic 2)  [V80.2]

What are two reasons that cause late collisions to increment on an Ethernet interface? (Choose two)

A. when the sending device waits 15 seconds before sending the frame again

B. when the cable length limits are exceeded

C. when one side of the connection is configured for half-duplex

D. when Carrier Sense Multiple Access/Collision Detection is used

E. when a collision occurs after the 32nd byte of a frame has been transmitted

Answer: B,C

Question 292  (Topic 2)  [V80.2]

The SW1 interface g0/1 is in the down/down state. Which two configurations are valid reasons for the interface conditions? (Choose two)

A. There is a duplex mismatch

B. There is a speed mismatch

C. There is a protocol mismatch

D. The interface is shut down

E. The interface is error-disabled

Answer: B,E

Question 297  (Topic 2)  [V7.1]

Which type of organization should use a collapsed-core architecture?

A. large and requires a flexible, scalable network design

B. large and must minimize downtime when hardware fails

C. small and needs to reduce networking costs currently

D. small but is expected to grow dramatically in the near future

Answer: C

Explanation: A collapsed-core architecture is a limited investment for a small company, and may be efficient and productive for a limited time.

Question 301  (Topic 2)  [V7.1]

An organization secures its network with multi-factor authentication using an authenticator app on employee smartphones. How is the application secured in the case of a user's smartphone being lost or stolen?

A. The application requires an administrator password to reactivate after a configured interval.

B. The application requires the user to enter a PIN before it provides the second factor.

C. The application challenges a user by requiring an administrator password to reactivate when the smartphone is rebooted.

D. The application verifies that the user is in a specific location before it provides the second factor.

Answer: B

Question 333  (Topic 2)  [V80.2]

Which set of actions satisfy the requirement for multifactor authentication?

A. The user swipes a key fob, then clicks through an email link

B. The user enters a user name and password, and then clicks a notification in an authentication app on a mobile device

C. The user enters a PIN into an RSA token, and then enters the displayed RSA key on a login screen

D. The user enters a user name and password and then re-enters the credentials on a second screen

Answer: B

Question 365  (Topic 2)  [V80.2]

An organization secures its network with multi-factor authentication using an authenticator app on employee smartphones. How is the application secured in the case of a user's smartphone being lost or stolen?

A. The application requires an administrator password to reactivate after a configured interval.

B. The application requires the user to enter a PIN before it provides the second factor.

C. The application challenges a user by requiring an administrator password to reactivate when the smartphone is rebooted.

D. The application verifies that the user is in a specific location before it provides the second factor.

Answer: B

Question 377  (Topic 2)  [V80.2]

Which type of organization should use a collapsed-core architecture?

A. large and requires a flexible, scalable network design

B. large and must minimize downtime when hardware fails

C. small and needs to reduce networking costs currently

D. small but is expected to grow dramatically in the near future

Answer: C

Question 402  (Topic 2)  [V80.2]

What are two reasons for an engineer to configure a floating static route? (Choose two)

A. to automatically route traffic on a secondary path when the primary path goes down

B. to route traffic differently based on the source IP of the packet

C. to enable fallback static routing when the dynamic routing protocol fails

D. to support load balancing via static routing

E. to control the return path of traffic that is sent from the router

Answer: A,C

Question 412  (Topic 2)  [V80.2]

Using direct sequence spread spectrum, which three 2.4-GHz channels are used to limit collisions?

A. 1, 6, 11

B. 1, 5, 10

C. 1, 2, 3

D. 5, 6, 7

Answer: A

Question 416  (Topic 2)  [V80.2]

Which goal is achieved by the implementation of private IPv4 addressing on a network?

A. provides an added level of protection against Internet exposure

B. provides a reduction in size of the forwarding table on network routers

C. allows communication across the Internet to other private networks

D. allows servers and workstations to communicate across public network boundaries

Answer: A

Question 426  (Topic 2)  [V80.2]

What is the function of a server?

A. It transmits packets between hosts in the same broadcast domain.

B. It provides shared applications to end users.

C. It routes traffic between Layer 3 devices.

D. It creates security zones between trusted and untrusted networks.

Answer: B

Question 434  (Topic 2)  [V80.2]

What is the path for traffic sent from one user workstation to another workstation on a separate switch in a three-tier architecture model?

A. access - core - distribution - access

B. access - distribution - distribution - access

C. access - core - access

D. access - distribution - core - distribution - access

Answer: D

Question 447  (Topic 3)  [V80.2]

What is the function of the controller in a software-defined network?

A. multicast replication at the hardware level

B. fragmenting and reassembling packets

C. making routing decisions

D. forwarding packets

Answer: C

Question 474  (Topic 1)  [V7.1]

Which level of severity must be set to get informational syslogs?

A. alert

B. critical

C. notice

D. debug

Answer: C

Question 483  (Topic 3)  [V80.2]

Which characteristic differentiates the concept of authentication from authorization and accounting?

A. user-activity logging

B. service limitations

C. consumption-based billing

D. identity verification

Answer: D

Question 529  (Topic 3)  [V80.2]

An engineer must configure R1 for a new user account. The account must meet these requirements:

* It must be configured in the local database.

* The username is engineer.

* It must use the strongest password configurable.

Which command must the engineer configure on the router?

A. R1(config)# username engineer2 algorithm-type scrypt secret test2021

B. R1(config)# username engineer2 secret 5 password S1$b1Ju$kZbBS1Pyh4QzwXyZ

C. R1(config)# username engineer2 privilege 1 password 7 test2021

D. R1(config)# username engineer2 secret 4 S1Sb1Ju$kZbBS1Pyh4QzwXyZ

Answer: B

Question 532  (Topic 3)  [V80.2]

Which two components comprise part of a PKI? (Choose two.)

A. preshared key that authenticates connections

B. RSA token

C. CA that grants certificates

D. clear-text password that authenticates connections

E. one or more CRLs

Answer: B,C

Question 570  (Topic 4)  [V80.2]

What is the PUT method within HTTP?

A. It is a read-only operation.

B. It is a nonidempotent operation.

C. It replaces data at the destination.

D. It displays a web site.

Answer: D

Question 588  (Topic 4)  [V80.2]

When should an engineer implement a collapsed-core architecture?

A. for small networks with minimal need for growth

B. the access and distribution layers must be on the same device

C. for large networks that are connected to multiple remote sites

D. only when using VSS technology

Answer: C

Question 605  (Topic 4)  [V80.2]

Which type of encryption does WPA1 use for data protection?

A. AES

B. TKIP

C. PEAP

D. EAP

Answer: B

Question 609  (Topic 1)  [V7.1]

Which technology allows for multiple operating systems to be run on a single host computer?

A. virtual routing and forwarding

B. network port ID visualization

C. virtual device contexts

D. server virtualization

Answer: D

Question 671  (Topic 4)  [V80.2]

Which type of IPv4 address type helps to conserve the globally unique address classes?

A. multicast

B. private

C. loopback

D. public

Answer: B

Question 684  (Topic 4)  [V80.2]

What is the function of a controller in a software-defined network?

A. multicast replication at the hardware level

B. forwarding packets

C. fragmenting and reassembling packets

D. setting packet-handling policies

Answer: B

Question 721  (Topic 4)  [V80.2]

What is a reason why an administrator would choose to implement an automated network management approach?

A. Reduce inconsistencies in the network configuration.

B. Enable "box by box" configuration and deployment.

C. Decipher simple password policies.

D. Increase recurrent management costs.

Answer: A

Question 733  (Topic 4)  [V80.2]

What is a zero-day exploit?

A. It is when a new network vulnerability is discovered before a fix is available.

B. It is when the perpetrator inserts itself in a conversation between two parties and captures or alters data.

C. It is when the network is saturated with malicious traffic that overloads resources and bandwidth.

D. It is when an attacker inserts malicious code into a SQL server.

Answer: A

Question 740  (Topic 4)  [V80.2]

Which plane is centralized in software-defined networking?

A. application

B. services

C. control

D. data

Answer: C

Question 745  (Topic 4)  [V80.2]

Which authentication method requires the user to provide a physical attribute to authenticate successfully?

A. password

B. multifactor

C. biometric

D. certificate

Answer: A

Question 803  (Topic 4)  [V80.2]

What determines the sequence in which materials are planned during the material requirements planning (MRP) run?

A. The control parameters of the MRP run

B. The creation date of the materials

C. The low-level code of the materials

D. The replenishment lead time of the materials

Answer: C

Question 809  (Topic 4)  [V80.2]

Which set of actions satisfies the requirement for multifactor authentication?

A. The user enters a user name and password, and then clicks a notification in an authentication app on a mobile device.

B. The user swipes a key fob, then clicks through an email link.

C. The user enters a PIN into an RSA token, and then enters the displayed RSA key on a login screen.

D. The user enters a user name and password, and then re-enters the credentials on a second screen.

Answer: A

Question 821  (Topic 4)  [V80.2]

Which two statements distinguish authentication from accounting? (Choose two.)

A. Only authentication records the duration of a user's connection.

B. Only authentication supports user-activity audits.

C. Only authentication provides supporting information for billing users.

D. Only authentication challenges users for their credentials and returns a response.

E. Only authentication validates "who you are."

Answer: D,E

Question 825  (Topic 4)  [V80.2]

What must be considered before deploying virtual machines?

A. location of the virtual machines within the data center environment

B. whether to leverage VSM to map multiple virtual processors to two or more virtual machines

C. resource limitations, such as the number of CPU cores and the amount of memory

D. support for physical peripherals, such as monitors, keyboards, and mice

Answer: C

Question 837  (Topic 4)  [V80.2]

What are two benefits for using private IPv4 addressing? (Choose two.)

A. They alleviate the shortage of public IPv4 addresses.

B. They supply redundancy in the case of failure.

C. They offer Internet connectivity to endpoints on private networks.

D. They allow for Internet access from IoT devices.

E. They provide a layer of security from Internet threats.

Answer: A,E

Question 895  (Topic 4)  [V80.2]

What is a characteristic of RSA?

A. It uses preshared keys for encryption

B. It requires both sides to have identical keys

C. It is a private-key encryption algorithm

D. It is a public-key cryptosystem

Answer: D

Question 902  (Topic 4)  [V80.2]

Which two server types support domain name to IP address resolution? (Choose two.)

A. ESX host

B. resolver

C. web

D. file transfer

E. authentication

Answer: A,C

Question 910  (Topic 4)  [V80.2]

Which alternative to password authentication is implemented to allow enterprise devices to log in to the corporate network?

A. magic links

B. one-time passwords

C. digital certificates

D. 90-day renewal policies

Answer: C

Question 915  (Topic 4)  [V80.2]

Which two features introduced in SNMPv2 provide the ability to retrieve large amounts of data in one request?

A. Get

B. GetNext

C. Set

D. GetBulk

E. Inform

Answer: A,D

Question 922  (Topic 4)  [V80.2]

What is the temporary state that switch ports always enter immediately after the boot process when Rapid PVST+ is used?

A. discarding

B. listening

C. forwarding

D. learning

Answer: A

Question 950  (Topic 4)  [V80.2]

Which two practices are recommended for an acceptable security posture in a network? (Choose two)

A. Backup device configurations to encrypted USB drives for secure retrieval

B. Maintain network equipment in a secure location

C. Use a cryptographic keychain to authenticate to network devices

D. Place internal email and file servers in a designated DMZ

E. Disable unused or unnecessary ports, interfaces and services

Answer: C,E

Question 972  (Topic 4)  [V80.2]

By default, how long will the switch continue to know a workstation MAC address after the workstation stops sending traffic?

A. 200 seconds

B. 300 seconds

C. 600 seconds

D. 900 seconds

Answer: B

Question 975  (Topic 4)  [V80.2]

What is the primary purpose of private address space?

A. conserve globally unique address space

B. simplify the addressing in the network

C. limit the number of nodes reachable via the Internet

D. reduce network complexity

Answer: D

Question 991  (Topic 4)  [V80.2]

Which action must be taken when password protection is implemented?

A. Use less than eight characters in length when passwords are complex.

B. Store passwords as contacts on a mobile device with single-factor authentication.

C. Include special characters and make passwords as long as allowed.

D. Share passwords with senior IT management to ensure proper oversight.

Answer: D

Question 998  (Topic 4)  [V80.2]

What is a reason to implement IPv4 private addressing?

A. Reduce the risk of a network security breach

B. Comply with PCI regulations

C. Comply with local law

D. Reduce the size of the forwarding table on network routers

Answer: D

Question 1019  (Topic 4)  [V80.2]

Which group of channels in the 802.11b/g/n/ac/ax 2.4 GHz frequency bands are nonoverlapping channels?

A. channels 1, 5, and 10

B. channels 1, 6, and 11

C. channels 1, 5, and 11

D. channels 1, 6, and 10

Answer: B

Question 1020  (Topic 4)  [V80.2]

Which signal frequency appears 60 times per minute?

A. 1 Hz signal

B. 1 GHz signal

C. 60 Hz signal

D. 60 GHz signal

Answer: B

Question 1028  (Topic 4)  [V80.2]

Company has decided to require multifactor authentication for all systems. Which set of parameters meets the requirement?

A. personal 10-digit PIN and RSA certificate

B. complex password and personal 10-digit PIN

C. password of 8 to 15 characters and personal 12-digit PIN

D. fingerprint scanning and facial recognition

Answer: A

Question 1036  (Topic 4)  [V80.2]

What are two examples of multifactor authentication? (Choose two.)

A. single sign-on

B. unique user knowledge

C. passwords that expire

D. soft tokens

E. shared password responsibility

Answer: B,D

Question 1053  (Topic 4)  [V80.2]

Which group of channels in the 802.11b/g/n/ax 2.4 GHz frequency bands are non-overlapping channels?

A. channels 1, 5, and 10

B. channels 1, 5, and 11

C. channels 1, 6, and 10

D. channels 1, 6, and 11

Answer: D

Question 1056  (Topic 4)  [V80.2]

What is a functionality of the control plane in the network?

A. It exchanges topology information with other routers.

B. It forwards traffic to the next hop.

C. It looks up an egress interface in the forwarding information base.

D. It provides CLI access to the network device.

Answer: A

Question 1059  (Topic 5)  [V80.2]

What is a characteristic of a Layer 2 switch?

A. Uses routers to create collision domains

B. Responsible for sending data in a particular sequence

C. Avoids MAC address storage for faster transmission

D. Uses the data link layer for communications

Answer: D

Question 1073  (Topic 5)  [V80.2]

How is AI used to identify issues within network traffic?

A. It enhances data packet delivery speeds

B. It analyzes patterns for anomaly detection

C. It exclusively predicts device malfunctions

D. It guarantees zero packet loss in the network

Answer: B

Question 1077  (Topic 5)  [V80.2]

Which feature of Digest Authentication prevents credentials from being sent in plaintext?

A. SSL/TLS encryption

B. Challenge-response mechanism

C. Token-based authorization

D. Public key infrastructure

Answer: B

Question 1084  (Topic 5)  [V80.2]

What is a characteristic of a Layer 2 switch?

A. Provides a single broadcast domain for all connected devices

B. Performs packet fragmentation and reassembly

C. Tracks the number of active TCP connections

D. Uses the data link layer for communications

Answer: D

Question 1092  (Topic 5)  [V80.2]

Which interface is used to send traffic to the destination network?

O 10.76.170.161/26 [110/102] via F0/17

O 10.76.170.161/26 [110/27831] via F0/20

R 10.76.170.161/26 [120/15] via F0/8

R 10.76.170.161/26 [120/10] via F0/12

A. F0/8

B. F0/20

C. F0/12

D. F0/17

Answer: B

Question 1094  (Topic 5)  [V80.2]

What is a characteristic of a Layer 2 switch?

A. tracks the number of active TCP connections

B. transmits exclusively at half duplex

C. offers link bundling to servers

D. uses routers to create collision domains

Answer: C

Question 1096  (Topic 5)  [V80.2]

Which AP mode wirelessly connects two separate network segments each set up within a different campus building?

A. mesh

B. local

C. bridge

D. point-to-point

Answer: C

Question 1106  (Topic 5)  [V80.2]

What is a characteristic of a Layer 2 switch?

A. performs packet fragmentation and reassembly

B. tracks the number of active TCP connections

C. provides a single broadcast domain for all connected devices

D. uses the data link layer for communications

Answer: D

Question 1121  (Topic 5)  [V80.2]

Which role do predictive AI models play in network load balancing?

A. They anticipate future traffic spikes.

B. They assign IP addresses to devices.

C. They select correct cabling types for deployment.

D. They solely monitor historical traffic volumes.

Answer: A

Question 1141  (Topic 5)  [V80.2]

Which interface is used to send traffic to the destination network?

* 10.174.208.210/28 [110/2708] via G0/21

* 10.174.208.210/28 [110/25445] via G0/20

* 10.174.208.210/28 [120/1] via G0/12

* 10.174.208.210/28 [120/4] via G0/16

A. G0/12

B. G0/20

C. G0/21

D. G0/16

Answer: C

Question 1144  (Topic 5)  [V80.2]

Which interface is used to send traffic to the destination network?

* 10.90.207.87/26 [110/1912] via F0/7

* 10.90.207.87/26 [110/28968] via F0/6

* 10.90.207.87/26 [120/14] via F0/4

* 10.90.207.87/26 [120/11] via F0/5

A. F0/7

B. F0/5

C. F0/4

D. F0/6

Answer: A

Question 1145  (Topic 5)  [V80.2]

What is the function of generative AI in network operations?

A. It disables unused services.

B. It deploys network firmware updates.

C. It creates synthetic network configurations.

D. It computes optimal data storage solutions.

Answer: C

Question 1146  (Topic 5)  [V80.2]

Which statement describes virtual machines?

A. They include a guest OS and the service.

B. They enable the network to become agile and hardware-centric.

C. They use a supervisor to provide management for services.

D. They facilitate local management of infrastructure devices.

Answer: A

Question 1156  (Topic 5)  [V80.2]

Which interface is used to send traffic to the destination network?

O 10.18.75.113/27 [110/6906] via G0/6

O 10.18.75.113/27 [110/23018] via G0/3

R 10.18.75.113/27 [120/16] via G0/16

R 10.18.75.113/27 [120/14] via G0/23

A. G0/23

B. G0/3

C. G0/16

D. G0/6

Answer: D

Question 1163  (Topic 5)  [V80.2]

How is AI used to identify issues within network traffic?

A. It exclusively predicts device malfunctions.

B. It enhances data packet delivery speeds.

C. It simplifies traffic route mapping.

D. It analyzes patterns for anomaly detection.

Answer: D

Question 1181  (Topic 5)  [V80.2]

Which interface is used to send traffic to the destination network?

* 10.249.210.56/25 [90/6144] via G0/15

* 10.249.210.56/25 [90/45053] via G0/13

* 10.249.210.56/25 [110/3693] via G0/16

* 10.249.210.56/25 [110/360] via G0/12

A. G0/16

B. G0/15

C. G0/13

D. G0/12

Answer: B

Question 1182  (Topic 5)  [V80.2]

Why would a network administrator choose to implement RFC 1918 address space?

A. to route traffic on the internet

B. to provide flexibility in the IP network design

C. to provide overlapping address space with another network

D. to limit the number of hosts on the network

Answer: B

Question 1192  (Topic 5)  [V80.2]

Which security element uses a combination of one-time passwords, a login name, and a personal smartphone?

A. software-defined segmentation

B. multifactor authentication

C. attribute-based access control

D. rule-based access control

Answer: B

Question 1209  (Topic 5)  [V80.2]

What is a function of Layer 3 switches?

A. They route traffic between devices in different VLANs.

B. They forward Ethernet frames between VLANs using only MAC addresses.

C. They move frames between endpoints limited to IP addresses.

D. They transmit broadcast traffic when operating in Layer 3 mode exclusively.

Answer: C

Question 1218  (Topic 5)  [V80.2]

What is the total number of users permitted to simultaneously browse the controller management pages when using the AireOS GUI?

A. 2

B. 5

C. 8

D. 9

Answer: C

Question 1223  (Topic 5)  [V80.2]

What is the main capability of multifactor authentication?

A. Identifying permissions for end users using three authentication factors

B. Authenticating and authorizing end users using two authentication factors

C. Confirming end-user identity using two or more authentication factors

D. Verifying end-user access permissions using two authentication factors

Answer: C

Question 1225  (Topic 5)  [V80.2]

Which combination of methods satisfies the minimum security requirements when a new multifactor authentication solution is deployed?

A. password of 8 to 15 characters and personal 12-digit PIN

B. authorized USB dongle and mobile phone

C. fingerprint scanning and facial recognition

D. complex password and time-based one-time password

Answer: D