# CCNA 200-301
ARP

Address Resolution Protocol

25 Questions

Multiple Choice: 24 | Drag & Drop: 1

Sources: V7.1 (7) | V80.2 (18)

Topic 1: 14 | Topic 2: 5 | Topic 4: 5 | Topic 5: 1

## Question 252  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

Only four switches are participating in the VLAN spanning-tree

process. Branch-1 priority 614440

Branch-2: priority 39082416

Branch-3: priority 0 Branch-4: root primary

Which switch becomes the permanent root bridge for VLAN 5?

A.  Branch-1

B.  Branch-2

C.  Branch-3

D.  Branch-4

Answer: C

Explanation:

Dynamic ARP inspection is an ingress security feature; it does not

perform any egress checking.

──────────────────────────────────────────────────

## Question 304  (Topic 2)  —  Multiple Choice  [V7.1]

What are two recommendations for protecting network ports from being

exploited when located in an office space outside of an IT closer?

(Choose two.)

A.  enable the PortFast feature on ports

B.  implement port-based authentication

C.  configure static ARP entries

D.  configure ports to a fixed speed

E.  shut down unused ports

Answer: BE

──────────────────────────────────────────────────

## Question 344  (Topic 1)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 1)

[See Exhibit Image]Drag and drop the threat-mitigation techniques from

the left onto the types of threat or attack they mitigate on the

right.

A.  Mastered

B.  Not Mastered

Answer: A

Explanation:

[See Exhibit Image]Double-Tagging attack:

In this attack, the attacking computer generates frames with two

802.1Q tags. The first tag matches the native VLAN of the trunk port

(VLAN 10 in this case), and the second matches the VLAN of a host it

wants to attack (VLAN 20).When the packet from the attacker reaches

Switch A, Switch A only sees the first VLAN 10 and it matches with its

native VLAN 10 so this VLAN tag is removed. Switch A forwards the

frame out all links with the same native VLAN 10. Switch B receives

the frame with an tag of VLAN 20 so it removes this tag and forwards

out to the Victim computer.Note: This attack only works if the trunk

(between two switches) has the same native VLAN as the attacker.To

mitigate this type of attack, you can use VLAN access control lists

(VACLs, which applies to all traffic within a VLAN. We can use VACL to

drop attacker traffic to specific victims/servers) or implement

Private VLANs.ARP attack (like ARP poisoning/spoofing) is a type of

attack in which a malicious actor sends falsified ARP messages over a

local area network as ARP allows a gratuitous reply from a host even

if an ARP request was not received. This results in the linking of an

attacker's MAC address with the IP address of a legitimate computer or

server on the network. This is an attack based on ARP which is at

Layer 2.Dynamic ARP inspection (DAI) is a security feature that

validates ARP packets in a network which can be used to mitigate this

type of attack.

──────────────────────────────────────────────────

## Question 403  (Topic 1)  —  Multiple Choice  [V7.1]

Which type of attack can be mitigated by dynamic ARP inspection?

A.  worm

B.  malware

C.  DDoS

D.  man-in-the-middle

Answer: D

──────────────────────────────────────────────────

## Question 441  (Topic 1)  —  Multiple Choice  [V7.1]

A network engineer must back up 20 network router configurations

globally within a customer environment. Which protocol allows the

engineer to perform this function using the Cisco IOS MIB?

A.  CDP

B.  SNMP

C.  SMTP

D.  ARP

Answer: B

Explanation:

SNMP is an application-layer protocol that provides a message format

for communication between SNMP managers and agents. SNMP provides a

standardized framework and a common language used for the monitoring

and management of devices in a network.The SNMP framework has three

parts:+ An SNMP manager+ An SNMP agent+ A Management Information Base

(MIB)The Management Information Base (MIB) is a virtual information

storage area for network management information, which consists of

collections of managed objects.With SNMP, the network administrator

can send commands to multiple routers to do the backup

──────────────────────────────────────────────────

## Question 477  (Topic 1)  —  Multiple Choice  [V7.1]

What is the benefit of using FHRP?

A.  reduced management overhead on network routers

B.  balancing traffic across multiple gateways in proportion to their

loads

C.  higher degree of availability

D.  reduced ARP traffic on the network

Answer: C

──────────────────────────────────────────────────

## Question 586  (Topic 1)  —  Multiple Choice  [V7.1]

If a notice-level messaging is sent to a syslog server, which event

has occurred?

A.  A network device has restarted

B.  An ARP inspection has failed

C.  A routing instance has flapped

D.  A debug operation is running

Answer: C

Explanation:

Usually no action is required when a route flaps so it generates the

notification syslog level message (level 5).

──────────────────────────────────────────────────

## Question 18  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

PC1 is trying to ping PC3 for the first time and sends out an ARP to S1 Which action is taken by S1?

A.  It forwards it out G0/3 only

B.  It is flooded out every port except G0/0.

C.  It drops the frame.

D.  It forwards it out interface G0/2 only.

Answer: B

──────────────────────────────────────────────────

## Question 21  (Topic 1)  —  Multiple Choice  [V80.2]

A network engineer must back up 20 network router configurations globally within a customer environment.

Which protocol allows the engineer to perform this function using the Cisco IOS MIB?

A.  CDP

B.  SNMP

C.  SMTP

D.  ARP

Answer: B

──────────────────────────────────────────────────

## Question 23  (Topic 1)  —  Multiple Choice  [V80.2]

What is a practice that protects a network from VLAN hopping attacks?

A.  Enable dynamic ARP inspection

B.  Configure an ACL to prevent traffic from changing VLANs

C.  Change native VLAN to an unused VLAN ID

D.  Implement port security on internet-facing VLANs

Answer: C

──────────────────────────────────────────────────

## Question 24  (Topic 1)  —  Multiple Choice  [V80.2]

Which access layer threat-mitigation technique provides security based on identity?

A.  Dynamic ARP Inspection

B.  using a non-default native VLAN

C.  802.1x

D.  DHCP snooping

Answer: C

──────────────────────────────────────────────────

## Question 58  (Topic 1)  —  Multiple Choice  [V80.2]

How are VLAN hopping attacks mitigated?

A.  enable dynamic ARP inspection

B.  manually implement trunk ports and disable DTP

C.  activate all ports and place in the default VLAN

D.  configure extended VLANs

Answer: B

──────────────────────────────────────────────────

## Question 176  (Topic 1)  —  Multiple Choice  [V80.2]

If a notice-level messaging is sent to a syslog server, which event has occurred?

A.  A network device has restarted

B.  An ARP inspection has failed

C.  A routing instance has flapped

D.  A debug operation is running

Answer: C

──────────────────────────────────────────────────

## Question 182  (Topic 1)  —  Multiple Choice  [V80.2]

Which type of attack can be mitigated by dynamic ARP inspection?

A.  worm

B.  malware

C.  DDoS

D.  man-in-the-middle

Answer: D

──────────────────────────────────────────────────

## Question 213  (Topic 1)  —  Multiple Choice  [V80.2]

Which protocol does an IPv4 host use to obtain a dynamically assigned IP address?

A.  ARP

B.  DHCP

C.  CDP

D.  DNS

Answer: B

──────────────────────────────────────────────────

## Question 233  (Topic 1)  —  Multiple Choice  [V80.2]

What is the benefit of using FHRP?

A.  reduced management overhead on network routers

B.  balancing traffic across multiple gateways in proportion to their loads

C.  higher degree of availability

D.  reduced ARP traffic on the network

Answer: C

──────────────────────────────────────────────────

## Question 250  (Topic 2)  —  Multiple Choice  [V80.2]

What are two recommendations for protecting network ports from being exploited when located in an office

space outside of an IT closer? (Choose two.)

A.  enable the PortFast feature on ports

B.  implement port-based authentication

C.  configure static ARP entries

D.  configure ports to a fixed speed

E.  shut down unused ports

Answer: B,E

──────────────────────────────────────────────────

## Question 355  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What is the effect of this configuration?

A.  The switch port interface trust state becomes untrusted

B.  The switch port remains administratively down until the interface is connected to another switch

C.  Dynamic ARP inspection is disabled because the ARP ACL is missing

D.  The switch port remains down until it is configured to trust or untrust incoming packets

Answer: A

──────────────────────────────────────────────────

## Question 388  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What is the effect of this configuration?

A.  All ARP packets are dropped by the switch

B.  Egress traffic is passed only if the destination is a DHCP server.

C.  All ingress and egress traffic is dropped because the interface is untrusted

D.  The switch discard all ingress ARP traffic with invalid MAC-to-IP address bindings.

Answer: D

──────────────────────────────────────────────────

## Question 590  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Router-WAN1 has a new connection via Gi0/0 to the ISP. Users running the web

applications indicate that connectivity is unstable to the internet. What is causing the interface issue?

A.  Small frames less than 64 bytes are rejected due to size.

B.  The receive buffer is full due to a broadcast storm.

C.  Frames are discarded due to a half-duplex negotiation.

D.  Broadcast packets are rejected because ARP timeout is enabled.

Answer: C

──────────────────────────────────────────────────

## Question 765  (Topic 4)  —  Multiple Choice  [V80.2]

What is the benefit of using FHRP (First Hop Redundancy Protocol)?

A.  Reduced ARP traffic on the network

B.  Balancing traffic across multiple gateways in proportion to their loads

C.  Reduced management overhead on network routers

D.  Higher degree of availability

Answer: D

──────────────────────────────────────────────────

## Question 879  (Topic 4)  —  Multiple Choice  [V80.2]

A switch is a forwarding a frame out of an interfaces except the interface that received the frame. What is

the technical term for this process?

A.  ARP

B.  CDP

C.  flooding

D.  multicast

Answer: C

──────────────────────────────────────────────────

## Question 1039  (Topic 4)  —  Multiple Choice  [V80.2]

A switch is forwarding a frame out of all interfaces except the interface that received the frame. What is the

technical term for this process?

A.  ARP

B.  Multicast

C.  CDP

D.  Flooding

Answer: D

──────────────────────────────────────────────────

## Question 1045  (Topic 4)  —  Multiple Choice  [V80.2]

Why would VRRP be implemented when configuring a new subnet in a multivendor environment?

A.  when a gateway protocol is required that support more than two Cisco devices for redundancy

B.  to enable normal operations to continue after a member failure without requiring a change In a host ARP

cache

C.  to ensure that the spanning-tree forwarding path to the gateway is loop-free

D.  to interoperate normally with all vendors and provide additional security features for Cisco devices

Answer: A

──────────────────────────────────────────────────

## Question 1087  (Topic 5)  —  Multiple Choice  [V80.2]

Which two transport layer protocols carry syslog messages? (Choose two.)

A.  UDP

B.  RTP

C.  TCP

D.  IP

E.  ARP

Answer: A,C