# CCNA 200-301
CDP and LLDP

Cisco Discovery Protocol and LLDP

32 Questions

Multiple Choice: 32 | Drag & Drop: 0

Sources: V7.1 (8) | V80.2 (24)

Topic 1: 11 | Topic 2: 9 | Topic 3: 2 | Topic 4: 9 | Topic 5: 1

## Question 81  (Topic 2)  —  Multiple Choice  [V7.1]

Which protocol does an access point use to draw power from a connected

switch?

A.  Internet Group Management Protocol

B.  Adaptive Wireless Path Protocol

C.  Cisco Discovery Protocol

D.  Neighbor Discovery Protocol

Answer: C

──────────────────────────────────────────────────

## Question 231  (Topic 2)  —  Multiple Choice  [V7.1]

What Is a syslog facility?

A.  Host that is configured for the system to send log messages

B.  password that authenticates a Network Management System to receive

log messages

C.  group of log messages associated with the configured severity level

D.  set of values that represent the processes that can generate a log

message

Answer: C

Explanation:

Cisco Community -- Difference between logging level and logging

facility Post by ahmednaas

"The logging facility command basically tells the syslog server where

to put the log message. You configure the syslog server with something

like: local7.debug /var/adm/local7.log

Now, when you use the "logging facility local7" on your device, all

messages with severity "debug" or greater should be saved in

/var/adm/local7.log." Example: on a switch, any process (CDP, SNMP,

etc.) can generate a log message. On a syslog server, the logging

facility is the place where all received messages with the same

priority level are stored.

──────────────────────────────────────────────────

## Question 259  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]An engineer configures interface Gi1/0

on the company PE router to connect to an ISP Neighbor discovery is

disabled

Which action is necessary to complete the configuration if the ISP

uses third-party network devices?

A.  Enable LLDP globally

B.  Disable autonegotiation

C.  Disable Cisco Discovery Protocol on the interface

D.  Enable LLDP-MED on the ISP device

Answer: D

──────────────────────────────────────────────────

## Question 311  (Topic 2)  —  Multiple Choice  [V7.1]

An engineer needs to configure LLDP to send the port description time

length value (TLV). What command sequence must be implemented?

A.  switch(config-line)#IIdp port-description

B.  switch(config)#IIdp port-description

C.  switch(config-if)#IIdp port-description

D.  switch#IIdp port-description

Answer: B

──────────────────────────────────────────────────

## Question 421  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

An administrator must turn off the Cisco Discovery Protocol on the

port configured with address last usable address in the 10.0.0.0/30

subnet. Which command set meets the requirement?

A.  interface gi0/1 no cdp enable

B.  interface gi0/1 clear cdp table

C.  interface gi0/0no cdp advertise-v2

D.  interface gi0/0 no cdp run

Answer: D

──────────────────────────────────────────────────

## Question 441  (Topic 1)  —  Multiple Choice  [V7.1]

A network engineer must back up 20 network router configurations

globally within a customer environment. Which protocol allows the

engineer to perform this function using the Cisco IOS MIB?

A.  CDP

B.  SNMP

C.  SMTP

D.  ARP

Answer: B

Explanation:

SNMP is an application-layer protocol that provides a message format

for communication between SNMP managers and agents. SNMP provides a

standardized framework and a common language used for the monitoring

and management of devices in a network.The SNMP framework has three

parts:+ An SNMP manager+ An SNMP agent+ A Management Information Base

(MIB)The Management Information Base (MIB) is a virtual information

storage area for network management information, which consists of

collections of managed objects.With SNMP, the network administrator

can send commands to multiple routers to do the backup

──────────────────────────────────────────────────

## Question 456  (Topic 1)  —  Multiple Choice  [V7.1]

An engineer is asked to protect unused ports that are configured in

the default VLAN on a switch. Which two steps will fulfill the

request? (Choose two)

A.  Configure the ports in an EtherChannel.

B.  Administratively shut down the ports

C.  Configure the port type as access and place in VLAN 99

D.  Configure the ports as trunk ports

E.  Enable the Cisco Discovery Protocol

Answer: BC

──────────────────────────────────────────────────

## Question 535  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

How must router A be configured so that it only sends Cisco Discovery

Protocol Information to router C?

[See Exhibit Image]

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 21  (Topic 1)  —  Multiple Choice  [V80.2]

A network engineer must back up 20 network router configurations globally within a customer environment.

Which protocol allows the engineer to perform this function using the Cisco IOS MIB?

A.  CDP

B.  SNMP

C.  SMTP

D.  ARP

Answer: B

──────────────────────────────────────────────────

## Question 31  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An administrator must turn off the Cisco Discovery Protocol on the port configured with address last usable

address in the 10.0.0.0/30 subnet. Which command set meets the requirement?

A.  interface gi0/1no cdp enable

B.  interface gi0/1clear cdp table

C.  interface gi0/0no cdp advertise-v2

D.  interface gi0/0no cdp run

Answer: D

──────────────────────────────────────────────────

## Question 115  (Topic 1)  —  Multiple Choice  [V80.2]

An engineer is asked to protect unused ports that are configured in the default VLAN on a switch.

Which two steps will fulfill the request? (Choose two)

A.  Configure the ports in an EtherChannel.

B.  Administratively shut down the ports

C.  Configure the port type as access and place in VLAN 99

D.  Configure the ports as trunk ports

E.  Enable the Cisco Discovery Protocol

Answer: B,C

──────────────────────────────────────────────────

## Question 146  (Topic 1)  —  Multiple Choice  [V80.2]

Which command is used to specify the delay time in seconds for LLDP to initialize on any interface?

A.  lldp timer

B.  lldp holdtimt

C.  lldp reinit

D.  lldp tlv-select

Answer: C

──────────────────────────────────────────────────

## Question 154  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

How must router A be configured so that it only sends Cisco Discovery Protocol Information to router C?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 158  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which command provides this output?

A.  show ip route

B.  show ip interface

C.  show interface

D.  show cdp neighbor

Answer: D

──────────────────────────────────────────────────

## Question 213  (Topic 1)  —  Multiple Choice  [V80.2]

Which protocol does an IPv4 host use to obtain a dynamically assigned IP address?

A.  ARP

B.  DHCP

C.  CDP

D.  DNS

Answer: B

──────────────────────────────────────────────────

## Question 256  (Topic 2)  —  Multiple Choice  [V80.2]

A network engineer must create a diagram of a multivendor network. Which command must be configured

on the Cisco devices so that the topology of the network can be mapped?

A.  Device(Config)#lldp run

B.  Device(Config)#cdp run

C.  Device(Config-if)#cdp enable

D.  Device(Config)#flow-sampler-map topology

Answer: A

──────────────────────────────────────────────────

## Question 361  (Topic 2)  —  Multiple Choice  [V80.2]

Which protocol does an access point use to draw power from a connected switch?

A.  Internet Group Management Protocol

B.  Adaptive Wireless Path Protocol

C.  Cisco Discovery Protocol

D.  Neighbor Discovery Protocol

Answer: C

──────────────────────────────────────────────────

## Question 393  (Topic 2)  —  Multiple Choice  [V80.2]

A network administrator needs to aggregate 4 ports into a single logical link which must negotiate layer 2

connectivity to ports on another switch. What must be configured when using active mode on both sides of

the connection?

A.  802.1q trunks

B.  Cisco vPC

C.  LLDP

D.  LACP

Answer: D

──────────────────────────────────────────────────

## Question 404  (Topic 2)  —  Multiple Choice  [V80.2]

An engineer configures interface Gi1/0 on the company PE router to connect to an ISP Neighbor discovery

is disabled

Which action is necessary to complete the configuration if the ISP uses third-party network devices?

A.  Enable LLDP globally

B.  Disable autonegotiation

C.  Disable Cisco Discovery Protocol on the interface

D.  Enable LLDP-MED on the ISP device

Answer: D

──────────────────────────────────────────────────

## Question 431  (Topic 2)  —  Multiple Choice  [V80.2]

An engineer needs to configure LLDP to send the port description time length value (TLV). What command

sequence must be implemented?

A.  switch(config-line)#IIdp port-description

B.  switch(config)#IIdp port-description

C.  switch(config-if)#IIdp port-description

D.  switch#IIdp port-description

Answer: B

──────────────────────────────────────────────────

## Question 456  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer must update the configuration on Switch2 so that it sends LLDP packets every minute

and the information sent via LLDP is refreshed every 3 minutes Which configuration must the engineer

apply?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: A

──────────────────────────────────────────────────

## Question 498  (Topic 3)  —  Multiple Choice  [V80.2]

A Cisco engineer must configure a single switch interface to meet these requirements

• accept untagged frames and place them in VLAN 20

• accept tagged frames in VLAN 30 when CDP detects a Cisco IP phone Which command set must the

engineer apply?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 608  (Topic 4)  —  Multiple Choice  [V80.2]

How does frame switching function on a switch?

A.  forwards frames to a neighbor port using CDP

B.  modifies frames that contain a known source VLAN

C.  inspects and drops frames from unknown destinations

D.  forwards known destinations to the destination port

Answer: D

──────────────────────────────────────────────────

## Question 649  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer must configure neighbor discovery between the company router and an ISP

What is the next step to complete the configuration if the ISP uses a third-party router?

A.  Enable LLDP globally.

B.  Disable CDP on gi0/0.

C.  Enable LLDP TLVs on the ISP router.

D.  Disable auto-negotiation.

Answer: A

──────────────────────────────────────────────────

## Question 750  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit Routers R1 R2 and R3 use a protocol to identify their neighbors' IP addresses hardware

platforms, and software versions. A network engineer must configure R2 to avoid sharing any neighbor

information with R3, and maintain its relationship with R1. What action meets this requirement?

A.  Configure the no cdp enable command on gO/2.

B.  Configure the no cdp run command globally.

C.  Configure the no lldp run command globally.

D.  Configure the no lldp receive command on gQV1.

Answer: A

──────────────────────────────────────────────────

## Question 758  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is configuring a Layer 3 port-channel interface with LACP. The configuration on the first device

is complete, and it is verified that both interfaces have registered the neighbor device in the CDP table.

Which task on the neighbor device enables the new port channel to come up without negotiating the

channel?

A.  Change the EtherChannel mode on the neighboring interfaces to auto.

B.  Configure the IP address of the neighboring device.

C.  Bring up the neighboring interfaces using the no shutdown command.

D.  Modify the static EtherChannel configuration of the device to passive mode.

Answer: D

──────────────────────────────────────────────────

## Question 869  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which action must be taken so that neighbofing devices rapidly discover switch Cat9300?

A.  Configure the cdp timer 10 command on switch Cat9300.

B.  Enable portfast on the ports that connect to neighboring devices.

C.  Configure the cdp holdtime 10 command on switch Cat9300.

D.  Configure the cdp timer 10 command on the neighbors of switch Cat9300.

Answer: A

──────────────────────────────────────────────────

## Question 870  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer must update the configuring on switch2 so that it sends LLDP packets.

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 872  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The network administrator must prevent the switch Cat9K-2 IP address from being visible in LLDP without

disabling the protocol. Which action must be taken must be taken to complete the task?

A.  Configure the no lldp tlv-select-management-address command globally on Cat9K-2

B.  Configure the no lldp transmit command on interface G1/0/21 in Cat9K-1

C.  Configure the no lldp receive command on interface G1/0/21 on Cat9K-1

D.  Configure the no lldp mac-phy-cfg command globally on Cat9K-2

Answer: A

──────────────────────────────────────────────────

## Question 879  (Topic 4)  —  Multiple Choice  [V80.2]

A switch is a forwarding a frame out of an interfaces except the interface that received the frame. What is

the technical term for this process?

A.  ARP

B.  CDP

C.  flooding

D.  multicast

Answer: C

──────────────────────────────────────────────────

## Question 1039  (Topic 4)  —  Multiple Choice  [V80.2]

A switch is forwarding a frame out of all interfaces except the interface that received the frame. What is the

technical term for this process?

A.  ARP

B.  Multicast

C.  CDP

D.  Flooding

Answer: D

──────────────────────────────────────────────────

## Question 1065  (Topic 5)  —  Multiple Choice  [V80.2]

How does MAC learning function?

A.  Enabled by default on all VLANs and interfaces

B.  Forwards frames to a neighbor port using CDP

C.  Overwrites the known source MAC address in the address table

D.  Protects against denial of service attacks

Answer: A