# CCNA 200-301
Cloud and Virtualization

Cloud models, virtualization, hypervisors

35 Questions

Multiple Choice: 34 | Drag & Drop: 1

Sources: V7.1 (4) | V80.2 (31)

Topic 1: 13 | Topic 2: 4 | Topic 4: 13 | Topic 5: 5

## Question 190  (Topic 2)  —  Multiple Choice  [V7.1]

What are two characteristics of a public cloud Implementation? (Choose

two.)

A.  It is owned and maintained by one party, but it is shared among

multiple organizations.

B.  It enables an organization to fully customize how It deploys network

resources.

C.  It provides services that are accessed over the Internet.

D.  It Is a data center on the public Internet that maintains cloud

services for only one company.

E.  It supports network resources from a centralized third-party

provider and privately- owned virtual resources

Answer: CE

Explanation:

Private cloud is cloud infrastructure operated solely for a single

organization, whether managed internally or by a third party, and

hosted either internally or externally. Most public-cloud providers

offer direct-connection services that allow customers to securely link

their legacy data centers to their cloud-resident applications.

──────────────────────────────────────────────────

## Question 357  (Topic 1)  —  Multiple Choice  [V7.1]

Which statement identifies the functionality of virtual machines?

A.  Virtualized servers run most efficiently when they are physically

connected to a switch that is separate from the hypervisor

B.  The hypervisor can virtualize physical components including CP

C.  memory, and storage

D.  Each hypervisor can support a single virtual machine and a single

software switch

E.  The hypervisor communicates on Layer 3 without the need for

additional resources

Answer: B

──────────────────────────────────────────────────

## Question 543  (Topic 1)  —  Multiple Choice  [V7.1]

What are two functions of a server on a network? (Choose two)

A.  achieves redundancy by exclusively using virtual server clustering

B.  runs applications that send and retrieve data for workstations that

make requests

C.  handles requests from multiple workstations at the same time

D.  runs the same operating system in order to communicate with other

servers

E.  housed solely in a data center that is dedicated to a single client

Answer: BC

──────────────────────────────────────────────────

## Question 601  (Topic 1)  —  Multiple Choice  [V7.1]

An organization has decided to start using cloud-provided services.

Which cloud service allows the organization to install its own

operating system on a virtual machine?

A.  platform-as-a-service

B.  software-as-a-service

C.  network-as-a-service

D.  infrastructure-as-a-service

Answer: B

Explanation:

Below are the 3 cloud supporting services cloud providers provide to

customer:

\+ SaaS (Software as a Service): SaaS uses the web to deliver

applications that are managed by a thirdparty vendor and whose

interface is accessed on the clients' side. Most SaaS applications can

be run directly from a web browser without any downloads or

installations required, although some require plugins.

\+ PaaS (Platform as a Service): are used for applications, and other

development, while providing cloud components to software. What

developers gain with PaaS is a framework they can build upon to

develop or customize applications. PaaS makes the development,

testing, and deployment of applications quick, simple, and

cost-effective. With this technology, enterprise operations, or a

thirdparty provider, can manage OSes, virtualization, servers,

storage, networking, and the PaaS software itself. Developers,

however, manage the applications.

\+ IaaS (Infrastructure as a Service): self-service models for

accessing, monitoring, and managing remote datacenter infrastructures,

such as compute (virtualized or bare metal), storage, networking, and

networking services (e.g. firewalls). Instead of having to purchase

hardware outright, users can purchase IaaS based on consumption,

similar to electricity or other utility billing.

In general, IaaS provides hardware so that an organization can install

their own operating system.

──────────────────────────────────────────────────

## Question 13  (Topic 1)  —  Multiple Choice  [V80.2]

What describes the operation of virtual machines?

A.  Virtual machines are responsible for managing and allocating host hardware resources

B.  In a virtual machine environment, physical servers must run one operating system at a time.

C.  Virtual machines are the physical hardware that support a virtual environment.

D.  Virtual machines are operating system instances that are decoupled from server hardware

Answer: B

──────────────────────────────────────────────────

## Question 27  (Topic 1)  —  Multiple Choice  [V80.2]

Which resource is able to be shared among virtual machines deployed on the same physical server?

A.  disk

B.  applications

C.  VM configuration file

D.  operating system

Answer: C

──────────────────────────────────────────────────

## Question 28  (Topic 1)  —  Multiple Choice  [V80.2]

What are two fundamentals of virtualization? (choose two)

A.  The environment must be configured with one hypervisor that serves solely as a network manager to

monitor SNMP traffic

B.  It allows logical network devices to move traffic between virtual machines and the rest of the physical

network

C.  It allows multiple operating systems and applications to run independently on one physical server.

D.  It allows a physical router to directly connect NICs from each virtual machine into the network

E.  It requires that some servers, virtual machines and network gear reside on the Internet

Answer: B,C

──────────────────────────────────────────────────

## Question 101  (Topic 1)  —  Multiple Choice  [V80.2]

Which technology allows for multiple operating systems to be run on a single host computer?

A.  virtual routing and forwarding

B.  network port ID visualization

C.  virtual device contexts

D.  server visualization

Answer: D

──────────────────────────────────────────────────

## Question 144  (Topic 1)  —  Multiple Choice  [V80.2]

Which statement identifies the functionality of virtual machines?

A.  Virtualized servers run most efficiently when they are physically connected to a switch that is separate

from the hypervisor

B.  The hypervisor can virtualize physical components including CPU. memory, and storage

C.  Each hypervisor can support a single virtual machine and a single software switch

D.  The hypervisor communicates on Layer 3 without the need for additional resources

Answer: B

──────────────────────────────────────────────────

## Question 185  (Topic 1)  —  Multiple Choice  [V80.2]

What is a characteristic of cloud-based network topology?

A.  wireless connections provide the sole access method to services

B.  onsite network services are provided with physical Layer 2 and Layer 3 components

C.  services are provided by a public, private, or hybrid deployment

D.  physical workstations are configured to share resources

Answer: A

──────────────────────────────────────────────────

## Question 209  (Topic 1)  —  Multiple Choice  [V80.2]

An organization has decided to start using cloud-provided services. Which cloud service allows the

organization to install its own operating system on a virtual machine?

A.  platform-as-a-service

B.  software-as-a-service

C.  network-as-a-service

D.  infrastructure-as-a-service

Answer: B

──────────────────────────────────────────────────

## Question 212  (Topic 1)  —  Multiple Choice  [V80.2]

What are two functions of a server on a network? (Choose two)

A.  achieves redundancy by exclusively using virtual server clustering

B.  runs applications that send and retrieve data for workstations that make requests

C.  handles requests from multiple workstations at the same time

D.  runs the same operating system in order to communicate with other servers

E.  housed solely in a data center that is dedicated to a single client

Answer: B,C

──────────────────────────────────────────────────

## Question 223  (Topic 1)  —  Multiple Choice  [V80.2]

A manager asks a network engineer to advise which cloud service models are used so employees do not

have to waste their time installing, managing, and updating software which is only used occasionally Which

cloud service model does the engineer recommend?

A.  infrastructure-as-a-service

B.  platform-as-a-service

C.  business process as service to support different types of service

D.  software-as-a-service

Answer: D

──────────────────────────────────────────────────

## Question 225  (Topic 1)  —  Multiple Choice  [V80.2]

How do servers connect to the network in a virtual environment?

A.  wireless to an access point that is physically connected to the network

B.  a cable connected to a physical switch on the network

C.  a virtual switch that links to an access point that is physically connected to the network

D.  a software switch on a hypervisor that is physically connected to the network

Answer: D

──────────────────────────────────────────────────

## Question 245  (Topic 2)  —  Multiple Choice  [V80.2]

What role does a hypervisor provide for each virtual machine in server virtualization?

A.  infrastructure-as-a-service.

B.  Software-as-a-service

C.  control and distribution of physical resources

D.  services as a hardware controller.

Answer: C

──────────────────────────────────────────────────

## Question 288  (Topic 2)  —  Multiple Choice  [V80.2]

An implementer is preparing hardware for virtualization to create virtual machines on a host. What is

needed to provide communication between hardware and virtual machines?

A.  hypervisor

B.  router

C.  straight cable

D.  switch

Answer: A

──────────────────────────────────────────────────

## Question 324  (Topic 2)  —  Multiple Choice  [V80.2]

What are two characteristics of a public cloud Implementation? (Choose two.)

A.  It is owned and maintained by one party, but it is shared among multiple organizations.

B.  It enables an organization to fully customize how It deploys network resources.

C.  It provides services that are accessed over the Internet.

D.  It Is a data center on the public Internet that maintains cloud services for only one company.

E.  It supports network resources from a centralized third-party provider and privately- owned virtual

resources

Answer: C,E

──────────────────────────────────────────────────

## Question 551  (Topic 4)  —  Multiple Choice  [V80.2]

What is a benefit for external users who consume public cloud resources?

A.  implemented over a dedicated WAN

B.  located in the same data center as the users

C.  all hosted on physical servers

D.  accessed over the Internet

Answer: D

──────────────────────────────────────────────────

## Question 602  (Topic 4)  —  Multiple Choice  [V80.2]

Which physical component is distributed among multiple virtual machines running on the same hypervisor?

A.  external storage

B.  hardware resources

C.  network interfaces

D.  backplane network

Answer: B

──────────────────────────────────────────────────

## Question 613  (Topic 4)  —  Multiple Choice  [V80.2]

What is a function of a southbound API?

A.  Automate configuration changes between a server and a switching fabric.

B.  Use orchestration to provision a virtual server configuration from a web server.

C.  Manage flow control between an SDN controller and a switching fabric.

D.  Facilitate the information exchange between an SDN controller and application.

Answer: C

──────────────────────────────────────────────────

## Question 687  (Topic 4)  —  Multiple Choice  [V80.2]

Which type of hypervisor operates without an underlying OS to host virtual machines?

A.  Type 1

B.  Type 2

C.  Type 3

D.  Type 12

Answer: A

──────────────────────────────────────────────────

## Question 702  (Topic 4)  —  Multiple Choice  [V80.2]

What must a network administrator consider when deciding whether to configure a new wireless network

with APs in autonomous mode or APs running in cloud-based mode?

Autonomous mode APs are less dependent on an underlay but more complex to maintain than APs in

cloud-based mode.

Cloud-based mode APs relay on underlays and are more complex to maintain than APs in autonomous

mode.

A.  Cloud-based mode APs are easy to deploy but harder to automate than APs in autonomous mode.

B.  Autonomous mode APs are easy to deploy and automate than APs in cloud-based mode.

Answer: A

──────────────────────────────────────────────────

## Question 754  (Topic 4)  —  Multiple Choice  [V80.2]

Which component controls and distributes physical resources for each virtual machine?

A.  OS

B.  hypervisor

C.  CPU

D.  physical enclosure

Answer: B

──────────────────────────────────────────────────

## Question 793  (Topic 4)  —  Multiple Choice  [V80.2]

A manager asks a network engineer to recommend a cloud service model so that employees do not spend

time installing, managing, and updating software that is only used occasionally. Which cloud service model

does the engineer recommend?

A.  Infrastructure-as-a-service

B.  software-as-a-service

C.  business process as a service

D.  platform-as-a-service

Answer: B

──────────────────────────────────────────────────

## Question 849  (Topic 4)  —  Multiple Choice  [V80.2]

A wireless access point is needed and must meet these requirements:

• "zero-touch" deployed and managed by a WLC

• process only real-time MAC functionality

• used in a split-MAC architecture. Which access point type must be used?

A.  autonomous

B.  lightweight

C.  mesh

D.  cloud-based

Answer: B

──────────────────────────────────────────────────

## Question 876  (Topic 4)  —  Multiple Choice  [V80.2]

Which components are contained within a virtual machine?

A.  physical resources, including the NIC, RAM, disk, and CPU

B.  configuration files backed by physical resources from the Hypervisor

C.  applications running on the Hypervisor

D.  processes running on the Hypervisor and a guest OS

Answer: B

──────────────────────────────────────────────────

## Question 877  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the virtualization concepts from the left onto the matching statements on the right.

──────────────────────────────────────────────────

## Question 890  (Topic 4)  —  Multiple Choice  [V80.2]

Which architecture is best for small offices with minimal wireless needs and no central management?

A.  cloud-based AP

B.  split MAC

C.  autonomous AP

D.  mesh network

Answer: C

──────────────────────────────────────────────────

## Question 920  (Topic 4)  —  Multiple Choice  [V80.2]

A network architect planning a new Wi-Fi network must decide between autonomous, cloud-based, and

split MAC architectures. Which two facts should the architect consider? (Choose two.)

A.  Lightweight access points are solely used by split MAC architectures.

B.  Cloud-based architectures uniquely use the CAPWAP protocol to communicate between access points

and clients.

C.  Each of the three architectures must use WLCs to manage their access points.

D.  All three architectures use access points to manage the wireless devices connected to the wired

infrastructure.

E.  Autonomous architectures exclusively use tunneling protocols to manage access points remotely.

Answer: A,E

──────────────────────────────────────────────────

## Question 989  (Topic 4)  —  Multiple Choice  [V80.2]

In a cloud-computing environment what is rapid elasticity?

A.  control and monitoring of resource consumption by the tenant

B.  automatic adjustment of capacity based on need

C.  pooling resources in a multitenant model based on need

D.  self-service of computing resources by the tenant

Answer: B

──────────────────────────────────────────────────

## Question 1069  (Topic 5)  —  Multiple Choice  [V80.2]

Which technology allows multiple operating systems to run a single physical server?

A.  Containers

B.  Cloud computing

C.  Application hosting

D.  Virtualization

Answer: D

──────────────────────────────────────────────────

## Question 1085  (Topic 5)  —  Multiple Choice  [V80.2]

Where are the real-time control functions processed in a split MAC architecture?

A.  Centralized cloud management platform

B.  Central WLC

C.  Individual AP

D.  Client device

Answer: C

──────────────────────────────────────────────────

## Question 1197  (Topic 5)  —  Multiple Choice  [V80.2]

Which cloud-provided service allows an organization to install its own operating system on a virtual

machine?

A.  platform-as-a-service

B.  software-as-a-service

C.  infrastructure-as-a-service

D.  network-as-a-service

Answer: C

──────────────────────────────────────────────────

## Question 1214  (Topic 5)  —  Multiple Choice  [V80.2]

Which statement describes virtualization on containers?

A.  It is a type of operating system virtualization that allows the host operating system to control the different

CPU memory processes.

B.  It emulates a physical computer and enables multiple machines to run with many operating systems on a

physical machine.

C.  It separates virtual machines from each other and allocates memory, processors, and storage to

compute.

D.  It contains a guest operating system and virtual partition of hardware for OS and requires application

libraries.

Answer: A

──────────────────────────────────────────────────

## Question 1215  (Topic 5)  —  Multiple Choice  [V80.2]

Which technology allows multiple operating systems lo run a single physical server?

A.  cloud computing

B.  virtualization

C.  application hosting

D.  containers

Answer: B