# CCNA 200-301
DHCP

Dynamic Host Configuration Protocol

56 Questions

Multiple Choice: 54 | Drag & Drop: 2

Sources: V7.1 (9) | V80.2 (47)

Topic 1: 22 | Topic 2: 9 | Topic 3: 5 | Topic 4: 17 | Topic 5: 3

## Question 1  (Topic 3)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

The DHCP server and clients are connected to the same switch. What is

the next step to complete the DHCP configuration to allow clients on

VLAN 1 to receive addresses from the DHCP server?

A.  Configure the ip dhcp snooping trust command on the interlace that

is connected to the DHCP client.

B.  Configure the ip dhcp relay information option command on the

interface that is connected to the DHCP client.

C.  Configure the ip dhcp snooping trust command on the interface that

is connected to the DHCP server.

D.  Configure the Ip dhcp relay information option command on the

interface that is connected to the DHCP server.

Answer: C

──────────────────────────────────────────────────

## Question 125  (Topic 2)  —  Multiple Choice  [V7.1]

Which function is performed by DHCP snooping?

A.  propagates VLAN information between switches

B.  listens to multicast traffic for packet forwarding

C.  provides DDoS mitigation

D.  rate-limits certain traffic

Answer: A

──────────────────────────────────────────────────

## Question 310  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

An engineer deploys a topology in which R1 obtains its IP

configuration from DHCP. If the switch and DHCP server configurations

are complete and correct. Which two sets of

commands must be configured on R1 and R2 to complete the task? (Choose

two)

A.  R1(config)# interface fa0/0R1(config-if)# ip helper-address

198.51.100.100

B.  R2(config)# interface gi0/0R2(config-if)# ip helper-address

198.51.100.100

C.  R1(config)# interface fa0/0 R1(config-if)# ip address dhcp

R1(config-if)# no shutdown

D.  R2(config)# interface gi0/0 R2(config-if)# ip address dhcp

E.  R1(config)# interface fa0/0R1(config-if)# ip helper-address

192.0.2.2

Answer: BC

──────────────────────────────────────────────────

## Question 330  (Topic 1)  —  Multiple Choice  [V7.1]

Which type of information resides on a DHCP server?

A.  a list of the available IP addresses in a pool

B.  a list of public IP addresses and their corresponding names

C.  usernames and passwords for the end users in a domain

D.  a list of statically assigned MAC addresses

Answer: A

──────────────────────────────────────────────────

## Question 348  (Topic 1)  —  Multiple Choice  [V7.1]

What are two roles of the Dynamic Host Configuration Protocol (DHCP)?

(Choose two)

A.  The DHCP server offers the ability to exclude specific IP addresses

from a pool of IP addresses

B.  The DHCP client can request up to four DNS server addresses

C.  The DHCP server assigns IP addresses without requiring the client to

renew them

D.  The DHCP server leases client IP addresses dynamically.

E.  The DHCP client maintains a pool of IP addresses it can assign.

Answer: AD

──────────────────────────────────────────────────

## Question 415  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

If the network environment is operating normally, which type of device

must be connected to interface FastEthernet 0/1?

A.  DHCP client

B.  access point

C.  router

D.  PC

Answer: C

──────────────────────────────────────────────────

## Question 484  (Topic 1)  —  Multiple Choice  [V7.1]

Which command enables a router to become a DHCP client?

A.  ip address dhcp

B.  ip helper-address

C.  ip dhcp pool

D.  ip dhcp client

Answer: A

Explanation:

Reference:

[https://www.cisco.com/c/en/us/td/docs/ios-xml/ios/ipaddr_dhcp/configuration/12-4/dhcp-12-4-book/config-dhcp-client.html](http://www.cisco.com/c/en/us/td/docs/ios-xml/ios/ipaddr_dhcp/configuration/12-4/dhcp-12-4-book/config-dhcp-client.html)

If we want to get an IP address from the DHCP server on a Cisco

device, we can use the command "ip address dhcp". Note: The command

"ip helper-address" enables a router to become a DHCP Relay Agent.

──────────────────────────────────────────────────

## Question 594  (Topic 1)  —  Multiple Choice  [V7.1]

Where does the configuration reside when a helper address Is

configured lo support DHCP?

A.  on the router closest to the server

B.  on the router closest to the client

C.  on every router along the path

D.  on the switch trunk interface

Answer: B

──────────────────────────────────────────────────

## Question 600  (Topic 1)  —  Multiple Choice  [V7.1]

On workstations running Microsoft Windows, which protocol provides the

default gateway for the device?

A.  DHCP

B.  STP

C.  SNMP

D.  DNS

Answer: A

──────────────────────────────────────────────────

## Question 15  (Topic 1)  —  Multiple Choice  [V80.2]

What is a DHCP client?

A.  a workstation that requests a domain name associated with its IP address

B.  a host that is configured to request an IP address automatically

C.  a server that dynamically assigns IP addresses to hosts.

D.  a router that statically assigns IP addresses to hosts.

Answer: B

──────────────────────────────────────────────────

## Question 24  (Topic 1)  —  Multiple Choice  [V80.2]

Which access layer threat-mitigation technique provides security based on identity?

A.  Dynamic ARP Inspection

B.  using a non-default native VLAN

C.  802.1x

D.  DHCP snooping

Answer: C

──────────────────────────────────────────────────

## Question 30  (Topic 1)  —  Multiple Choice  [V80.2]

Which command automatically generates an IPv6 address from a specified IPv6 prefix and MAC address of

an interface?

A.  ipv6 address dhcp

B.  ipv6 address 2001:DB8:5:112::/64 eui-64

C.  ipv6 address autoconfig

D.  ipv6 address 2001:DB8:5:112::2/64 link-local

Answer: C

──────────────────────────────────────────────────

## Question 33  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the DHCP snooping terms from the left onto the descriptions on the right.

──────────────────────────────────────────────────

## Question 43  (Topic 1)  —  Multiple Choice  [V80.2]

A network administrator must enable DHCP services between two sites. What must be configured for the

router to pass DHCPDISCOVER messages on to the server?

A.  a DHCP Relay Agent

B.  DHCP Binding

C.  a DHCP Pool

D.  DHCP Snooping

Answer: A

──────────────────────────────────────────────────

## Question 59  (Topic 1)  —  Multiple Choice  [V80.2]

What is a DHCP client?

A.  a host that is configured to request an IP address automatically

B.  a server that dynamically assigns IP addresses to hosts

C.  a workstation that requests a domain name associated with its IP address

D.  a rooter that statically assigns IP addresses to hosts

Answer: A

──────────────────────────────────────────────────

## Question 85  (Topic 1)  —  Multiple Choice  [V80.2]

When implementing a router as a DHCP server, which two features must be configured'? (Choose two)

A.  relay agent information

B.  database agent

C.  address pool

D.  smart-relay

E.  manual bindings

Answer: C,E

──────────────────────────────────────────────────

## Question 117  (Topic 1)  —  Multiple Choice  [V80.2]

When DHCP is configured on a router, which command must be entered so the default gateway is

automatically distributed?

A.  default-router

B.  default-gateway

C.  ip helper-address

D.  dns-server

Answer: A

──────────────────────────────────────────────────

## Question 127  (Topic 1)  —  Multiple Choice  [V80.2]

On workstations running Microsoft Windows, which protocol provides the default gateway for the device?

A.  DHCP

B.  STP

C.  SNMP

D.  DNS

Answer: A

──────────────────────────────────────────────────

## Question 130  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

If the network environment is operating normally, which type of device must be connected to interface

FastEthernet 0/1?

A.  DHCP client

B.  access point

C.  router

D.  PC

Answer: C

──────────────────────────────────────────────────

## Question 148  (Topic 1)  —  Multiple Choice  [V80.2]

Which type of information resides on a DHCP server?

A.  a list of the available IP addresses in a pool

B.  a list of public IP addresses and their corresponding names

C.  usernames and passwords for the end users in a domain

D.  a list of statically assigned MAC addresses

Answer: A

──────────────────────────────────────────────────

## Question 163  (Topic 1)  —  Multiple Choice  [V80.2]

What are two roles of the Dynamic Host Configuration Protocol (DHCP)? (Choose two)

A.  The DHCP server offers the ability to exclude specific IP addresses from a pool of IP addresses

B.  The DHCP client can request up to four DNS server addresses

C.  The DHCP server assigns IP addresses without requiring the client to renew them

D.  The DHCP server leases client IP addresses dynamically.

E.  The DHCP client maintains a pool of IP addresses it can assign.

Answer: A,D

──────────────────────────────────────────────────

## Question 167  (Topic 1)  —  Multiple Choice  [V80.2]

Which command enables a router to become a DHCP client?

A.  ip address dhcp

B.  ip helper-address

C.  ip dhcp pool

D.  ip dhcp client

Answer: A

──────────────────────────────────────────────────

## Question 195  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the functions of DHCP from the left onto any of the positions on the right Not all functions are

used

──────────────────────────────────────────────────

## Question 213  (Topic 1)  —  Multiple Choice  [V80.2]

Which protocol does an IPv4 host use to obtain a dynamically assigned IP address?

A.  ARP

B.  DHCP

C.  CDP

D.  DNS

Answer: B

──────────────────────────────────────────────────

## Question 230  (Topic 1)  —  Multiple Choice  [V80.2]

Where does the configuration reside when a helper address Is configured lo support DHCP?

A.  on the router closest to the server

B.  on the router closest to the client

C.  on every router along the path

D.  on the switch trunk interface

Answer: B

──────────────────────────────────────────────────

## Question 289  (Topic 2)  —  Multiple Choice  [V80.2]

Which function is performed by DHCP snooping?

A.  propagates VLAN information between switches

B.  listens to multicast traffic for packet forwarding

C.  provides DDoS mitigation

D.  rate-limits certain traffic

Answer: A

──────────────────────────────────────────────────

## Question 290  (Topic 2)  —  Multiple Choice  [V80.2]

Which command must be entered to configure a DHCP relay?

A.  ip helper-address

B.  ip address dhcp

C.  ip dhcp pool

D.  ip dhcp relay

Answer: A

──────────────────────────────────────────────────

## Question 301  (Topic 2)  —  Multiple Choice  [V80.2]

When a client and server are not on the same physical network, which device is used to forward requests

and replies between client and server for DHCP?

A.  DHCP relay agent

B.  DHCP server

C.  DHCPDISCOVER

D.  DHCPOFFER

Answer: A

──────────────────────────────────────────────────

## Question 323  (Topic 2)  —  Multiple Choice  [V80.2]

What prevents a workstation from receiving a DHCP address?

A.  DTP

B.  STP

C.  VTP

D.  802.10

Answer: B

──────────────────────────────────────────────────

## Question 351  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer deploys a topology in which R1 obtains its IP configuration from DHCP. If the switch and

DHCP server configurations are complete and correct. Which two sets of

commands must be configured on R1 and R2 to complete the task? (Choose two)

A.  R1(config)# interface fa0/0R1(config-if)# ip helper-address 198.51.100.100

B.  R2(config)# interface gi0/0R2(config-if)# ip helper-address 198.51.100.100

C.  R1(config)# interface fa0/0R1(config-if)# ip address dhcpR1(config-if)# no shutdown

D.  R2(config)# interface gi0/0R2(config-if)# ip address dhcp

E.  R1(config)# interface fa0/0R1(config-if)# ip helper-address 192.0.2.2

Answer: B,C

──────────────────────────────────────────────────

## Question 388  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What is the effect of this configuration?

A.  All ARP packets are dropped by the switch

B.  Egress traffic is passed only if the destination is a DHCP server.

C.  All ingress and egress traffic is dropped because the interface is untrusted

D.  The switch discard all ingress ARP traffic with invalid MAC-to-IP address bindings.

Answer: D

──────────────────────────────────────────────────

## Question 422  (Topic 2)  —  Multiple Choice  [V80.2]

Where does a switch maintain DHCP snooping information?

A.  in the MAC address table

B.  in the CAM table

C.  in the binding database

D.  in the frame forwarding database

Answer: C

──────────────────────────────────────────────────

## Question 476  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which configuration enables DHCP addressing for hosts connected to interface FastEthernetO/1 on router

R4?

A.  interface FastEthernet0/0ip helper-address 10.0.1.1iaccess-list 100 permit udp host 10.0.1.1 eq bootps

host 10.148.2.1

B.  interface FastEthernot0/1ip helper-address 10.0.1.1!access-list 100 permit tcp host 10.0.1.1 eq 67 host

10.148.2.1

C.  interface FastEthernetO/0ip helper-address 10.0.1.1Iaccess-list 100 permit host 10.0.1.1 host

10.148.2.1 eq bootps

D.  interface FastEthernet0/1ip helper-address 10.0.1.1!access-list 100 permit udp host 10.0.1.1 eq bootps

host 10.148.2.1

Answer: B

──────────────────────────────────────────────────

## Question 505  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is configuring a new router on the network and applied this configuration. Which additional

configuration allows the PC to obtain its IP address from a DHCP server?

A.  Configure the ip dhcp relay information command under interface Gi0/1.

B.  Configure the ip dhcp smart-relay command globally on the router

C.  Configure the ip helper-address 172.16.2.2 command under interface Gi0/0

D.  Configure the ip address dhcp command under interface Gi0/0

Answer: C

──────────────────────────────────────────────────

## Question 516  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The DHCP server and clients are connected to the same switch. What is the next step to complete the

DHCP configuration to allow clients on VLAN 1 to receive addresses from the

DHCP server?

A.  Configure the ip dhcp snooping trust command on the interlace that is connected to the DHCP client.

B.  Configure the ip dhcp relay information option command on the interface that is connected to the DHCP

client.

C.  Configure the ip dhcp snooping trust command on the interface that is connected to the DHCP server.

D.  Configure the Ip dhcp relay information option command on the interface that is connected to the DHCP

server.

Answer: C

──────────────────────────────────────────────────

## Question 533  (Topic 3)  —  Multiple Choice  [V80.2]

What is the purpose of the ip address dhcp command?

A.  to configure an Interface as a DHCP server

B.  to configure an interface as a DHCP helper

C.  to configure an interface as a DHCP relay

D.  to configure an interface as a DHCP client

Answer: D

──────────────────────────────────────────────────

## Question 543  (Topic 4)  —  Multiple Choice  [V80.2]

What is a DHCP client?

A.  a server that dynamically assigns IP addresses to hosts

B.  a router that statically assigns IP addresses to hosts

C.  a host that is configured to request an IP address automatically

D.  a workstation that requests a domain name associated with its IP address

Answer: C

──────────────────────────────────────────────────

## Question 644  (Topic 4)  —  Multiple Choice  [V80.2]

What are two functions of DHCP servers? (Choose two.)

A.  prevent users from assigning their own IP addresses to hosts

B.  assign dynamic IP configurations to hosts in a network

C.  support centralized IP management

D.  issue DHCPDISCOVER messages when added to the network

E.  respond to client DHCPOFFER requests by issuing an IP address

Answer: B,C

──────────────────────────────────────────────────

## Question 666  (Topic 4)  —  Multiple Choice  [V80.2]

A HCP pool has been created with the name CONTROL. The pool uses the next to last usable IP address

as the default gateway for the DHCP clients. The server is located at 172.16 32.15. What is the step in the

process for clients on the 192.168.52.0/24 subnet to reach the DHCP server?

A.  ip forward-protocol udp 137

B.  ip default-network 192.168.52.253

C.  ip helper-address 172.16.32.15

D.  ip default-gateway 192.168.52.253

Answer: C

──────────────────────────────────────────────────

## Question 714  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer is installing a new wireless printer with a static IP address on the Wi-Fi network. Which feature

must be enabled and configured to prevent connection issues with the printer?

A.  client exclusion

B.  passive client

C.  DHCP address assignment

D.  static IP tunneling

Answer: C

──────────────────────────────────────────────────

## Question 723  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The DHCP server is configured with a DHCP pool for each of the subnets represented. Which command

must be configured on switch SW1 to allow DHCP clients on VLAN 10 to receive dynamic IP addresses

from the DHCP server?

A.  SW1(config-if)#ip helper-address 192.168.20.2

B.  SW1(config-if)#ip helper-address 192.168.20.1

C.  SW1(config-if)#ip helper-address 192.168.10.2

D.  SW1(config-if)#ip helper-address 192.168.10.1

Answer: C

──────────────────────────────────────────────────

## Question 735  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A newly configured PC fails to connect to the internet using TCP port 80 to www cisco com Which setting

must be modified for the connection to work?

A.  Subnet Mask

B.  DNS Servers

C.  Default Gateway

D.  DHCP Server

Answer: B

──────────────────────────────────────────────────

## Question 743  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The P2P blocking action option is disabled on the WLC.

A.  Enable the Static IP Tunneling option.

B.  Disable the Coverage Hole Detection option.

C.  Check the DHCP Addr. Assignment check box.

D.  Set the P2P Blocking Action option to Forward-UpStream.

Answer: A

──────────────────────────────────────────────────

## Question 794  (Topic 4)  —  Multiple Choice  [V80.2]

Which IPv6 address range is suitable for anycast addresses for distributed services such DHCP or DNS?

A.  FF00:1/12

B.  2001:db8:0234:ca3e::1/128

C.  2002:db84:3f37:ca98:be05:8/64

D.  FE80::1/10

Answer: A

──────────────────────────────────────────────────

## Question 800  (Topic 4)  —  Multiple Choice  [V80.2]

What are two features of the DHCP relay agent? (Choose two.)

A.  assigns DNS locally and then forwards request to DHCP server

B.  permits one IP helper command under an individual Layer 3 interface

C.  allows only MAC-to-IP reservations to determine the local subnet of a client

D.  minimizes the necessary number of DHCP servers

E.  configured under the Layer 3 interface of a router on the client subnet

Answer: B,E

──────────────────────────────────────────────────

## Question 835  (Topic 4)  —  Multiple Choice  [V80.2]

When an access point is seeking to join wireless LAN controller, which message is sent to the AP- Manager

interface?

A.  Discovery response

B.  DHCP request

C.  DHCP discover

D.  Discovery request

Answer: D

──────────────────────────────────────────────────

## Question 881  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit An IPv6 address must be obtained automatically on the LAN interface on R1 Which

command must be implemented to accomplish the task?

A.  Ipv6 address 2001:dbB:d8d2:1008:4343:61:0010::/64

B.  Ipv6 address autoconfig

C.  Ipv6 address fe80::/10

D.  Ipv6 address dhcp

Answer: B

──────────────────────────────────────────────────

## Question 899  (Topic 4)  —  Multiple Choice  [V80.2]

What is used to identify spurious DHCP servers?

A.  DHCPREQUEST

B.  DHCPDISCOVER

C.  DHCPACK

D.  DHCPOFFER

Answer: D

──────────────────────────────────────────────────

## Question 926  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What does the host do when using the IPv4 Preferred function?

A.  It continues to use a statically assigned IPv4 address

B.  It forces the DNS server to provide the same IPv4 address at each renewal.

C.  It requests the same IPv4 address when it renews its lease with the DHCP server.

D.  It prefers a pool of addresses when renewing the IPv4 host IP address

Answer: C

──────────────────────────────────────────────────

## Question 948  (Topic 4)  —  Multiple Choice  [V80.2]

The clients and OHCP server reside on different subnets. Which command must be used to forward

requests and replies between clients on the 10.10.0.1/24 subnet and the DHCP server at 192.168.10.1?

A.  ip route 192.168.10.1

B.  ip default-gateway 192.168.10.1

C.  ip helper-address 192.168.10.1

D.  ip dhcp address 192.168.10.1

Answer: C

──────────────────────────────────────────────────

## Question 985  (Topic 4)  —  Multiple Choice  [V80.2]

A DHCP pool has been created with the name NOCC. The pool is using 192.168.20.0/24 and must use the

next to last usable IP address as the default gateway for the DHCP clients. What is the next step in the

process?

A.  default-router192.168.20.253

B.  network 192.168.20.254 255.255.255.0 secondary

C.  ip default-gateway 0.0.0.0 0.0.0.0 192.168.20.253

D.  next-server 192.168.20.254

Answer: A

──────────────────────────────────────────────────

## Question 988  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An architect is managing a wireless network with APs from several branch offices connecting to the WLC in

the data center. There is a new requirement for a single WLAN to process the client data traffic without

sending it to the WLC. Which action must be taken to complete the request?

A.  Enable local HTTP profiling.

B.  Enable Disassociation Imminent.

C.  Enable FlexConnect Local Switching.

D.  Enable local DHCP Profiling.

Answer: C

──────────────────────────────────────────────────

## Question 994  (Topic 4)  —  Multiple Choice  [V80.2]

What are two reasons to implement DHCP in a network? (Choose two.)

A.  reduce administration time in managing IP address ranges for clients

B.  control the length of time an IP address is used by a network device

C.  manually control and configure IP addresses on network devices

D.  dynamic control over the best path to reach an IP address

E.  access a website by name instead of by IP address

Answer: D,E

──────────────────────────────────────────────────

## Question 1101  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A network engineer must configure the CPE router to allow computers in the

172.20.1.0/24 network to obtain their IP configurations from the central DHCP server. Which configuration

must the engineer apply to the CPE?

A.  interface GigabitEthernet0/1ip helper-address 172.20.254.1

B.  interface GigabitEthernet0/0ip helper-address 172.20.1.1

C.  interface GigabitEthernet0/0ip helper-address 172.20.255.1

D.  interface GigabitEthernet0/ip helper-address 172.20.255.11

Answer: A

──────────────────────────────────────────────────

## Question 1124  (Topic 5)  —  Multiple Choice  [V80.2]

Which interface on the WLC is used exclusively as a DHCP relay?

A.  distribution

B.  service

C.  AP-manager

D.  virtual

Answer: B

──────────────────────────────────────────────────

## Question 1142  (Topic 5)  —  Multiple Choice  [V80.2]

A new DHCP server has been deployed in a corporate environment with lease time set to eight hours.

Which CMD command on a Windows-based device allows the engineer to verify the DHCP lease

expiration?

A.  ipconfig /renew

B.  ipconfig

C.  ipconfig /all

D.  ipconfig /displaydns

Answer: C