# CCNA 200-301
DNS

Domain Name System

45 Questions

Multiple Choice: 40 | Drag & Drop: 5

Sources: V7.1 (6) | V80.2 (39)

Topic 1: 15 | Topic 2: 5 | Topic 3: 3 | Topic 4: 17 | Topic 5: 5

## Question 54  (Topic 3)  —  Multiple Choice  [V7.1]

A network engineer is configuring a switch so that it is remotely

reachable via SSH. The engineer has already configured the host name

on the router. Which additional command must the engineer configure

before entering the command to generate the RSA key?

A.  password password

B.  crypto key generate rsa modulus 1024

C.  ip domain-name domain

D.  ip ssh authentication-retries 2

Answer: C

Explanation:

[https://www.cisco.com/c/en/us/solutions/small-business/resource-center/networking/how-to-setup-network-switch.html](http://www.cisco.com/c/en/us/solutions/small-business/resource-center/networking/how-to-setup-network-switch.html)

──────────────────────────────────────────────────

## Question 192  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

The default-information originate command is configured under the R1

OSPF configuration After testing workstations on VLAN 20 at Site B

cannot reach a DNS server on the Internet Which action corrects the

configuration issue?

A.  Add the default-information originate command onR2

B.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.18 command on R1

C.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.2 command on R2

D.  Add the always keyword to the default-information originate command

on R1

Answer: B

──────────────────────────────────────────────────

## Question 194  (Topic 2)  —  Multiple Choice  [V7.1]

Which configuration is needed to generate an RSA key for SSH on a

router?

A.  Configure the version of SSH

B.  Configure VTY access.

C.  Create a user with a password.

D.  Assign a DNS domain name

Answer: D

──────────────────────────────────────────────────

## Question 348  (Topic 1)  —  Multiple Choice  [V7.1]

What are two roles of the Dynamic Host Configuration Protocol (DHCP)?

(Choose two)

A.  The DHCP server offers the ability to exclude specific IP addresses

from a pool of IP addresses

B.  The DHCP client can request up to four DNS server addresses

C.  The DHCP server assigns IP addresses without requiring the client to

renew them

D.  The DHCP server leases client IP addresses dynamically.

E.  The DHCP client maintains a pool of IP addresses it can assign.

Answer: AD

──────────────────────────────────────────────────

## Question 577  (Topic 1)  —  Multiple Choice  [V7.1]

What facilitates a Telnet connection between devices by entering the

device name?

A.  SNMP

B.  DNS lookup

C.  syslog

D.  NTP

Answer: B

──────────────────────────────────────────────────

## Question 600  (Topic 1)  —  Multiple Choice  [V7.1]

On workstations running Microsoft Windows, which protocol provides the

default gateway for the device?

A.  DHCP

B.  STP

C.  SNMP

D.  DNS

Answer: A

──────────────────────────────────────────────────

## Question 15  (Topic 1)  —  Multiple Choice  [V80.2]

What is a DHCP client?

A.  a workstation that requests a domain name associated with its IP address

B.  a host that is configured to request an IP address automatically

C.  a server that dynamically assigns IP addresses to hosts.

D.  a router that statically assigns IP addresses to hosts.

Answer: B

──────────────────────────────────────────────────

## Question 47  (Topic 1)  —  Drag & Drop  [V80.2]

Drag the IPv6 DNS record types from the left onto the description on the right.

──────────────────────────────────────────────────

## Question 59  (Topic 1)  —  Multiple Choice  [V80.2]

What is a DHCP client?

A.  a host that is configured to request an IP address automatically

B.  a server that dynamically assigns IP addresses to hosts

C.  a workstation that requests a domain name associated with its IP address

D.  a rooter that statically assigns IP addresses to hosts

Answer: A

──────────────────────────────────────────────────

## Question 64  (Topic 1)  —  Multiple Choice  [V80.2]

What facilitates a Telnet connection between devices by entering the device name?

A.  SNMP

B.  DNS lookup

C.  syslog

D.  NTP

Answer: B

──────────────────────────────────────────────────

## Question 76  (Topic 1)  —  Multiple Choice  [V80.2]

What is a DNS lookup operation?

A.  DNS server pings the destination to verify that it is available

B.  serves requests over destination port 53

C.  DNS server forwards the client to an alternate IP address when the primary IP is down

D.  responds to a request for IP address to domain name resolution to the DNS server

Answer: D

──────────────────────────────────────────────────

## Question 109  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the DNS lookup components from the left onto the functions on the right.

──────────────────────────────────────────────────

## Question 117  (Topic 1)  —  Multiple Choice  [V80.2]

When DHCP is configured on a router, which command must be entered so the default gateway is

automatically distributed?

A.  default-router

B.  default-gateway

C.  ip helper-address

D.  dns-server

Answer: A

──────────────────────────────────────────────────

## Question 127  (Topic 1)  —  Multiple Choice  [V80.2]

On workstations running Microsoft Windows, which protocol provides the default gateway for the device?

A.  DHCP

B.  STP

C.  SNMP

D.  DNS

Answer: A

──────────────────────────────────────────────────

## Question 151  (Topic 1)  —  Multiple Choice  [V80.2]

What are two roles of Domain Name Services (DNS)? (Choose Two)

A.  builds a flat structure of DNS names for more efficient IP operations

B.  encrypts network Traffic as it travels across a WAN by default

C.  improves security by protecting IP addresses under Fully Qualified Domain Names (FQDNs)

D.  enables applications to identify resources by name instead of IP address

E.  allows a single host name to be shared across more than one IP address

Answer: D,E

──────────────────────────────────────────────────

## Question 163  (Topic 1)  —  Multiple Choice  [V80.2]

What are two roles of the Dynamic Host Configuration Protocol (DHCP)? (Choose two)

A.  The DHCP server offers the ability to exclude specific IP addresses from a pool of IP addresses

B.  The DHCP client can request up to four DNS server addresses

C.  The DHCP server assigns IP addresses without requiring the client to renew them

D.  The DHCP server leases client IP addresses dynamically.

E.  The DHCP client maintains a pool of IP addresses it can assign.

Answer: A,D

──────────────────────────────────────────────────

## Question 169  (Topic 1)  —  Multiple Choice  [V80.2]

What does a router do when configured with the default DNS lookup settings, and a URL is entered on the

CLI?

A.  initiates a ping request to the URL

B.  prompts the user to specify the desired IP address

C.  continuously attempts to resolve the URL until the command is cancelled

D.  sends a broadcast message in an attempt to resolve the URL

Answer: D

──────────────────────────────────────────────────

## Question 213  (Topic 1)  —  Multiple Choice  [V80.2]

Which protocol does an IPv4 host use to obtain a dynamically assigned IP address?

A.  ARP

B.  DHCP

C.  CDP

D.  DNS

Answer: B

──────────────────────────────────────────────────

## Question 280  (Topic 2)  —  Multiple Choice  [V80.2]

Which configuration is needed to generate an RSA key for SSH on a router?

A.  Configure the version of SSH

B.  Configure VTY access.

C.  Create a user with a password.

D.  Assign a DNS domain name

Answer: D

──────────────────────────────────────────────────

## Question 354  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The default-information originate command is configured under the R1 OSPF configuration After testing

workstations on VLAN 20 at Site B cannot reach a DNS server on the Internet Which action corrects the

configuration issue?

A.  Add the default-information originate command onR2

B.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.18 command on R1

C.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.2 command on R2

D.  Add the always keyword to the default-information originate command on R1

Answer: B

──────────────────────────────────────────────────

## Question 408  (Topic 2)  —  Multiple Choice  [V80.2]

Which two must be met before SSH can operate normally on a Cisco IOS switch? (Choose two)

A.  The switch must be running a k9 (crypto) IOS image

B.  The Ip domain-name command must be configured on the switch

C.  IP routing must be enabled on the switch

D.  A console password must be configured on the switch

E.  Telnet must be disabled on the switch

Answer: A,B

──────────────────────────────────────────────────

## Question 467  (Topic 3)  —  Multiple Choice  [V80.2]

An engineer is configuring remote access to a router from IP subnet 10.139.58.0/28. The domain name,

crypto keys, and SSH have been configured. Which configuration enables the traffic on the destination

router?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 479  (Topic 3)  —  Multiple Choice  [V80.2]

A network engineer is configuring a switch so that it is remotely reachable via SSH. The engineer has

already configured the host name on the router. Which additional command must the engineer configure

before entering the command to generate the RSA key?

A.  password password

B.  crypto key generate rsa modulus 1024

C.  ip domain-name domain

D.  ip ssh authentication-retries 2

Answer: C

──────────────────────────────────────────────────

## Question 541  (Topic 4)  —  Multiple Choice  [V80.2]

An on-site service desk technician must verify the IP address and DNS server information on a users

Windows computer. Which command must the technician enter at the command prompt on the user's

computer?

A.  ipconfig /all

B.  ifconfig -a

C.  show interface

D.  netstat -r

Answer: A

──────────────────────────────────────────────────

## Question 543  (Topic 4)  —  Multiple Choice  [V80.2]

What is a DHCP client?

A.  a server that dynamically assigns IP addresses to hosts

B.  a router that statically assigns IP addresses to hosts

C.  a host that is configured to request an IP address automatically

D.  a workstation that requests a domain name associated with its IP address

Answer: C

──────────────────────────────────────────────────

## Question 579  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the steps in a standard DNS lookup operation from the left into the order on the right.

──────────────────────────────────────────────────

## Question 615  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. The user has connectivity to devices on network 192.168.3 0/24 but cannot reach users

on the network 10.10.1.0724.

What is the first step to verify connectivity?

A.  Is the internet reachable?

B.  Is the default gateway reachable?

C.  Is the DNS server reachable?

Answer: B

──────────────────────────────────────────────────

## Question 639  (Topic 4)  —  Multiple Choice  [V80.2]

A network engineer is configuring a switch so that it is remotely reachable via SSH. The engineer has

already configured the host name. Which additional command must the engineer configure before entering

the command to generate the RSA key?

A.  ip domain-name domain

B.  password password

C.  crypto key generate rsa modulus 1024

D.  ip ssh authentication-retries 2

Answer: A

──────────────────────────────────────────────────

## Question 680  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer has configured the domain name, user name, and password on the local router. What is the

next step to complete the configuration tor a Secure Shell access RSA key?

A.  crypto key Import rsa pem

B.  crypto key pubkey-chain rsa

C.  crypto key generate rsa

D.  crypto key zeroize rsa

Answer: C

──────────────────────────────────────────────────

## Question 690  (Topic 4)  —  Multiple Choice  [V80.2]

When is the PUT method used within HTTP?

A.  when a nonidempotent operation is needed

B.  to update a DNS server

C.  to display a web site

D.  when a read-only operation it required

Answer: B

──────────────────────────────────────────────────

## Question 704  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which configuration parameter is preventing host C from reaching the internet?

A.  automatic DNS

B.  default gateway

C.  IP network mask

D.  IP address assignment

Answer: B

──────────────────────────────────────────────────

## Question 735  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A newly configured PC fails to connect to the internet using TCP port 80 to www cisco com Which setting

must be modified for the connection to work?

A.  Subnet Mask

B.  DNS Servers

C.  Default Gateway

D.  DHCP Server

Answer: B

──────────────────────────────────────────────────

## Question 794  (Topic 4)  —  Multiple Choice  [V80.2]

Which IPv6 address range is suitable for anycast addresses for distributed services such DHCP or DNS?

A.  FF00:1/12

B.  2001:db8:0234:ca3e::1/128

C.  2002:db84:3f37:ca98:be05:8/64

D.  FE80::1/10

Answer: A

──────────────────────────────────────────────────

## Question 800  (Topic 4)  —  Multiple Choice  [V80.2]

What are two features of the DHCP relay agent? (Choose two.)

A.  assigns DNS locally and then forwards request to DHCP server

B.  permits one IP helper command under an individual Layer 3 interface

C.  allows only MAC-to-IP reservations to determine the local subnet of a client

D.  minimizes the necessary number of DHCP servers

E.  configured under the Layer 3 interface of a router on the client subnet

Answer: B,E

──────────────────────────────────────────────────

## Question 822  (Topic 4)  —  Multiple Choice  [V80.2]

What is a valid IPv6 address record in DNS?

A.  A

B.  MX

C.  AAAA

D.  CNAME

Answer: C

──────────────────────────────────────────────────

## Question 861  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. This ACL is configured to allow client access only to HTTP, HTTPS, and DNS services

via UDP. The new administrator wants to add TCP access to the DNS service. Which configuration updates

the ACL efficiently?

A.  ip access-list extended Services35 permit tcp 10.0.0.0 0.255.255.255 host 198.51.100.11 eq domain

B.  no ip access-list extended Servicesip access-list extended Services30 permit tcp 10.0.0.0 0.255.255.255

host 198.51.100.11 eq domain

C.  ip access-list extended Servicespermit tcp 10.0.0.0 0.255.255.255 host 198.51.100.11 eq domain

D.  no ip access-list extended Servicesip access-list extended Servicespermit udp 10.0.0.0 0.255.255.255

any eq 53permit tcp 10.0.0.0 0.255.255.255 host 198.51.100.11 eq domaindeny ip any any log

Answer: A

──────────────────────────────────────────────────

## Question 887  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An engineer executed the script and added commands that were not necessary for

SSH and must now remove the commands. Which two commands must be executed to correct the

configuration? (Choose two.)

A.  no ip domain name ccna.cisco.com

B.  no login local

C.  no service password-encryption

D.  no ip name-server 198.51.100.210

E.  no hostname CPE

Answer: C,D

──────────────────────────────────────────────────

## Question 926  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What does the host do when using the IPv4 Preferred function?

A.  It continues to use a statically assigned IPv4 address

B.  It forces the DNS server to provide the same IPv4 address at each renewal.

C.  It requests the same IPv4 address when it renews its lease with the DHCP server.

D.  It prefers a pool of addresses when renewing the IPv4 host IP address

Answer: C

──────────────────────────────────────────────────

## Question 1017  (Topic 4)  —  Multiple Choice  [V80.2]

Which type of DNS record is used to specify the mail server responsible for accepting email messages on

behalf of a recipient's domain?

A.  MX record

B.  TXT record

C.  A record

D.  SRV record

Answer: C

──────────────────────────────────────────────────

## Question 1044  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the DNS commands from the left onto their effects on the right.

──────────────────────────────────────────────────

## Question 1097  (Topic 5)  —  Drag & Drop  [V80.2]

Drag and drop the steps in a standard DNS lookup operation from the let into the order on the right.

──────────────────────────────────────────────────

## Question 1113  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. What is preventing host A from reaching the internet?

A.  The domain name server is unreachable.

B.  LAN and WAN network segments are different.

C.  IP address assignment is incorrect.

D.  The default gateway should be the first usable IP address.

Answer: C

──────────────────────────────────────────────────

## Question 1118  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. During initial configuration testing, the Windows workstation PC1 cannot connect with

the 172.16.2.0/24 network. Which set of actions corrects the configuration?

A.  Change the IP address to 172.16.1.6 and change the subnet mask to 255.255.255.248.

B.  Change the IP address to 172.16.1.9 and change the default gateway to 172.16.1.7.

C.  Change the IP address to 172.16.1.9 and change the DNS server to 172.16.1.12 only.

D.  Change the IP address to 172.16.1.6 and change the DNS servers to 172.18.1.12 and 172.16.1.13.

Answer: B

──────────────────────────────────────────────────

## Question 1153  (Topic 5)  —  Multiple Choice  [V80.2]

What is the main purpose of SSH management access?

A.  To support DES 56-bit and 3DES (168-bit) ciphers

B.  To enable secured access to the inbound management interface

C.  To validate management access with username and domain name only

D.  To allow passwords protected with HTTPS encryption to be sent

Answer: B

──────────────────────────────────────────────────

## Question 1206  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. During initial configuration testing, the Windows workstation PC1 cannot connect with

the 172.16.2.0/24 network.

Which set of actions corrects the configuration?

A.  Change the IP address to 172.16.1.6 and change the subnet mask to 255.255.255.248.

B.  Change the IP address to 172.16.1.6 and change the DNS servers to 172.16.1.12 and 172.16.1.13.

C.  Change the IP address to 172.16.1.9 and change the default gateway to 172.16.1.7.

D.  Change the IP address to 172.16.1.9 and change the DNS server to 172.16.1.12 only.

Answer: A