# CCNA 200-301
IPv6

IPv6 addressing, NDP, SLAAC, and configuration

80 Questions

Multiple Choice: 58 | Drag & Drop: 22

Sources: V7.1 (10) | V80.2 (70)

Topic 1: 13 | Topic 2: 14 | Topic 3: 8 | Topic 4: 39 | Topic 5: 6

## Question 18  (Topic 3)  —  Multiple Choice  [V7.1]

A network engineer is installing an IPv6-only capable device. The

client has requested that the device IP address be reachable only from

the internal network. Which type of IPv6 address must the engineer

assign?

A.  unique local address

B.  link-local address

C.  aggregatable global address

D.  IPv4-compatible IPv6 address

Answer: B

──────────────────────────────────────────────────

## Question 41  (Topic 3)  —  Multiple Choice  [V7.1]

A network engineer must implement an IPv6 configuration on the vlan

2000 interface to create a routable locally-unique unicast address

that is blocked from being advertised to the internet. Which

configuration must the engineer apply?

A.  interface vlan 2000ipv6 address ffc0:0000:aaaa::1234:2343/64

B.  interface vlan 2000Ipv6 address

fc00:0000:aaaa:a15d:1234:2343:8aca/64

C.  interface vlan 2000ipv6 address fe80;0000:aaaa::1234:2343/64

D.  interface vlan 2000ipv6 address fd00::1234:2343/64

Answer: B

──────────────────────────────────────────────────

## Question 128  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

Which IPv6 configuration is required for R17 to successfully ping the

WAN interface on R18? A)

[See Exhibit Image]

[See Exhibit Image]B)

C\)

[See Exhibit Image]

[See Exhibit Image]D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

Explanation:

ipv6 unicast-routing statement included (IPv6 is enabled on the

router).Compared to the exhibit, Fa0/0 and Fa0/1 have correct

configurations.The route to subnet 2001:db8:4::/64 points to R18's

Fa1/0 (correct next-hop).

──────────────────────────────────────────────────

## Question 170  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

The New York router is configured with static routes pointing to the

Atlanta and Washington sites. Which two tasks must be performed so

that the Serial0/0/0 interfaces on the Atlanta and Washington routers

can reach one another?

(Choose two.)

A.  Configure the ipv6 route 2012::/126 2023::1 command on the

Washington router.

B.  Configure the ipv6 route 2023::/126 2012::1 command on the Atlanta

router.

C.  Configure the Ipv6 route 2012::/126 s0/0/0 command on the Atlanta

router.

D.  Configure the ipv6 route 2023::/126 2012::2 command on the Atlanta

router.

E.  Configure the ipv6 route 2012::/126 2023::2 command on the

Washington router.

Answer: DE

Explanation:

The short syntax of static IPv6 route is:ipv6 route

\<destination-IPv6-address\>

{next-hop-IPv6-address \| exit-interface}

──────────────────────────────────────────────────

## Question 210  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

An engineer configured the New York router with state routes that

point to the Atlanta and Washington sites. When command must be

configured on the Atlanta and Washington routers so that both sites

are able to reach the loopback2 interface on the New York router?

A.  ipv6 route ::/0 Serial 0/0/1

B.  ipv6 route 0/0 Serial 0/0/0

C.  ipv6 route ::/0 Serial 0/0/0

D.  ip route 0.0.0.0.0.0.0.0 Serial 0/0/0

E.  ipv6 route ::/0 2000::2

Answer: C

──────────────────────────────────────────────────

## Question 307  (Topic 2)  —  Multiple Choice  [V7.1]

Which IPv6 address type provides communication between subnets and is

unable to route on the Internet?

A.  global unicast

B.  unique local

C.  link-local

D.  multicast

Answer: B

──────────────────────────────────────────────────

## Question 370  (Topic 1)  —  Multiple Choice  [V7.1]

Which two minimum parameters must be configured on an active interface

to enable OSPFv2 to operate? (Choose two)

A.  OSPF area

B.  OSPF MD5 authentication key

C.  iPv6 address

D.  OSPf process ID

E.  OSPf stub flag

Answer: AD

──────────────────────────────────────────────────

## Question 503  (Topic 1)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 1)

Drag and drop the IPv6 address type characteristics from the left to

the right.

[See Exhibit Image]

A.  Mastered

B.  [See Exhibit Image]Not Mastered **Answer:** A

──────────────────────────────────────────────────

## Question 526  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Which two commands, when configured on router R1, fulfill these

requirements? (Choose two.)

Packets towards the entire network 2001:db8:2::/64 must be forwarded

through router R2. Packets toward host 2001:db8:23::14 preferably must

be forwarded through R3.

A.  Ipv6 route 2001:db8:23::/128 fd00:12::2

B.  Ipv6 route 2001:db8:23::14/128 fd00:13::3

C.  Ipv6 route 2001:db8:23::14/64 fd00:12::2

D.  Ipv6 route 2001:db8:23::/64 fd00:12::2

E.  Ipv6 route 2001:db8:23::14/64 fd00:12::2 200

Answer: DE

──────────────────────────────────────────────────

## Question 623  (Topic 4)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 4)

[See Exhibit Image]Drag and drop the characteristic from the

left onto the IPv6 address type on the right.

A.  Mastered

B.  [See Exhibit Image]Not Mastered **Answer:** A

──────────────────────────────────────────────────

## Question 30  (Topic 1)  —  Multiple Choice  [V80.2]

Which command automatically generates an IPv6 address from a specified IPv6 prefix and MAC address of

an interface?

A.  ipv6 address dhcp

B.  ipv6 address 2001:DB8:5:112::/64 eui-64

C.  ipv6 address autoconfig

D.  ipv6 address 2001:DB8:5:112::2/64 link-local

Answer: C

──────────────────────────────────────────────────

## Question 45  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which two commands, when configured on router R1, fulfill these requirements? (Choose two.)

Packets towards the entire network 2001:db8:2::/64 must be forwarded through router R2. Packets toward

host 2001:db8:23::14 preferably must be forwarded through R3.

A.  Ipv6 route 2001:db8:23::/128 fd00:12::2

B.  Ipv6 route 2001:db8:23::14/128 fd00:13::3

C.  Ipv6 route 2001:db8:23::14/64 fd00:12::2

D.  Ipv6 route 2001:db8:23::/64 fd00:12::2

E.  Ipv6 route 2001:db8:23::14/64 fd00:12::2 200

Answer: D,E

──────────────────────────────────────────────────

## Question 47  (Topic 1)  —  Drag & Drop  [V80.2]

Drag the IPv6 DNS record types from the left onto the description on the right.

──────────────────────────────────────────────────

## Question 70  (Topic 1)  —  Multiple Choice  [V80.2]

Why was the RFC 1918 address space defined?

A.  conserve public IPv4 addressing

B.  preserve public IPv6 address space

C.  reduce instances of overlapping IP addresses

D.  support the NAT protocol

Answer: A

──────────────────────────────────────────────────

## Question 80  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the IPv6 address type characteristics from the left to the right.

──────────────────────────────────────────────────

## Question 111  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to Exhibit.

The loopback1 interface of the Atlanta router must reach the loopback3 interface of the Washington router.

Which two static host routes must be configured on the NEW York router? (Choose two)

A.  ipv6 route 2000::1/128 2012::1

B.  ipv6 route 2000::3/128 2023::3

C.  ipv6 route 2000::3/128 s0/0/0

D.  ipv6 route 2000::1/128 2012::2

E.  ipv6 route 2000::1/128 s0/0/1

Answer: A,B

──────────────────────────────────────────────────

## Question 143  (Topic 1)  —  Multiple Choice  [V80.2]

An engineer must configure the IPv6 address 2001:0db8:0000:0000:0700:0003:400F:572B on the serial0/0

interface of the HQ router and wants to compress it for easier

configuration. Which command must be issued on the router interface?

A.  ipv6 address 2001:db8::700:3:400F:572B

B.  ipv6 address 2001:db8:0::700:3:4F:572B

C.  ipv6 address 2001:Odb8::7:3:4F:572B

D.  ipv6 address 2001::db8:0000::700:3:400F:572B

Answer: A

──────────────────────────────────────────────────

## Question 198  (Topic 1)  —  Multiple Choice  [V80.2]

When configuring IPv6 on an interface, which two IPv6 multicast groups are joined? (Choose two)

A.  2000::/3

B.  2002::5

C.  FC00::/7

D.  FF02::1

E.  FF02::2

Answer: D,E

──────────────────────────────────────────────────

## Question 206  (Topic 1)  —  Multiple Choice  [V80.2]

Which IPv6 address block sends packets to a group address rather than a single address?

A.  2000::/3

B.  FC00::/7

C.  FE80::/10

D.  FF00::/8

Answer: D

──────────────────────────────────────────────────

## Question 222  (Topic 1)  —  Multiple Choice  [V80.2]

Which two minimum parameters must be configured on an active interface to enable OSPFv2 to operate?

(Choose two)

A.  OSPF area

B.  OSPF MD5 authentication key

C.  iPv6 address

D.  OSPf process ID

E.  OSPf stub flag

Answer: A,D

──────────────────────────────────────────────────

## Question 247  (Topic 2)  —  Multiple Choice  [V80.2]

Which IPv6 address type provides communication between subnets and is unable to route on the Internet?

A.  global unicast

B.  unique local

C.  link-local

D.  multicast

Answer: B

──────────────────────────────────────────────────

## Question 251  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to Exhibit.

An engineer is configuring the NEW York router to reach the Lo1 interface of the Atlanta router using

interface Se0/0/0 as the primary path. Which two commands must be configured on the New York router so

that it can reach the Lo1 interface of the Atlanta router via Washington when the link between New York

and Atlanta goes down? (Choose two)

A.  ipv6 router 2000::1/128 2012::1

B.  ipv6 router 2000::1/128 2012::1 5

C.  ipv6 router 2000::1/128 2012::2

D.  ipv6 router 2000::1/128 2023::2 5

E.  ipv6 router 2000::1/128 2023::3 5

Answer: A,E

──────────────────────────────────────────────────

## Question 267  (Topic 2)  —  Multiple Choice  [V80.2]

Which type of IPv6 address is publicly routable in the same way as IPv4 public address?

A.  global unicast

B.  link-local

C.  unique local

D.  multicast

Answer: A

──────────────────────────────────────────────────

## Question 277  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which IPv6 configuration is required for R17 to successfully ping the WAN interface on R18?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 312  (Topic 2)  —  Multiple Choice  [V80.2]

A network engineer must configure the router R1 GigabitEthernet1/1 interface to connect to the router R2

GigabitEthernet1/1 interface. For the configuration to be applied the engineer must compress the address

2001:0db8:0000:0000:0500:000a:400F:583B. Which command must be issued on the interface?

A.  ipv6 address 2001:0db8::5: a: 4F 583B

B.  ipv6 address 2001:db8::500:a:400F:583B

C.  ipv6 address 2001 db8:0::500:a:4F:583B

D.  ipv6 address 2001::db8:0000::500:a:400F:583B

Answer: B

──────────────────────────────────────────────────

## Question 359  (Topic 2)  —  Multiple Choice  [V80.2]

What is the expected outcome when an EUI-64 address is generated?

A.  The seventh bit of the original MAC address of the interface is inverted

B.  The interface ID is configured as a random 64-bit value

C.  The characters FE80 are inserted at the beginning of the MAC address of the interface

D.  The MAC address of the interface is used as the interface ID without modification

Answer: A

──────────────────────────────────────────────────

## Question 363  (Topic 2)  —  Multiple Choice  [V80.2]

which IPv6 address block forwards packets to a multicast address rather than a unicast address?

A.  2000::/3

B.  FC00::/7

C.  FE80::/10

D.  FF00::/12

Answer: D

──────────────────────────────────────────────────

## Question 371  (Topic 2)  —  Multiple Choice  [V80.2]

Which action must be taken to assign a global unicast IPv6 address on an interface that is derived from the

MAC address of that interface?

A.  configure a stateful DHCPv6 server on the network

B.  enable SLAAC on an interface

C.  disable the EUI-64 bit process

D.  explicitly assign a link-local address

Answer: A

──────────────────────────────────────────────────

## Question 381  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The New York router is configured with static routes pointing to the Atlanta and Washington sites. Which

two tasks must be performed so that the Serial0/0/0 interfaces on the Atlanta and Washington routers can

reach one another?

(Choose two.)

A.  Configure the ipv6 route 2012::/126 2023::1 command on the Washington router.

B.  Configure the ipv6 route 2023::/126 2012::1 command on the Atlanta router.

C.  Configure the Ipv6 route 2012::/126 s0/0/0 command on the Atlanta router.

D.  Configure the ipv6 route 2023::/126 2012::2 command on the Atlanta router.

E.  Configure the ipv6 route 2012::/126 2023::2 command on the Washington router.

Answer: D,E

──────────────────────────────────────────────────

## Question 386  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer configured the New York router with state routes that point to the Atlanta and Washington sites.

When command must be configured on the Atlanta and Washington routers so that both sites are able to

reach the loopback2 interface on the New York router?

A.  ipv6 route ::/0 Serial 0/0/1

B.  ipv6 route 0/0 Serial 0/0/0

C.  ipv6 route ::/0 Serial 0/0/0

D.  ip route 0.0.0.0.0.0.0.0 Serial 0/0/0

E.  ipv6 route ::/0 2000::2

Answer: C

──────────────────────────────────────────────────

## Question 450  (Topic 3)  —  Multiple Choice  [V80.2]

Which type of IPv6 address is similar to a unicast address but is assigned to multiple devices on the same

network at the same time?

A.  global unicast address

B.  anycast address

C.  multicast address

D.  link-local address

Answer: B

──────────────────────────────────────────────────

## Question 461  (Topic 3)  —  Multiple Choice  [V80.2]

What is the difference between IPv6 unicast and anycast addressing?

A.  IPv6 anycast nodes must be explicitly configured to recognize the anycast address, but IPv6 unicast

nodes require no special configuration

B.  IPv6 unicast nodes must be explicitly configured to recognize the unicast address, but IPv6 anycast

nodes require no special configuration

C.  An individual IPv6 unicast address is supported on a single interface on one node but an IPv6 anycast

address is assigned to a group of interfaces on multiple nodes.

D.  Unlike an IPv6 anycast address, an IPv6 unicast address is assigned to a group of interfaces on multiple

nodes

Answer: C

──────────────────────────────────────────────────

## Question 462  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is configuring the HO router. Which IPv6 address configuration must be applied to the router

fa0'1 interface for the router to assign a unique 64-brt IPv6 address to Itself?

A.  ipv6 address 2001:DB8:0:1:C601:42FF:FE0F:7/64

B.  ipv6 address 2001:DB8:0:1:C601:42FE:800F:7/64

C.  ipv6 address 2001 :DB8:0:1:FFFF:C601:420F:7/64

D.  iov6 address 2001 :DB8:0:1:FE80:C601:420F:7/64

Answer: A

──────────────────────────────────────────────────

## Question 464  (Topic 3)  —  Multiple Choice  [V80.2]

A network engineer is installing an IPv6-only capable device. The client has requested that the device IP

address be reachable only from the internal network. Which type of IPv6 address must the engineer

assign?

A.  unique local address

B.  link-local address

C.  aggregatable global address

D.  IPv4-compatible IPv6 address

Answer: B

──────────────────────────────────────────────────

## Question 503  (Topic 3)  —  Multiple Choice  [V80.2]

A network administrator is setting up a new IPv6 network using the 64-bit address 2001 0EB8 00C1

2200:0001 0000 0000 0331/64 To simplify the configuration the administrator has decided to compress the

address Which IP address must the administrator configure?

A.  ipv6 address 21:EB8:C1:2200:1::331/64

B.  ipv6 address 2001:EB8:C1:22:1::331/64

C.  ipv6 address 2001 :EB8:C 1:2200.1 ::331-64

D.  ipv6 address 2001:EB8:C1:2200:1:0000:331/64

Answer: C

──────────────────────────────────────────────────

## Question 510  (Topic 3)  —  Multiple Choice  [V80.2]

A network engineer must implement an IPv6 configuration on the vlan 2000 interface to create a routable

locally-unique unicast address that is blocked from being advertised to the internet. Which configuration

must the engineer apply?

A.  interface vlan 2000ipv6 address ffc0:0000:aaaa::1234:2343/64

B.  interface vlan 2000Ipv6 address fc00:0000:aaaa:a15d:1234:2343:8aca/64

C.  interface vlan 2000ipv6 address fe80;0000:aaaa::1234:2343/64

D.  interface vlan 2000ipv6 address fd00::1234:2343/64

Answer: B

──────────────────────────────────────────────────

## Question 580  (Topic 4)  —  Multiple Choice  [V80.2]

In which circumstance would a network architect decide to implement a global unicast subnet instead of a

unique local unicast subnet?

A.  when the subnet must be available only within an organization

B.  when the subnet does not need to be routable

C.  when the addresses on the subnet must be equivalent to private IPv4 addresses

D.  when the subnet must be routable over the internet

Answer: D

──────────────────────────────────────────────────

## Question 587  (Topic 4)  —  Multiple Choice  [V80.2]

What is a link-local all-nodes IPv6 multicast address?

A.  ff02:0:0:0:0:0:0:1

B.  2004:31c:73d9:683e:255::

C.  fffe:034:0dd:45d6:789e::

D.  fe80:4433:034:0dd::2

Answer: D

──────────────────────────────────────────────────

## Question 638  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the IPv6 address details from the left onto the corresponding types on the right.

──────────────────────────────────────────────────

## Question 648  (Topic 4)  —  Multiple Choice  [V80.2]

A network engineer must configure the router R1 GigabitEthernet1/1 interface to connect to the router R2

GigabitEthernet1/1 interface. For the configuration to be applied, the engineer must compress the address

2001:0db8:0000:0000:0500:000a:400F:583B. Which command must be issued on the interface?

A.  ipv6 address 2001:db8 :: 500:a:400F:583B

B.  ipv6 address 2001:0db8 :: 5:a:4F:583B

C.  ipv6 address 2001 :: db8:0000 :: 500:a:400F:583B

D.  ipv6 address 2001:db8:0 :: 500:a:4F:583B

Answer: A

──────────────────────────────────────────────────

## Question 657  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 675  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 683  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 688  (Topic 4)  —  Multiple Choice  [V80.2]

What is a function of the core and distribution layers in a collapsed-core architecture?

A.  The router must use IPv4 and IPv6 addresses at Layer 3.

B.  The core and distribution layers are deployed on two different devices to enable failover.

C.  The router can support HSRP for Layer 2 redundancy in an IPv6 network.

D.  The router operates on a single device or a redundant pair.

Answer: D

──────────────────────────────────────────────────

## Question 689  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The loopback1 interface of the Atlanta router must reach the lookback3 interface of the Washington router.

A.  ipv6 route 2000::1/128 2012::2

B.  ipv6 route 2000::1/128 2012::1

C.  ipv6 route 2000:3 123 s0/0/0

D.  ipv6 route 2000::3/128 2023::3

E.  ipv6 route 2000::1/128 s0/0/1

Answer: B,D

──────────────────────────────────────────────────

## Question 726  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the IPv6 address types from the left onto their description on the right.

──────────────────────────────────────────────────

## Question 728  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An engineer configured the New York router with static routes that point to the Atlanta

and Washington sites. Which command must be configured on the Atlanta and Washington routers so that

both sites are able to reach the Lo2 interface on the New York router?

A.  ipv6 route ::/0 Serial 0/0/0

B.  ipv6 route 0.0.0.0 0.0.0.0 Serial 0/0/0

C.  ipv6 route ::/0 2000::2

D.  ipv6 route ::/0 Serial 0/0/1

Answer: A

──────────────────────────────────────────────────

## Question 732  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Refer to the exhibit. The IPv6 address for the LAN segment on router R1 must be configured using the

EUI-64 format. When configured which ipv6 address is produced by the router?

A.  2001:db8:1a44:41a4:C801:BEFF:FE4A:1

B.  2001:db8:1a44:41a4:C081:BFFF:FE4A:1

C.  2001:db8:1a44:41a4:4562:098F:FE36:1

D.  2001:db8:1a44:41a4:C800:BAFE:FF00:1

Answer: B

──────────────────────────────────────────────────

## Question 737  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which format matches the Modified EUI-64 IPv6 interface address for the network 2001:db8::/64?

A.  2001 :db8::5000:0004:5678:0090/64

B.  2001 :db8:4425:5400:77ft:fe07:/64

C.  2001 :db8::5000:00ff:fe04 0000/64

D.  2001 :db8::5200:00ff:fe04:0000/64

Answer: C

──────────────────────────────────────────────────

## Question 761  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 763  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. IPv6 must be implemented on R1 to the ISP The uplink between R1 and the ISP must

be configured with a manual assignment, and the LAN interface must be self-provisioned Both connections

must use the applicable IPv6 networks Which two configurations must be applied to R1? (Choose two.)

A.  interface Gi0/1ipv6 address 2001:db8:0F1B:FCCB:ACCE:FCED:ABCD:FA02:/127

B.  interface Gi0/0ipv6 address 2001:db8:1:AFFF::/64 eui-64

C.  interface Gi0/1ipv6 address 2001:db8:0F1B:FCCB:ACCE:FCED:ABCD:FA00:/127

D.  interface Gi0/0ipv6 address 2001:db8:0:AFFF::/64 eui-64

E.  interface Gi0/0ipv6 address 2001:db8:0F1B:FCCB:ACCE:FCED:ABCD:FA03;/127

Answer: C,D

──────────────────────────────────────────────────

## Question 794  (Topic 4)  —  Multiple Choice  [V80.2]

Which IPv6 address range is suitable for anycast addresses for distributed services such DHCP or DNS?

A.  FF00:1/12

B.  2001:db8:0234:ca3e::1/128

C.  2002:db84:3f37:ca98:be05:8/64

D.  FE80::1/10

Answer: A

──────────────────────────────────────────────────

## Question 795  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the IPv6 address description from the left onto the IPv6 address types on the right. Not all

options are used.

──────────────────────────────────────────────────

## Question 796  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 807  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The New York router is configured with static routes pointing to the Atlanta and Washington sites. Which

two tasks must be performed so that the Se0/0/0 interfaces on the Atlanta and Washington routers reach

one another? (Choose two.)

A.  Configure the ipv6 route 2023::/126 2012::2 command on the Atlanta router.

B.  Configure the ipv6 route 2012::/126 2023::1 command on the Washington router.

C.  Configure the ipv6 route 2023::/126 2012::1 command on the Atlanta router.

D.  Configure the ipv6 route 2012::/126 s0/0/0 command on the Atlanta router.

E.  Configure the ipv6 route 2012::/126 2023::2 command on the Washington router.

Answer: A,D

──────────────────────────────────────────────────

## Question 822  (Topic 4)  —  Multiple Choice  [V80.2]

What is a valid IPv6 address record in DNS?

A.  A

B.  MX

C.  AAAA

D.  CNAME

Answer: C

──────────────────────────────────────────────────

## Question 826  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 836  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The administrator must configure a floating sialic default route that points to 2001:db8:1234:2::1 and

replaces the current default route only if it fails. Which command must the engineer configure on the CPE?

A.  ipv6 route ::/0 2001:db8:1234:2::1 3

B.  ipv6 route ::/128 2001 :db8:1234:2::1 3

C.  ipv6 route ::/0 2001:db8:1234:2::1 1

D.  ipv6 route ::/0 2001:db8:1234:2::1 2

Answer: B

──────────────────────────────────────────────────

## Question 839  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 840  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 862  (Topic 4)  —  Multiple Choice  [V80.2]

A network engineer must migrate a router loopback interface to the IPv6 address space. If the current IPv4

address of the interface is 10.54.73.1/32, and the engineer configures IPv6 address 0.0.0.0.0:ffff:a36:4901,

which prefix length must be used?

A.  /64

B.  /96

C.  /124

D.  /128

Answer: D

──────────────────────────────────────────────────

## Question 881  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit An IPv6 address must be obtained automatically on the LAN interface on R1 Which

command must be implemented to accomplish the task?

A.  Ipv6 address 2001:dbB:d8d2:1008:4343:61:0010::/64

B.  Ipv6 address autoconfig

C.  Ipv6 address fe80::/10

D.  Ipv6 address dhcp

Answer: B

──────────────────────────────────────────────────

## Question 888  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 894  (Topic 4)  —  Multiple Choice  [V80.2]

What is the difference between an IPv6 link-local address and a unique local address?

A.  The scope of an IPv6 link-local address is limited to a loopback address, and an IPv6 unique local

address is limited to a directly attached interface.

B.  The scope of an IPv6 link-local address can be used throughout a company site or network, but an IPv6

unique local address is limited to a loopback address.

C.  The scope of an IPv6 link-local address is global, but the scope of an IPv6 unique local address is limited

to a loopback address.

D.  The scope of an IPv6 link-local address is limited to a directly attached interface, but an IPv6 unique

local address is used throughout a company site or network.

Answer: B

──────────────────────────────────────────────────

## Question 908  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer is configuring router R1 with an IPv6 static route for prefix 2019:C15C:0CAF:E001::/64. The

next hop must be 2019:C15C:0CAF:E002::1 The route must be reachable via the R1 Gigabit 0/0 interface.

Which command configures the designated route?

A.  R1(config)#ipv6 route 2019:C15C:0CAF:E001::/64 2019:C15C:0CAF:E002::1

B.  R1(config-if)#ipv6 route 2019:C15C:0CAF:E001::/64 2019:C15C:0CAF:E002::1

C.  R1(config-if)#ip route 2019:C15C:0CAF:E001::/64 GigabitEthernet0/0

D.  R1(config)#ip route 2019:C15C:0CAF:E001::/64 GigabitEthernet0/0

Answer: C

──────────────────────────────────────────────────

## Question 927  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 987  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the IPv6 addresses from the left onto the corresponding address types on the right.

──────────────────────────────────────────────────

## Question 1000  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 1016  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. IPv6 is being Implemented within the enterprise. The command Ipv6 unlcast-routing is

configure. Interlace GlgO/0 on R1 must be configured to provide a dynamic assignment using the assigned

IPv6 block Which command accomplishes this task?

A.  ipv6 address 2001:DB8:FFFF:FCF3::1/64

B.  ipv6 address autoconfig 2001:DB8:FFFF:FCF2::/64

C.  ipv6 address 2001:DB8:FFFF:FCF3::/64 eui-64

D.  ipv6 address 2001:DB8:FFFF:FCF3::/64 link-local

Answer: C

──────────────────────────────────────────────────

## Question 1043  (Topic 4)  —  Multiple Choice  [V80.2]

Which type of IPv4 address must be assigned to a server to protect it from external access and allow only

internal users access while restricting internet access?

A.  global unicast

B.  public

C.  private

D.  multicast

Answer: C

──────────────────────────────────────────────────

## Question 1048  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer is configuring a WLAN to connect with the 172.16.10.0/24 network on VLAN 20. The

engineer wants to limit the number of devices that connect to the WLAN on the USERWL SSID to 125.

Which configuration must the engineer perform on the WLC?

A.  In the Management Software activation configuration, set the Clients value to 125.

B.  In the Controller IPv6 configuration, set the Throttle value to 125.

C.  In the WLAN configuration, set the Maximum Allowed Clients value to 125.

D.  In the Advanced configuration, set the DTIM value to 125.

Answer: C

──────────────────────────────────────────────────

## Question 1049  (Topic 4)  —  Multiple Choice  [V80.2]

Which two IPv6 addresses are used to provide connectivity between two routers on a shared link? (Choose

two)

A.  ::ffif 1014 1011/96

B.  2001 7011046:1111:1/64

C.  ;jff06bb43cd4dd111bbff02 4545234d

D.  2002 5121204b 1111:1/64

E.  FF02::0WlFF00:0l)00/104

Answer: B

──────────────────────────────────────────────────

## Question 1050  (Topic 4)  —  Multiple Choice  [V80.2]

What is a difference between an IPv6 multicast address and an IPv6 anycast address?

A.  A packet sent to an IPv6 multicast address is delivered to one or more destinations at once, but a packet

sent to an IPv6 anycast address is routed to the closest interface with that address.

B.  An IPv6 multicast address uses the prefix 2002::/15 and forwards to one destination, and an IPv6

anycast address uses the prefix ff00::/8 and forwards to any destination in a group.

C.  IPv6 multicast addresses are used to transition from IPv4 to IPv6, and IPv6 anycast addresses are used

for address aggregation in an IPv6-only environment.

D.  An IPv6 multicast address is assigned to numerous interfaces within a subnet, but an IPv6 anycast

address is used for a predefined group of nodes in an all-IPv6 routers group.

Answer: A

──────────────────────────────────────────────────

## Question 1054  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The New York router must be configured so that traffic to 2000::1 is sent primarily via the Atlanta site, with a

secondary path via Washington that has an administrative distance of 2. Which two commands must be

configured on the New York router? (Choose two.)

A.  ipv6 route 2000::1/128 2012::1

B.  ipv6 route 2000::1/128 2023::2 5

C.  ipv6 route 2000::1/128 2012::1 5

D.  ipv6 route 2000::1/128 2023::3 2

E.  ipv6 route 2000::1/128 2012::2

Answer: D,E

──────────────────────────────────────────────────

## Question 1070  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Which two commands, when configured on router R1. fulfill these requirements?

(Choose two.) ' Packets toward the entire network 2001:db8:23: :/64 must be forwarded through router R2. '

Packets toward host 2001: db8:23::14 preferably must be forwarded through R3.

A.  ipv6 route 2001:db8:23: :/128 fd00:12::2

B.  Ipv6 route 2001:db8:23::14/128 fd00:13::3

C.  ipv6 route 2001:db8:23::14/64 fd00:12::2 200

D.  ipv6 route 2001:db8:23: l4/64 fd00:12::2

E.  ipv6 route 2001:db8:23: :/64 fd00:12::2

Answer: B,E

──────────────────────────────────────────────────

## Question 1151  (Topic 5)  —  Multiple Choice  [V80.2]

What is a similarity between global and unique local IPv6 addresses?

A.  They are allocated by the same organization.

B.  They are routable on the global internet.

C.  They use the same process for subnetting.

D.  They are part of the multicast IPv6 group type.

Answer: C

──────────────────────────────────────────────────

## Question 1187  (Topic 5)  —  Drag & Drop  [V80.2]

Drag and drop the IPv6 address from the left onto the type on the right.

──────────────────────────────────────────────────

## Question 1198  (Topic 5)  —  Drag & Drop  [V80.2]

Drag and drop the characteristic from the left onto the IPv6 address type on the right.

──────────────────────────────────────────────────

## Question 1205  (Topic 5)  —  Drag & Drop  [V80.2]

Drag and drop the IPv6 address from the left onto the type on the right.

──────────────────────────────────────────────────

## Question 1213  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. The routers R1-LAB and R2-LAB are configured with link-local addresses. What

command must be applied to interface Gi0/0 on R1-LAB for an automated address self-assignment on the

IPv6 network?

A.  ipv6 address 2001:db8:1:0FFA:0::/64

B.  ipv6 address 2001:db8:0:0FFA::1/64

C.  ipv6 address 2001:db8:0:0FFA::/64 eui-64

D.  ipv6 address 2001:db8:0:0FFA::/64 anycast

Answer: C