# CCNA 200-301
NTP

Network Time Protocol and clock management

14 Questions

Multiple Choice: 14 | Drag & Drop: 0

Sources: V7.1 (3) | V80.2 (11)

Topic 1: 3 | Topic 2: 4 | Topic 3: 2 | Topic 4: 4 | Topic 5: 1

## Question 4  (Topic 3)  —  Multiple Choice  [V7.1]

R1 as an NTP server must have:

-   NTP authentication enabled

-   NTP packets sourced from Interface loopback 0

-   NTP stratum 2

-   NTP packets only permitted to client IP 209.165 200 225 How should

R1 be configured?

A\)

[See Exhibit Image]

[See Exhibit Image]B)

C\)

[See Exhibit Image]

[See Exhibit Image]D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 235  (Topic 2)  —  Multiple Choice  [V7.1]

Which command must be entered when a device is configured as an NTP

server?

A.  ntp authenticate

B.  ntp server

C.  ntp peer

D.  ntp master

Answer: D

Explanation:

To configure a Cisco device as an Authoritative NTP Server, use the

ntp master \[stratum\] command.To configure a Cisco device as a NTP

client, use the command ntp server \<IP address\>. For

example:Router(config)#ntp server 192.168.1.1. This command will

instruct the router to query 192.168.1.1 for the time.

──────────────────────────────────────────────────

## Question 577  (Topic 1)  —  Multiple Choice  [V7.1]

What facilitates a Telnet connection between devices by entering the

device name?

A.  SNMP

B.  DNS lookup

C.  syslog

D.  NTP

Answer: B

──────────────────────────────────────────────────

## Question 9  (Topic 1)  —  Multiple Choice  [V80.2]

A network analyst is tasked with configured the date and time on a router using EXEC mode. The date must

be set to 12:00am. Which command should be used?

A.  Clock timezone

B.  Clock summer-time-recurring

C.  Clock summer-time date

D.  Clock set

Answer: D

──────────────────────────────────────────────────

## Question 64  (Topic 1)  —  Multiple Choice  [V80.2]

What facilitates a Telnet connection between devices by entering the device name?

A.  SNMP

B.  DNS lookup

C.  syslog

D.  NTP

Answer: B

──────────────────────────────────────────────────

## Question 274  (Topic 2)  —  Multiple Choice  [V80.2]

Which command must be entered when a device is configured as an NTP server?

A.  ntp authenticate

B.  ntp server

C.  ntp peer

D.  ntp master

Answer: D

──────────────────────────────────────────────────

## Question 329  (Topic 2)  —  Multiple Choice  [V80.2]

Which two tasks must be performed to configure NTP to a trusted server in client mode on a single network

device? (Choose two)

A.  Enable NTP authentication.

B.  Verify the time zone.

C.  Disable NTP broadcasts

D.  Specify the IP address of the NTP server

E.  Set the NTP server private key

Answer: A,D

──────────────────────────────────────────────────

## Question 382  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The nip server 192.168.0.3 command has been configured on router 1 to make it an NTP client of router 2.

Which command must be configured on router 2 so that it operates in server-only mode and relies only on

its internal clock?

A.  Router2(config)#ntp passive

B.  Router2(config)#ntp server 172.17.0.1

C.  Router2(config)#ntp master 4

D.  Router2(config)#ntp server 192.168.0.2

Answer: B

──────────────────────────────────────────────────

## Question 526  (Topic 3)  —  Multiple Choice  [V80.2]

R1 as an NTP server must have:

• NTP authentication enabled

• NTP packets sourced from Interface loopback 0

• NTP stratum 2

• NTP packets only permitted to client IP 209.165 200 225 How should R1 be configured?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 668  (Topic 4)  —  Multiple Choice  [V80.2]

A network analyst is tasked with configuring the date and time on a router using EXEC mode. The date

must be set to January 1. 2020 and the time must be set to 12:00 am. Which command should be used?

A.  clock summer-time recurring

B.  clock timezone

C.  clock summer-time date

D.  clock set

Answer: D

──────────────────────────────────────────────────

## Question 681  (Topic 4)  —  Multiple Choice  [V80.2]

Which protocol is implemented when an organization must verify network performance, troubleshoot issues,

and use an agent to communicate between monitoring tools and end devices?

A.  SNMP

B.  FTP

C.  NTP

D.  NFS

Answer: D

──────────────────────────────────────────────────

## Question 741  (Topic 4)  —  Multiple Choice  [V80.2]

A network engineer is configuring a new router at a branch office. The router is connected to an upstream

WAN network that allows the branch to communicate with the head office. The central time server with IP

address 172.24.54.8 is located behind a firewall at the head office. Which command must the engineer

configure so that the software clock of the new router synchronizes with the time server?

A.  ntp master 172.24.54.8

B.  ntp client 172.24.54.8

C.  ntp peer 172.24.54.8

D.  ntp server 172.24.54.8

Answer: D

──────────────────────────────────────────────────

## Question 767  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer is configuring switch SW1 to act an NTP server when all upstream NTP server connectivity

fails. Which configuration must be used?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 1194  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A network engineer must replicate the AccessSw1 NTP configuration on a new switch.

The engineer could not access privileged mode on AccessSw1 to view its configuration.

Which command must be applied to the new switch to replicate the configuration?

A.  ntp master

B.  ntp master 3

C.  ntp server 2001:db8:12::1

D.  ntp server 127.127.1.1

Answer: C