# CCNA 200-301
Network Architecture

Network architecture, three-tier, spine-and-leaf, campus design

23 Questions

Multiple Choice: 23 | Drag & Drop: 0

Sources: V7.1 (6) | V80.2 (17)

Topic 1: 13 | Topic 2: 5 | Topic 3: 1 | Topic 4: 3 | Topic 5: 1

## Question 207  (Topic 2)  —  Multiple Choice  [V7.1]

What are two descriptions of three-tier network topologies? (Choose

two)

A.  The core and distribution layers perform the same functions

B.  The access layer manages routing between devices in different

domains

C.  The network core is designed to maintain continuous connectivity

when devices fail.

D.  The core layer maintains wired connections for each host

E.  The distribution layer runs Layer 2 and Layer 3 technologies

Answer: CE

──────────────────────────────────────────────────

## Question 262  (Topic 2)  —  Multiple Choice  [V7.1]

What benefit does controller-based networking provide versus

traditional networking?

A.  moves from a two-tier to a three-tier network architecture to

provide maximum redundancy

B.  provides an added layer of security to protect from DDoS attacks

C.  allows configuration and monitoring of the network from one

centralized port

D.  combines control and data plane functionality on a single device to

minimize latency

Answer: C

──────────────────────────────────────────────────

## Question 388  (Topic 1)  —  Multiple Choice  [V7.1]

in Which way does a spine and-leaf architecture allow for scalability

in a network when additional access ports are required?

A.  A spine switch and a leaf switch can be added with redundant

connections between them

B.  A spine switch can be added with at least 40 GB uplinks

C.  A leaf switch can be added with a single connection to a core spine

switch.

D.  A leaf switch can be added with connections to every spine switch

Answer: D

Explanation:

Spine-leaf architecture is typically deployed as two layers: spines

(such as an aggregation layer), and leaves (such as an access layer).

Spine-leaf topologies provide high-bandwidth, low-latency, nonblocking

server-to-server connectivity.

Leaf (aggregation) switches are what provide devices access to the

fabric (the network of spine and leaf switches) and are typically

deployed at the top of the rack. Generally, devices connect to the

leaf switches.

Devices can include servers, Layer 4-7 services (firewalls and load

balancers), and WAN or Internet routers. Leaf switches do not connect

to other leaf switches. In spine-and-leaf architecture, every leaf

should connect to every spine in a full mesh.

Spine (aggregation) switches are used to connect to all leaf switches

and are typically deployed at the end or middle of the row. Spine

switches do not connect to other spine switches.

──────────────────────────────────────────────────

## Question 444  (Topic 1)  —  Multiple Choice  [V7.1]

Which function is performed by the collapsed core layer in a two-tier

architecture?

A.  enforcing routing policies

B.  marking interesting traffic for data polices

C.  attaching users to the edge of the network

D.  applying security policies

Answer: A

──────────────────────────────────────────────────

## Question 469  (Topic 1)  —  Multiple Choice  [V7.1]

What are two characteristics of the distribution layer in a three-tier

network architecture? (Choose two.)

A.  serves as the network aggregation point

B.  provides a boundary between Layer 2 and Layer 3 communications

C.  designed to meet continuous, redundant uptime requirements

D.  is the backbone for the network topology

E.  physical connection point for a LAN printer

Answer: BC

──────────────────────────────────────────────────

## Question 546  (Topic 1)  —  Multiple Choice  [V7.1]

What is a characteristic of a SOHO network?

A.  connects each switch to every other switch in the network

B.  enables multiple users to share a single broadband connection

C.  provides high throughput access for 1000 or more users

D.  includes at least three tiers of devices to provide load balancing

and redundancy

Answer: B

──────────────────────────────────────────────────

## Question 24  (Topic 1)  —  Multiple Choice  [V80.2]

Which access layer threat-mitigation technique provides security based on identity?

A.  Dynamic ARP Inspection

B.  using a non-default native VLAN

C.  802.1x

D.  DHCP snooping

Answer: C

──────────────────────────────────────────────────

## Question 38  (Topic 1)  —  Multiple Choice  [V80.2]

What are two characteristics of the distribution layer in a three-tier network architecture? (Choose two.)

A.  serves as the network aggregation point

B.  provides a boundary between Layer 2 and Layer 3 communications

C.  designed to meet continuous, redundant uptime requirements

D.  is the backbone for the network topology

E.  physical connection point for a LAN printer

Answer: B,C

──────────────────────────────────────────────────

## Question 84  (Topic 1)  —  Multiple Choice  [V80.2]

How are the switches in a spine-and-leaf topology interconnected?

A.  Each leaf switch is connected to one of the spine switches.

B.  Each leaf switch is connected to two spine switches, making a loop.

C.  Each leaf switch is connected to each spine switch.

D.  Each leaf switch is connected to a central leaf switch, then uplinked to a core spine switch.

Answer: C

──────────────────────────────────────────────────

## Question 105  (Topic 1)  —  Multiple Choice  [V80.2]

Which two WAN architecture options help a business scalability and reliability for the network? (Choose

two)

A.  asychronous routing

B.  single-homed branches

C.  dual-homed branches

D.  static routing

E.  dynamic routing

Answer: A,C

──────────────────────────────────────────────────

## Question 106  (Topic 1)  —  Multiple Choice  [V80.2]

What is a characteristic of a SOHO network?

A.  connects each switch to every other switch in the network

B.  enables multiple users to share a single broadband connection

C.  provides high throughput access for 1000 or more users

D.  includes at least three tiers of devices to provide load balancing and redundancy

Answer: B

──────────────────────────────────────────────────

## Question 121  (Topic 1)  —  Multiple Choice  [V80.2]

Which two functions are performed by the core layer in a three-tier architecture? (Choose two)

A.  Provide uninterrupted forwarding service.

B.  Police traffic that is sent to the edge of the network.

C.  Provide direct connectivity for end user devices.

D.  Ensure timely data transfer between layers.

E.  Inspect packets for malicious activity.

Answer: A,D

──────────────────────────────────────────────────

## Question 142  (Topic 1)  —  Multiple Choice  [V80.2]

in Which way does a spine and-leaf architecture allow for scalability in a network when additional access

ports are required?

A.  A spine switch and a leaf switch can be added with redundant connections between them

B.  A spine switch can be added with at least 40 GB uplinks

C.  A leaf switch can be added with a single connection to a core spine switch.

D.  A leaf switch can be added with connections to every spine switch

Answer: D

──────────────────────────────────────────────────

## Question 168  (Topic 1)  —  Multiple Choice  [V80.2]

Which two WAN architecture options help a business improve scalability and reliability for the network?

(Choose two.)

A.  asynchronous routing

B.  single-homed branches

C.  dual-homed branches

D.  static routing

E.  dynamic routing

Answer: A,C

──────────────────────────────────────────────────

## Question 234  (Topic 1)  —  Multiple Choice  [V80.2]

Which function is performed by the collapsed core layer in a two-tier architecture?

A.  enforcing routing policies

B.  marking interesting traffic for data polices

C.  attaching users to the edge of the network

D.  applying security policies

Answer: A

──────────────────────────────────────────────────

## Question 296  (Topic 2)  —  Multiple Choice  [V80.2]

What are two descriptions of three-tier network topologies? (Choose two)

A.  The core and distribution layers perform the same functions

B.  The access layer manages routing between devices in different domains

C.  The network core is designed to maintain continuous connectivity when devices fail.

D.  The core layer maintains wired connections for each host

E.  The distribution layer runs Layer 2 and Layer 3 technologies

Answer: C,E

──────────────────────────────────────────────────

## Question 327  (Topic 2)  —  Multiple Choice  [V80.2]

What benefit does controller-based networking provide versus traditional networking?

A.  moves from a two-tier to a three-tier network architecture to provide maximum redundancy

B.  provides an added layer of security to protect from DDoS attacks

C.  allows configuration and monitoring of the network from one centralized port

D.  combines control and data plane functionality on a single device to minimize latency

Answer: C

──────────────────────────────────────────────────

## Question 385  (Topic 2)  —  Multiple Choice  [V80.2]

What is a characteristic of spine-and-leaf architecture?

A.  Each device is separated by the same number of hops

B.  It provides variable latency

C.  It provides greater predictability on STP blocked ports.

D.  Each link between leaf switches allows for higher bandwidth.

Answer: A

──────────────────────────────────────────────────

## Question 452  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which plan must be Implemented to ensure optimal QoS marking practices on this network?

A.  As traffic traverses MLS1 remark the traffic, but trust all markings at the access layer.

B.  Trust the IP phone markings on SW1 and mark traffic entering SW2 at SW2.

C.  Remark traffic as it traverses R1 and trust all markings at the access layer.

D.  As traffic enters from the access layer on SW1 and SW2. trust all traffic markings.

Answer: C

──────────────────────────────────────────────────

## Question 645  (Topic 4)  —  Multiple Choice  [V80.2]

What is the collapsed layer in collapsed core architectures?

A.  core and WAN

B.  access and WAN

C.  distribution and access

D.  core and distribution

Answer: D

──────────────────────────────────────────────────

## Question 665  (Topic 4)  —  Multiple Choice  [V80.2]

What are two characteristics of a small office / home office connection environment? (Choose two.)

A.  It requires 10Gb ports on all uplinks.

B.  It supports between 50 and 100 users.

C.  It supports between 1 and 50 users.

D.  It requires a core, distribution, and access layer architecture.

E.  A router port connects to a broadband connection.

Answer: C,E

──────────────────────────────────────────────────

## Question 952  (Topic 4)  —  Multiple Choice  [V80.2]

What is a characteristics of a collapsed-core network topology?

A.  It allows the core and distribution layers to run as a single combined layer.

B.  It enables the core and access layers to connect to one logical distribution device over

an EtherChannel.

C.  It enables all workstations in a SOHO environment to connect on a single switch with internet access.

D.  It allows wireless devices to connect directly to the core layer, which enables faster data transmission.

Answer: B

──────────────────────────────────────────────────

## Question 1188  (Topic 5)  —  Multiple Choice  [V80.2]

In which way does a spine-and-leaf architecture allow for scalability in a network when additional access

ports are required?

A.  A spine switch and a leaf switch are added with redundant connections between them.

B.  A spine switch is added with at least 40GB uplinks.

C.  A leaf switch is added with a single connection to a core spine switch.

D.  A leaf switch is added with connections to every spine switch.

Answer: D