# CCNA 200-301
Network Security

AAA, port security, 802.1X, threats, and firewalls

157 Questions

Multiple Choice: 135 | Drag & Drop: 22

Sources: V7.1 (25) | V80.2 (132)

Topic 1: 42 | Topic 2: 26 | Topic 3: 17 | Topic 4: 52 | Topic 5: 20

## Question 1  (Topic 3)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

The DHCP server and clients are connected to the same switch. What is

the next step to complete the DHCP configuration to allow clients on

VLAN 1 to receive addresses from the DHCP server?

A.  Configure the ip dhcp snooping trust command on the interlace that

is connected to the DHCP client.

B.  Configure the ip dhcp relay information option command on the

interface that is connected to the DHCP client.

C.  Configure the ip dhcp snooping trust command on the interface that

is connected to the DHCP server.

D.  Configure the Ip dhcp relay information option command on the

interface that is connected to the DHCP server.

Answer: C

──────────────────────────────────────────────────

## Question 2  (Topic 3)  —  Multiple Choice  [V7.1]

What causes a port to be placed in the err-disabled state?

A.  nothing plugged into the port

B.  link flapping

C.  shutdown command issued on the port

D.  latency

Answer: B

──────────────────────────────────────────────────

## Question 60  (Topic 3)  —  Multiple Choice  [V7.1]

What is a function of a Next-Generation IPS?

A.  makes forwarding decisions based on learned MAC addresses

B.  serves as a controller within a controller-based network

C.  integrates with a RADIUS server to enforce Layer 2 device

authentication rules

D.  correlates user activity with network events

Answer: D

──────────────────────────────────────────────────

## Question 61  (Topic 3)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 3)

An engineer is tasked to configure a switch with port security to

ensure devices that forward unicasts multicasts and broadcasts are

unable to flood the port The port must be configured to permit only

two random MAC addresses at a time Drag and drop the required

configuration commands from the left onto the sequence on the right

Not all commands are used.

[See Exhibit Image]

A.  Mastered

B.  Not Mastered **Answer:** A **Explanation:**

[See Exhibit Image]

──────────────────────────────────────────────────

## Question 69  (Topic 2)  —  Multiple Choice  [V7.1]

What is the primary different between AAA authentication and

authorization?

A.  Authentication verifies a username and password, and authorization

handles the communication between the authentication agent and the

user database.

B.  Authentication identifies a user who is attempting to access a

system, and authorization validates the users password

C.  Authentication identifies and verifies a user who is attempting to

access a system, and authorization controls the tasks the user can

perform.

D.  Authentication controls the system processes a user can access and

authorization logs the activities the user initiates

Answer: C

Explanation:

AAA stands for Authentication, Authorization and Accounting.+

Authentication: Specify who you are (usually via login username &

password)+ Authorization: Specify what actions you can do, what

resource you can access+ Accounting: Monitor what you do, how long you

do it (can be used for billing and auditing)An example of AAA is shown

below:+ Authentication: "I am a normal user. My username/password is

user_tom/learnforever"+ Authorization: "user_tom can access LearnCCNA

server via HTTP and FTP"+ Accounting: "user_tom accessed LearnCCNA

server for 2 hours". This user only uses "show" commands.

──────────────────────────────────────────────────

## Question 125  (Topic 2)  —  Multiple Choice  [V7.1]

Which function is performed by DHCP snooping?

A.  propagates VLAN information between switches

B.  listens to multicast traffic for packet forwarding

C.  provides DDoS mitigation

D.  rate-limits certain traffic

Answer: A

──────────────────────────────────────────────────

## Question 171  (Topic 2)  —  Multiple Choice  [V7.1]

Which action is taken by a switch port enabled for PoE power

classification override?

A.  When a powered device begins drawing power from a PoE switch port a

syslog message is generated

B.  As power usage on a PoE switch port is checked data flow to the

connected device is temporarily paused

C.  If a switch determines that a device is using less than the minimum

configured power it assumes the device has failed and disconnects

D.  Should a monitored port exceeds the maximum administrative value for

power, the port is shutdown and err-disabled

Answer: D

Explanation:

Reference:

[https://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst6500/ios/12-2SX/configuration/guide/book/power_over_ethernet.pdf](http://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst6500/ios/12-2SX/configuration/guide/book/power_over_ethernet.pdf)

PoE monitoring and policing compares the power consumption on ports

with the administrative maximum value (either a configured maximum

value or the port's default value). If the power consumption on a

monitored port exceeds the administrative maximum value, the following

actions occur:-- A syslog message is issued.-- The monitored port is

shut down and error-disabled.-- The allocated power is freed.

──────────────────────────────────────────────────

## Question 208  (Topic 2)  —  Multiple Choice  [V7.1]

Which protocol prompts the Wireless LAN Controller to generate its own

local web administration SSL certificate for GUI access?

A.  HTTPS

B.  RADIUS

C.  TACACS+

D.  HTTP

Answer: A

Explanation:

Reference:

[https://www.cisco.com/c/en/us/td/docs/wireless/controller/8-0/configuration-guide/b_cg80/b_cg80_chapter_011.html](http://www.cisco.com/c/en/us/td/docs/wireless/controller/8-0/configuration-guide/b_cg80/b_cg80_chapter_011.html)

──────────────────────────────────────────────────

## Question 211  (Topic 2)  —  Multiple Choice  [V7.1]

How do AAA operations compare regarding user identification, user

services and access control?

A.  Authorization provides access control and authentication tracks user

services

B.  Authentication identifies users and accounting tracks user services

C.  Accounting tracks user services, and authentication provides access

control

D.  Authorization identifies users and authentication provides access

control

Answer: B

──────────────────────────────────────────────────

## Question 238  (Topic 2)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 2)

[See Exhibit Image]Drag and drop the AAA terms from the left

onto the description on the right.

A.  Mastered

B.  Not Mastered

Answer: A

Explanation:

1-1, 2-4, 3-3, 4-2

──────────────────────────────────────────────────

## Question 249  (Topic 2)  —  Multiple Choice  [V7.1]

An administrator must secure the WLC from receiving spoofed

association requests. Which steps must be taken to configure the WLC

to restrict the requests and force the user to wait 10 ms to retry an

association request?

A.  Enable Security Association Teardown Protection and set the SA Query

timeout to 10

B.  Enable MAC filtering and set the SA Query timeout to 10

C.  Enable 802.1x Layer 2 security and set me Comeback timer to 10

D.  Enable the Protected Management Frame service and set the Comeback

timer to 10

Answer: C

──────────────────────────────────────────────────

## Question 262  (Topic 2)  —  Multiple Choice  [V7.1]

What benefit does controller-based networking provide versus

traditional networking?

A.  moves from a two-tier to a three-tier network architecture to

provide maximum redundancy

B.  provides an added layer of security to protect from DDoS attacks

C.  allows configuration and monitoring of the network from one

centralized port

D.  combines control and data plane functionality on a single device to

minimize latency

Answer: C

──────────────────────────────────────────────────

## Question 344  (Topic 1)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 1)

[See Exhibit Image]Drag and drop the threat-mitigation techniques from

the left onto the types of threat or attack they mitigate on the

right.

A.  Mastered

B.  Not Mastered

Answer: A

Explanation:

[See Exhibit Image]Double-Tagging attack:

In this attack, the attacking computer generates frames with two

802.1Q tags. The first tag matches the native VLAN of the trunk port

(VLAN 10 in this case), and the second matches the VLAN of a host it

wants to attack (VLAN 20).When the packet from the attacker reaches

Switch A, Switch A only sees the first VLAN 10 and it matches with its

native VLAN 10 so this VLAN tag is removed. Switch A forwards the

frame out all links with the same native VLAN 10. Switch B receives

the frame with an tag of VLAN 20 so it removes this tag and forwards

out to the Victim computer.Note: This attack only works if the trunk

(between two switches) has the same native VLAN as the attacker.To

mitigate this type of attack, you can use VLAN access control lists

(VACLs, which applies to all traffic within a VLAN. We can use VACL to

drop attacker traffic to specific victims/servers) or implement

Private VLANs.ARP attack (like ARP poisoning/spoofing) is a type of

attack in which a malicious actor sends falsified ARP messages over a

local area network as ARP allows a gratuitous reply from a host even

if an ARP request was not received. This results in the linking of an

attacker's MAC address with the IP address of a legitimate computer or

server on the network. This is an attack based on ARP which is at

Layer 2.Dynamic ARP inspection (DAI) is a security feature that

validates ARP packets in a network which can be used to mitigate this

type of attack.

──────────────────────────────────────────────────

## Question 353  (Topic 1)  —  Multiple Choice  [V7.1]

What are network endpoints?

A.  act as routers to connect a user to the service prowler network

B.  a threat to the network if they are compromised

C.  support inter-VLAN connectivity

D.  enforce policies for campus-wide traffic going to the internet

Answer: B

──────────────────────────────────────────────────

## Question 396  (Topic 1)  —  Multiple Choice  [V7.1]

What is a network appliance that checks the state of a packet to

determine whether the packet is legitimate?

A.  Layer 2 switch

B.  load balancer

C.  firewall

D.  LAN controller

Answer: C

──────────────────────────────────────────────────

## Question 403  (Topic 1)  —  Multiple Choice  [V7.1]

Which type of attack can be mitigated by dynamic ARP inspection?

A.  worm

B.  malware

C.  DDoS

D.  man-in-the-middle

Answer: D

──────────────────────────────────────────────────

## Question 420  (Topic 1)  —  Multiple Choice  [V7.1]

Which command prevents passwords from being stored in the

configuration as plain text on a router or switch?

A.  enable secret

B.  service password-encryption

C.  username Cisco password encrypt

D.  enable password

Answer: B

──────────────────────────────────────────────────

## Question 461  (Topic 1)  —  Multiple Choice  [V7.1]

Which device performs stateful inspection of traffic?

A.  firewall

B.  switch

C.  access point

D.  wireless controller

Answer: A

──────────────────────────────────────────────────

## Question 472  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

When PC-A sends traffic to PC-B, which network component is in charge

of receiving the packet from PC-A verifying the IP addresses, and

forwarding the packet to PC-B?

A.  Layer 2 switch

B.  Router

C.  Load balancer

D.  firewall

Answer: B

Explanation:

PC\--A and PC-B are not in the same network. Switches send traffic in

layer 2 and within the same VLA while routers route traffic to

different subnet and at layer 3.

──────────────────────────────────────────────────

## Question 481  (Topic 1)  —  Multiple Choice  [V7.1]

Which global command encrypt all passwords in the running

configuration?

A.  password-encrypt

B.  enable password-encryption

C.  enable secret

D.  service password-encryption

Answer: B

──────────────────────────────────────────────────

## Question 512  (Topic 1)  —  Multiple Choice  [V7.1]

Which feature on the Cisco Wireless LAN Controller when enabled

restricts management access from specific networks?

A.  CPU ACL

B.  TACACS

C.  Flex ACL

D.  RADIUS

Answer: A

Explanation:

Reference:

[https://www.cisco.com/c/en/us/support/docs/wireless-mobility/wlan-security/71978-acl-wlc.html](http://www.cisco.com/c/en/us/support/docs/wireless-mobility/wlan-security/71978-acl-wlc.html)

──────────────────────────────────────────────────

## Question 549  (Topic 1)  —  Multiple Choice  [V7.1]

What are two functions of an SDN controller? (Choose two)

A.  Layer 2 forwarding

B.  coordinating VTNs

C.  tracking hosts

D.  managing the topology

E.  protecting against DDoS attacks

Answer: BD

──────────────────────────────────────────────────

## Question 568  (Topic 1)  —  Multiple Choice  [V7.1]

Which device tracks the state of active connections in order to make a

decision to forward a packet through?

A.  wireless access point

B.  firewall

C.  wireless LAN controller

D.  router

Answer: B

Explanation:

Stateful inspection, also known as dynamic packet filtering, is a

firewall technology that monitors the state of active connections and

uses this information to determine which network packets to allow

through the firewall.

──────────────────────────────────────────────────

## Question 581  (Topic 1)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 1)

Drag and drop the attack-mitigation techniques from the left onto the

Types of attack that they mitigate on the right.

[See Exhibit Image]

A.  Mastered

B.  [See Exhibit Image]Not Mastered **Answer:** A

──────────────────────────────────────────────────

## Question 584  (Topic 1)  —  Multiple Choice  [V7.1]

Which type of security program is violated when a group of employees

enters a building using the ID badge of only one person?

A.  intrusion detection

B.  user awareness

C.  physical access control

D.  network authorization

Answer: C

──────────────────────────────────────────────────

## Question 2  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An extended ACL has been configured and applied to router R2 The configuration failed to work as

intended Which two changes stop outbound traffic on TCP ports 25 and 80 to 10.0.20 0 26 from the 10.0.10

0/26 subnet while still allowing all other traffic? (Choose two )

A.  Add a "permit ip any any" statement to the begining of ACL 101 for allowed traffic.

B.  Add a "permit ip any any" statement at the end of ACL 101 for allowed traffic

C.  The source and destination IPs must be swapped in ACL 101

D.  The ACL must be configured the Gi0/2 interface inbound on R1

E.  The ACL must be moved to the Gi0/1 interface outbound on R2

Answer: B,C

──────────────────────────────────────────────────

## Question 10  (Topic 1)  —  Multiple Choice  [V80.2]

Which type of security program is violated when a group of employees enters a building using the ID badge

of only one person?

A.  intrusion detection

B.  user awareness

C.  physical access control

D.  network authorization

Answer: C

──────────────────────────────────────────────────

## Question 14  (Topic 1)  —  Multiple Choice  [V80.2]

Which feature on the Cisco Wireless LAN Controller when enabled restricts management access from

specific networks?

A.  CPU ACL

B.  TACACS

C.  Flex ACL

D.  RADIUS

Answer: A

──────────────────────────────────────────────────

## Question 16  (Topic 1)  —  Multiple Choice  [V80.2]

An email user has been lured into clicking a link in an email sent by their company's security organization.

The webpage that opens reports that it was safe but the link could have contained malicious code. Which

type of security program is in place?

A.  Physical access control

B.  Social engineering attack

C.  brute force attack

D.  user awareness

Answer: D

──────────────────────────────────────────────────

## Question 23  (Topic 1)  —  Multiple Choice  [V80.2]

What is a practice that protects a network from VLAN hopping attacks?

A.  Enable dynamic ARP inspection

B.  Configure an ACL to prevent traffic from changing VLANs

C.  Change native VLAN to an unused VLAN ID

D.  Implement port security on internet-facing VLANs

Answer: C

──────────────────────────────────────────────────

## Question 24  (Topic 1)  —  Multiple Choice  [V80.2]

Which access layer threat-mitigation technique provides security based on identity?

A.  Dynamic ARP Inspection

B.  using a non-default native VLAN

C.  802.1x

D.  DHCP snooping

Answer: C

──────────────────────────────────────────────────

## Question 32  (Topic 1)  —  Multiple Choice  [V80.2]

What are two functions of an SDN controller? (Choose two)

A.  Layer 2 forwarding

B.  coordinating VTNs

C.  tracking hosts

D.  managing the topology

E.  protecting against DDoS attacks

Answer: B,D

──────────────────────────────────────────────────

## Question 33  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the DHCP snooping terms from the left onto the descriptions on the right.

──────────────────────────────────────────────────

## Question 42  (Topic 1)  —  Multiple Choice  [V80.2]

Which global command encrypt all passwords in the running configuration?

A.  password-encrypt

B.  enable password-encryption

C.  enable secret

D.  service password-encryption

Answer: B

──────────────────────────────────────────────────

## Question 43  (Topic 1)  —  Multiple Choice  [V80.2]

A network administrator must enable DHCP services between two sites. What must be configured for the

router to pass DHCPDISCOVER messages on to the server?

A.  a DHCP Relay Agent

B.  DHCP Binding

C.  a DHCP Pool

D.  DHCP Snooping

Answer: A

──────────────────────────────────────────────────

## Question 50  (Topic 1)  —  Multiple Choice  [V80.2]

What does physical access control regulate?

A.  access to spec fie networks based on business function

B.  access to servers to prevent malicious activity

C.  access to computer networks and file systems

D.  access to networking equipment and facilities

Answer: D

──────────────────────────────────────────────────

## Question 53  (Topic 1)  —  Multiple Choice  [V80.2]

Which command prevents passwords from being stored in the configuration as plain text on a router or

switch?

A.  enable secret

B.  service password-encryption

C.  username Cisco password encrypt

D.  enable password

Answer: B

──────────────────────────────────────────────────

## Question 74  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the AAA functions from the left onto the correct AAA services on the right

──────────────────────────────────────────────────

## Question 81  (Topic 1)  —  Multiple Choice  [V80.2]

What are network endpoints?

A.  act as routers to connect a user to the service prowler network

B.  a threat to the network if they are compromised

C.  support inter-VLAN connectivity

D.  enforce policies for campus-wide traffic going to the internet

Answer: B

──────────────────────────────────────────────────

## Question 93  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the threat-mitigation techniques from the left onto the types of threat or attack they mitigate

on the right.

──────────────────────────────────────────────────

## Question 97  (Topic 1)  —  Multiple Choice  [V80.2]

After installing a new Cisco ISE server, which task must the engineer perform on the Cisco WLC to connect

wireless clients on a specific VLAN based on their credentials?

A.  Enable the allow AAA Override

B.  Enable the Even: Driven RRM.

C.  Disable the LAG Mode or Next Reboot.

D.  Enable the Authorized MIC APs against auth-list or AAA.

Answer: A

──────────────────────────────────────────────────

## Question 99  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the attack-mitigation techniques from the left onto the Types of attack that they mitigate on

the right.

──────────────────────────────────────────────────

## Question 107  (Topic 1)  —  Multiple Choice  [V80.2]

What is an appropriate use for private IPv4 addressing?

A.  on the public-facing interface of a firewall

B.  to allow hosts inside to communicate in both directions with hosts outside the organization

C.  on internal hosts that stream data solely to external resources

D.  on hosts that communicates only with other internal hosts

Answer: D

──────────────────────────────────────────────────

## Question 145  (Topic 1)  —  Multiple Choice  [V80.2]

Which device controls the forwarding of authentication requests for users when connecting to the network

using a lightweight access point?

A.  TACACS server

B.  wireless access point

C.  RADIUS server

D.  wireless LAN controller

Answer: B

──────────────────────────────────────────────────

## Question 161  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

When PC-A sends traffic to PC-B, which network component is in charge of receiving the packet from PC-A

verifying the IP addresses, and forwarding the packet to PC-B?

A.  Layer 2 switch

B.  Router

C.  Load balancer

D.  firewall

Answer: B

──────────────────────────────────────────────────

## Question 182  (Topic 1)  —  Multiple Choice  [V80.2]

Which type of attack can be mitigated by dynamic ARP inspection?

A.  worm

B.  malware

C.  DDoS

D.  man-in-the-middle

Answer: D

──────────────────────────────────────────────────

## Question 188  (Topic 1)  —  Multiple Choice  [V80.2]

Which device tracks the state of active connections in order to make a decision to forward a packet

through?

A.  wireless access point

B.  firewall

C.  wireless LAN controller

D.  router

Answer: B

──────────────────────────────────────────────────

## Question 191  (Topic 1)  —  Multiple Choice  [V80.2]

In which two ways does a password manager reduce the chance of a hacker stealing a users password?

(Choose two.)

A.  It automatically provides a second authentication factor that is unknown to the original user.

B.  It uses an internal firewall to protect the password repository from unauthorized access.

C.  It protects against keystroke logging on a compromised device or web site.

D.  It stores the password repository on the local workstation with built-in antivirus and anti- malware

functionality

E.  It encourages users to create stronger passwords.

Answer: C,E

──────────────────────────────────────────────────

## Question 196  (Topic 1)  —  Multiple Choice  [V80.2]

What causes a port to be placed in the err-disabled state?

A.  latency

B.  port security violation

C.  shutdown command issued on the port

D.  nothing plugged into the port

Answer: B

──────────────────────────────────────────────────

## Question 197  (Topic 1)  —  Multiple Choice  [V80.2]

What is a network appliance that checks the state of a packet to determine whether the packet is

legitimate?

A.  Layer 2 switch

B.  load balancer

C.  firewall

D.  LAN controller

Answer: C

──────────────────────────────────────────────────

## Question 203  (Topic 1)  —  Multiple Choice  [V80.2]

Which device performs stateful inspection of traffic?

A.  firewall

B.  switch

C.  access point

D.  wireless controller

Answer: A

──────────────────────────────────────────────────

## Question 207  (Topic 1)  —  Multiple Choice  [V80.2]

Which security program element involves installing badge readers on data-center doors to allow workers to

enter and exit based on their job roles?

A.  role-based access control

B.  biometrics

C.  multifactor authentication

D.  physical access control

Answer: D

──────────────────────────────────────────────────

## Question 210  (Topic 1)  —  Multiple Choice  [V80.2]

What is the role of a firewall in an enterprise network?

A.  Forwards packets based on stateless packet inspection

B.  Processes unauthorized packets and allows passage to less secure segments of the network

C.  determines which packets are allowed to cross from unsecured to secured networks

D.  explicitly denies all packets from entering an administrative domain

Answer: C

──────────────────────────────────────────────────

## Question 228  (Topic 1)  —  Multiple Choice  [V80.2]

Which two protocols are supported on service-port interfaces? (Choose two.)

A.  RADIUS

B.  TACACS+

C.  SCP

D.  Telnet

E.  SSH

Answer: D,E

──────────────────────────────────────────────────

## Question 254  (Topic 2)  —  Multiple Choice  [V80.2]

Which protocol prompts the Wireless LAN Controller to generate its own local web administration SSL

certificate for GUI access?

A.  HTTPS

B.  RADIUS

C.  TACACS+

D.  HTTP

Answer: A

──────────────────────────────────────────────────

## Question 263  (Topic 2)  —  Drag & Drop  [V80.2]

Drag and drop the AAA terms from the left onto the description on the right.

──────────────────────────────────────────────────

## Question 272  (Topic 2)  —  Multiple Choice  [V80.2]

A network administrator must to configure SSH for remote access to router R1 The requirement is to use a

public and private key pair to encrypt management traffic to and from the connecting client.

Which configuration, when applied, meets the requirements?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 273  (Topic 2)  —  Multiple Choice  [V80.2]

What is a role of access points in an enterprise network?

A.  connect wireless devices to a wired network

B.  support secure user logins to devices or the network

C.  integrate with SNMP in preventing DDoS attacks

D.  serve as a first line of defense in an enterprise network

Answer: A

──────────────────────────────────────────────────

## Question 289  (Topic 2)  —  Multiple Choice  [V80.2]

Which function is performed by DHCP snooping?

A.  propagates VLAN information between switches

B.  listens to multicast traffic for packet forwarding

C.  provides DDoS mitigation

D.  rate-limits certain traffic

Answer: A

──────────────────────────────────────────────────

## Question 299  (Topic 2)  —  Multiple Choice  [V80.2]

A wireless administrator has configured a WLAN; however, the clients need access to a less congested

5-GHz network for their voice quality. What action must be taken to meet the requirement?

A.  enable AAA override

B.  enable RX-SOP

C.  enable DTIM

D.  enable Band Select

Answer: D

──────────────────────────────────────────────────

## Question 311  (Topic 2)  —  Multiple Choice  [V80.2]

Which technology can prevent client devices from arbitrarily connecting to the network without state

remediation?

A.  802.1x

B.  IP Source Guard

C.  MAC Authentication Bypass

D.  802.11n

Answer: A

──────────────────────────────────────────────────

## Question 318  (Topic 2)  —  Multiple Choice  [V80.2]

What is a difference between RADIUS and TACACS+?

A.  RADIUS is most appropriate for dial authentication, but TACACS+ can be used for multiple types of

authentication

B.  TACACS+ encrypts only password information and RADIUS encrypts the entire payload

C.  TACACS+ separates authentication and authorization, and RADIUS merges them

D.  RADIUS logs all commands that are entered by the administrator, but TACACS+ logs only start, stop,

and interim commands

Answer: C

──────────────────────────────────────────────────

## Question 327  (Topic 2)  —  Multiple Choice  [V80.2]

What benefit does controller-based networking provide versus traditional networking?

A.  moves from a two-tier to a three-tier network architecture to provide maximum redundancy

B.  provides an added layer of security to protect from DDoS attacks

C.  allows configuration and monitoring of the network from one centralized port

D.  combines control and data plane functionality on a single device to minimize latency

Answer: C

──────────────────────────────────────────────────

## Question 336  (Topic 2)  —  Multiple Choice  [V80.2]

Which function does an SNMP agent perform?

A.  it sends information about MIB variables in response to requests from the NMS

B.  it requests information from remote network nodes about catastrophic system events.

C.  it manages routing between Layer 3 devices in a network

D.  it coordinates user authentication between a network device and a TACACS+ or RADIUS server

Answer: A

──────────────────────────────────────────────────

## Question 364  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to Exhibit.

Which action do the switches take on the trunk link?

A.  The trunk does not form and the ports go into an err-disabled status.

B.  The trunk forms but the mismatched native VLANs are merged into a single broadcast domain.

C.  The trunk does not form, but VLAN 99 and VLAN 999 are allowed to traverse the link.

D.  The trunk forms but VLAN 99 and VLAN 999 are in a shutdown state.

Answer: B

──────────────────────────────────────────────────

## Question 410  (Topic 2)  —  Multiple Choice  [V80.2]

An administrator must secure the WLC from receiving spoofed association requests. Which steps must be

taken to configure the WLC to restrict the requests and force the user to wait 10 ms to retry an association

request?

A.  Enable Security Association Teardown Protection and set the SA Query timeout to 10

B.  Enable MAC filtering and set the SA Query timeout to 10

C.  Enable 802.1x Layer 2 security and set me Comeback timer to 10

D.  Enable the Protected Management Frame service and set the Comeback timer to 10

Answer: C

──────────────────────────────────────────────────

## Question 411  (Topic 2)  —  Multiple Choice  [V80.2]

How do AAA operations compare regarding user identification, user services and access control?

A.  Authorization provides access control and authentication tracks user services

B.  Authentication identifies users and accounting tracks user services

C.  Accounting tracks user services, and authentication provides access control

D.  Authorization identifies users and authentication provides access control

Answer: B

──────────────────────────────────────────────────

## Question 422  (Topic 2)  —  Multiple Choice  [V80.2]

Where does a switch maintain DHCP snooping information?

A.  in the MAC address table

B.  in the CAM table

C.  in the binding database

D.  in the frame forwarding database

Answer: C

──────────────────────────────────────────────────

## Question 423  (Topic 2)  —  Multiple Choice  [V80.2]

What is the primary different between AAA authentication and authorization?

A.  Authentication verifies a username and password, and authorization handles the communication

between the authentication agent and the user database.

B.  Authentication identifies a user who is attempting to access a system, and authorization validates the

users password

C.  Authentication identifies and verifies a user who is attempting to access a system, and authorization

controls the tasks the user can perform.

D.  Authentication controls the system processes a user can access and authorization logs the activities the

user initiates

Answer: C

──────────────────────────────────────────────────

## Question 425  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit. After the configuration is applied, the two routers fail to establish an OSPF neighbor

relationship. what is the reason for the problem?

A.  The OSPF router IDs are mismatched.

B.  Router2 is using the default hello timer.

C.  The network statement on Router1 is misconfigured.

D.  The OSPF process IDs are mismatched.

Answer: B

──────────────────────────────────────────────────

## Question 429  (Topic 2)  —  Multiple Choice  [V80.2]

Which action is taken by a switch port enabled for PoE power classification override?

A.  When a powered device begins drawing power from a PoE switch port a syslog message is generated

B.  As power usage on a PoE switch port is checked data flow to the connected device is temporarily

paused

C.  If a switch determines that a device is using less than the minimum configured power it assumes the

device has failed and disconnects

D.  Should a monitored port exceeds the maximum administrative value for power, the port is shutdown and

err-disabled

Answer: D

──────────────────────────────────────────────────

## Question 432  (Topic 2)  —  Multiple Choice  [V80.2]

A network administrator enabled port security on a switch interface connected to a printer. What is the next

configuration action in order to allow the port to learn the MAC address of the printer and insert it into the

table automatically?

A.  enable dynamic MAC address learning

B.  implement static MAC addressing.

C.  enable sticky MAC addressing

D.  implement auto MAC address learning

Answer: C

──────────────────────────────────────────────────

## Question 441  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Users need to connect to the wireless network with IEEE 802. 11r-compatible devices. The connection

must be maintained as users travel between floors or to other areas in the building What must be the

configuration of the connection?

A.  Select the WPA Policy option with the CCKM option.

B.  Disable AES encryption.

C.  Enable Fast Transition and select the FT 802.1x option.

D.  Enable Fast Transition and select the FT PSK option.

Answer: D

──────────────────────────────────────────────────

## Question 448  (Topic 3)  —  Drag & Drop  [V80.2]

An engineer is tasked to configure a switch with port security to ensure devices that forward unicasts

multicasts and broadcasts are unable to flood the port The port must be configured to permit only two

random MAC addresses at a time Drag and drop the required configuration commands from the left onto

the sequence on the right Not all commands are used.

──────────────────────────────────────────────────

## Question 460  (Topic 3)  —  Multiple Choice  [V80.2]

What is one reason to implement LAG on a Cisco WLC?

A.  to increase security and encrypt management frames

B.  to provide link redundancy and load balancing

C.  to allow for stateful and link-state failover

D.  to enable connected switch ports to failover and use different VLANs

Answer: B

──────────────────────────────────────────────────

## Question 465  (Topic 3)  —  Multiple Choice  [V80.2]

Which WLC management connection type is vulnerable to man-in-the-middle attacks?

A.  SSH

B.  HTTPS

C.  Telnet

D.  console

Answer: C

──────────────────────────────────────────────────

## Question 468  (Topic 3)  —  Drag & Drop  [V80.2]

Drag and drop the descriptions of AAA services from the left onto the corresponding services on the right.

──────────────────────────────────────────────────

## Question 469  (Topic 3)  —  Multiple Choice  [V80.2]

What is a function of a Next-Generation IPS?

A.  makes forwarding decisions based on learned MAC addresses

B.  serves as a controller within a controller-based network

C.  integrates with a RADIUS server to enforce Layer 2 device authentication rules

D.  correlates user activity with network events

Answer: D

──────────────────────────────────────────────────

## Question 470  (Topic 3)  —  Multiple Choice  [V80.2]

Which field within the access-request packet is encrypted by RADIUS?

A.  authorized services

B.  authenticator

C.  username

D.  password

Answer: D

──────────────────────────────────────────────────

## Question 478  (Topic 3)  —  Multiple Choice  [V80.2]

What causes a port to be placed in the err-disabled state?

A.  nothing plugged into the port

B.  link flapping

C.  shutdown command issued on the port

D.  latency

Answer: B

──────────────────────────────────────────────────

## Question 494  (Topic 3)  —  Multiple Choice  [V80.2]

Which action implements physical access control as part of the security program of an organization?

A.  configuring a password for the console port

B.  backing up syslogs at a remote location

C.  configuring enable passwords on network devices

D.  setting up IP cameras to monitor key infrastructure

Answer: A

──────────────────────────────────────────────────

## Question 509  (Topic 3)  —  Multiple Choice  [V80.2]

What provides centralized control of authentication and roaming In an enterprise network?

A.  a lightweight access point

B.  a firewall

C.  a wireless LAN controller

D.  a LAN switch

Answer: C

──────────────────────────────────────────────────

## Question 512  (Topic 3)  —  Multiple Choice  [V80.2]

Which type of network attack overwhelms the target server by sending multiple packets to a port until the

half-open TCP resources of the target are exhausted?

A.  SYIM flood

B.  reflection

C.  teardrop

D.  amplification

Answer: A

──────────────────────────────────────────────────

## Question 516  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The DHCP server and clients are connected to the same switch. What is the next step to complete the

DHCP configuration to allow clients on VLAN 1 to receive addresses from the

DHCP server?

A.  Configure the ip dhcp snooping trust command on the interlace that is connected to the DHCP client.

B.  Configure the ip dhcp relay information option command on the interface that is connected to the DHCP

client.

C.  Configure the ip dhcp snooping trust command on the interface that is connected to the DHCP server.

D.  Configure the Ip dhcp relay information option command on the interface that is connected to the DHCP

server.

Answer: C

──────────────────────────────────────────────────

## Question 531  (Topic 3)  —  Drag & Drop  [V80.2]

Drag and drop the threat-mitigation techniques from the left onto the types of threat or attack they mitigate

on the right.

──────────────────────────────────────────────────

## Question 545  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The network engineer is configuring a new WLAN and is told to use a static password for authentication

instead of the RADIUS servers. Which additional set of tasks must the engineer perform to complete the

configuration?

A.  Crable PSKEnable iod 1s

B.  Select WPA2 PolicyDisable PMFEnable PSK

C.  Select WPA PolicyEnable PSK

D.  Select WPA PolicySelect WRAS Policy

Answer: B

──────────────────────────────────────────────────

## Question 550  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A network engineer is configuring a WLAN to use a WPA2 PSK and allow only specific

clients to join. Which two actions must be taken to complete the process? (Choose two.)

A.  Enable the 802.1X option for Authentication Key Management

B.  Enable the WPA2 Policy option

C.  Enable the CCKM option for Authentication Key Management

D.  Enable the MAC Filtering option

E.  Enable the OSEN Policy option

Answer: B,D

──────────────────────────────────────────────────

## Question 564  (Topic 4)  —  Multiple Choice  [V80.2]

Which enhancement is implemented in WPA3?

A.  applies 802.1x authentication

B.  usesTKIP

C.  employs PKI to identify access points

D.  protects against brute force attacks

Answer: D

──────────────────────────────────────────────────

## Question 567  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to Itie exhibit

A network engineer started to configure port security on a new switch. These requirements must be met:

* MAC addresses must be learned dynamically

* Log messages must be generated without disabling the interface when unwanted traffic is seen

Which two commands must be configured to complete this task"? (Choose two)

A.  SW(ccnfig-if)=switchport port-security mac-address sticky

B.  SW(confKj-if)=switchport port-security violation restrict

C.  SW(config.if)sswitchport port-security mac-address 0010.7B84.45E6

D.  SW(config-if)aswitchport port-security maximum 2

E.  SW(ccnfig-if)=switchport port-security violation shutdown

Answer: C

──────────────────────────────────────────────────

## Question 573  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Clients on the WLAN are required to use 802.11r. What action must be taken to meet

the requirement?

A.  Set the Fast Transition option to Enable and enable FT 802.1X under Authentication Key Management

B.  Set the Fast Transition option and the WPA gik-randomize State to disable

C.  Under Protected Management Frames, set the PMF option to Required

D.  Enable CCKM under Authentication Key Management

Answer: A

──────────────────────────────────────────────────

## Question 577  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Company A wants to use a RADIUS server to service all user and device

authentication attempts with a more secure and granular authentication approach. Not all client devices

support dot1x authentication. Which two configuration changes must be made to accomplish the task?

(Choose two.)

A.  Enable AutoConfig IPSK under the Layer 2 tab.

B.  Select Authentication server under the AAA servers tab.

C.  Configure Enterprise Security type under the Layer 2 tab.

D.  Set Authentication under the Layer 3 tab.

E.  Enable WPA2 Policy under the Layer 2 tab.

Answer: B,C

──────────────────────────────────────────────────

## Question 582  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the Cisco IOS attack mitigation features from the left onto the types of network attack they

mitigate on the right.

──────────────────────────────────────────────────

## Question 584  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the AAA lerms from the left onto the descriptions on the right.

──────────────────────────────────────────────────

## Question 601  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the statements about AAA services firm the left onto the corresponding AAA services on the

right Not all options are used.

──────────────────────────────────────────────────

## Question 611  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A network administrator is configuring a router for user access via SSH. The

service-password encryption command has been issued. The configuration must meet these requirements:

• Create the username as CCUser.

• Create the password as NA!2Scc.

• Encrypt the user password.

What must be configured to meet the requirements?

A.  username CCUser privies 10 password NA!2Scc

B.  username CCUser password NA!2Scc enable password level 5 NA!2$cc

C.  username CCUser secret NA!2Scc

D.  username CCUser privilege 15 password NA!2Scc enable secret 0 NA!2$cc

Answer: B

──────────────────────────────────────────────────

## Question 625  (Topic 4)  —  Multiple Choice  [V80.2]

Which selections must be used on the WLC when implementing a RADIUS server for wireless

authentication?

A.  Client Exclusion and SSH

B.  802.1x and the MAC address of the server

C.  Network Access Control State and SSH

D.  AAA Override and the IP address of the server

Answer: B

──────────────────────────────────────────────────

## Question 633  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer is configuring a switch port that is connected to a VoIP handset. Which command must the

engineer configure to enable port security with a manually assigned MAC address of abod-bod on voice

VLAN 4?

A.  switchport port-security mac-address abcd.abcd.abcd

B.  switchport port-security mac-address abed.abed.abed vlan 4

C.  switchport port-security mac-address sticky abcd.abcd.abcd vlan 4

D.  switchport port-security mac-address abcd.abcd.abcd vlan voice

Answer: A

──────────────────────────────────────────────────

## Question 654  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the AAA features from the left onto the corresponding AAA security services on the right. Not

all options are used.

──────────────────────────────────────────────────

## Question 670  (Topic 4)  —  Multiple Choice  [V80.2]

A network engineer starts to implement a new wireless LAN by configuring the authentication server and

creating the dynamic Interface. What must be performed next to complete the basic configuration?

A.  Install the management interface and add the management IP.

B.  Configure high availability and redundancy tor the access points.

C.  Enable Telnet and RADIUS access on the management interface.

D.  Create the new WLAN and bind the dynamic interface to It.

Answer: D

──────────────────────────────────────────────────

## Question 672  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the statement about AAA services from the left to the corresponding AAA services on the

right.

──────────────────────────────────────────────────

## Question 700  (Topic 4)  —  Multiple Choice  [V80.2]

What does WPA3 provide in wireless networking?

A.  safeguards against brute force attacks with SAE

B.  optional Protected Management Frame negotiation

C.  backward compatibility with WPAand WPA2

D.  increased security and requirement of a complex configuration

Answer: A

──────────────────────────────────────────────────

## Question 701  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the AAA features from the left onto the corresponding AAA security services on the right. Not

all options are used.

──────────────────────────────────────────────────

## Question 730  (Topic 4)  —  Multiple Choice  [V80.2]

How does machine learning improve the detection of unauthorized network access?

A.  It monitors for outdated software.

B.  It dictates security policy updates.

C.  It identifies patterns indicating intrusions.

D.  It assigns security clearance levels.

Answer: A

──────────────────────────────────────────────────

## Question 741  (Topic 4)  —  Multiple Choice  [V80.2]

A network engineer is configuring a new router at a branch office. The router is connected to an upstream

WAN network that allows the branch to communicate with the head office. The central time server with IP

address 172.24.54.8 is located behind a firewall at the head office. Which command must the engineer

configure so that the software clock of the new router synchronizes with the time server?

A.  ntp master 172.24.54.8

B.  ntp client 172.24.54.8

C.  ntp peer 172.24.54.8

D.  ntp server 172.24.54.8

Answer: D

──────────────────────────────────────────────────

## Question 774  (Topic 4)  —  Drag & Drop  [V80.2]

Refer to the exhibit.

Drag and drop the destination IPs from the left onto the paths to reach those destinations on the right.

──────────────────────────────────────────────────

## Question 782  (Topic 4)  —  Multiple Choice  [V80.2]

Which device separates networks by security domains?

A.  firewall

B.  access point

C.  intrusion protection system

D.  wireless controller

Answer: A

──────────────────────────────────────────────────

## Question 786  (Topic 4)  —  Multiple Choice  [V80.2]

What is used as a solution for protecting an individual network endpoint from attack?

A.  Router

B.  Wireless controller

C.  Anti software

D.  Cisco DNA Center

Answer: C

──────────────────────────────────────────────────

## Question 797  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What is the issue with the interface GigabitEthernet0/0/1?

A.  Port security

B.  High throughput

C.  Cable disconnect

D.  duplex mismatch

Answer: C

──────────────────────────────────────────────────

## Question 808  (Topic 4)  —  Multiple Choice  [V80.2]

What is a feature of WPA?

A.  802.1x authentication

B.  preshared key

C.  TKIP/MIC encryption

D.  small Wi-Fi application

Answer: A

──────────────────────────────────────────────────

## Question 847  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A network engineer started to change default settings on SW1 to allow remote access

and has entered the following in the configuration mode:

SW1(config)#line vty 0 15

SW1(config-line)#password Labtest32!

Which set of commands are needed to allow only SSH access and hide passwords in the running

configuration?

A.  SW1(config-line)#exitSW1(config)#aaa new-model

B.  SW1(config-line)#login localSW1(config-line)#exitSW1(config)#enable secret test!2E

C.  SW1(config-line)#transport input sshSW1(config-line)#exitSW1(config)#service password-encryption

D.  SW1(config-line)#login localSW1(config-line)#exitSW1(config)#crypto key generate rsa

Answer: C

──────────────────────────────────────────────────

## Question 867  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the elements of a security program from the left onto the corresponding descriptions on the

right.

──────────────────────────────────────────────────

## Question 873  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What are the two steps an engineer must take to provide the highest encryption and authentication using

domain credentials from LDAP?

A.  Select PSK under Authentication Key Management

B.  Select WPA+WPA2 on Layer 2 Security

C.  Select Static-WEP + 802.1X on Layer 2 Security

D.  Select WPA Policy with TKIP Encryption

E.  Select 802.1X from under Authentication Key Management

Answer: B,E

──────────────────────────────────────────────────

## Question 875  (Topic 4)  —  Multiple Choice  [V80.2]

What is the default port-security behavior on a trunk link?

A.  It causes a network loop when a violation occurs.

B.  It disables the native VLAN configuration as soon as port security is enabled.

C.  It places the port in the err-disabled state if it learns more than one MAC address.

D.  It places the port in the err-disabled slate after 10 MAC addresses are statically configured.

Answer: A

──────────────────────────────────────────────────

## Question 880  (Topic 4)  —  Multiple Choice  [V80.2]

Which protocol should be used to transfer large files on a company intranet that allows TCP ports 20 and 21

through the firewall?

A.  REST API

B.  FTP

C.  SMTP

D.  TFTP

Answer: B

──────────────────────────────────────────────────

## Question 885  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Clients on the WLAN are required to use 802.11r. What action must be taken to meet the requirement?

A.  Under Protected Management Frames, set the PMF option to Required.

B.  Enable CCKM under Authentication Key Management.

C.  Set the Fast Transition option and the WPA gtk-randomize State to disable.

D.  Set the Fast Transition option to Enable and enable FT 802.1X under Authentication Key Management.

Answer: D

──────────────────────────────────────────────────

## Question 898  (Topic 4)  —  Multiple Choice  [V80.2]

After a recent security breach and a RADIUS failure, an engineer must secure the console port of each

enterprise router with a local username and password. Which configuration must the engineer apply to

accomplish this task?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 913  (Topic 4)  —  Multiple Choice  [V80.2]

It work security team noticed that an increasing number of employees are becoming victims of phishing

attacks. Which security program should be implemented to mitigate the problem?

A.  email system patches

B.  physical access control

C.  software firewall enabled on all PCs

D.  user awareness training

Answer: D

──────────────────────────────────────────────────

## Question 933  (Topic 4)  —  Multiple Choice  [V80.2]

What is the purpose of the Cisco DNA Center controller?

A.  to secure physical access to a data center

B.  to scan a network and generate a Layer 2 network diagram

C.  to securely manage and deploy network devices

D.  to provide Layer 3 services to autonomous access points

Answer: C

──────────────────────────────────────────────────

## Question 943  (Topic 4)  —  Multiple Choice  [V80.2]

To improve corporate security, an organization is planning to implement badge authentication to limit

access to the data center. Which element of a security program is being deployed?

A.  user training

B.  user awareness

C.  vulnerability verification

D.  physical access control

Answer: D

──────────────────────────────────────────────────

## Question 949  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Which functionalities will this SSID have while being used by wireless clients?

A.  decreases network security against offline dictionary attacks and encourages easy access to the

network

B.  increases network security against offline dictionary attacks and discourages time- consuming brute

force attacks

C.  increases network security against man in the middle attacks and discourages denial of service attacks

D.  decreases network security against air sniffing attacks and discourages the use of complex passwords

Answer: D

──────────────────────────────────────────────────

## Question 954  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer must provide configured IP addressing details to investigate a firewall rule Issue. Which

subnet and mask Identify what is configured on the en0 interface?

A.  10.8.0.0/16

B.  10.8.64.0/18

C.  10.8.128.0/19

D.  10.8.138.0/24

Answer: D

──────────────────────────────────────────────────

## Question 955  (Topic 4)  —  Multiple Choice  [V80.2]

Which two protocols are used by an administrator for authentication and configuration on access points?

A.  Kerberos

B.  802.1Q

C.  802.1x

D.  TACACS+

E.  RADIUS

Answer: D,E

──────────────────────────────────────────────────

## Question 956  (Topic 4)  —  Multiple Choice  [V80.2]

Which action implements physical access control as part of the security program of an organization1?

A.  backing up syslogs at a remote location

B.  configuring a password for the console port

C.  configuring enable passwords on network devices

D.  setting up IP cameras to monitor key infrastructure

Answer: B

──────────────────────────────────────────────────

## Question 958  (Topic 4)  —  Multiple Choice  [V80.2]

Which device segregates a network into separate zones that have their own security

policies?

A.  IPS

B.  firewall

C.  access point

D.  switch

Answer: C

──────────────────────────────────────────────────

## Question 960  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the statements about AAA from the left onto the corresponding AAA services on the right.

Not all options are used.

──────────────────────────────────────────────────

## Question 961  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the functions of AAA supporting protocols from the left onto the protocols on the right.

──────────────────────────────────────────────────

## Question 968  (Topic 4)  —  Multiple Choice  [V80.2]

Which security method is used to prevent man-in-the-middle attack?

A.  authorization

B.  authentication

C.  anti-replay

D.  accounting

Answer: B

──────────────────────────────────────────────────

## Question 980  (Topic 4)  —  Multiple Choice  [V80.2]

What is an enhancement implemented in WPA3?

A.  employs PKI and RADIUS to identify access points

B.  applies 802.1x authentication and AES-128 encryption

C.  uses TKIP and per-packet keying

D.  defends against deauthentication and disassociation attacks

Answer: D

──────────────────────────────────────────────────

## Question 992  (Topic 4)  —  Multiple Choice  [V80.2]

Which protocol must be implemented to support separate authorization and authentication solutions for

wireless APs?

A.  RADIUS

B.  TACACS+

C.  802.1X

D.  Kerberos

Answer: A

──────────────────────────────────────────────────

## Question 1003  (Topic 4)  —  Multiple Choice  [V80.2]

What is the definition of backdoor malware?

A.  malicious code that is installed onto a computer to allow access by an unauthorized user

B.  malicious code with the main purpose of downloading other malicious code

C.  malicious program that is used to launch other malicious programs

D.  malicious code that infects a user machine and then uses that machine to send spam

Answer: A

──────────────────────────────────────────────────

## Question 1005  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer configures the Cisco WLC to authenticate local wireless clients against a RADIUS

server Which task must be performed to complete the process?

A.  Change the Server Status to Disabled

B.  Select Enable next to Management

C.  Select Enable next to Network User

D.  Change the Support for CoA to Enabled.

Answer: C

──────────────────────────────────────────────────

## Question 1021  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer configures interface fa0/1 on SW1 and SW2 to pass traffic from two different VLANs. For

security reasons, company policy requires the native VLAN to be set to a nondefault value. Which

configuration meets this requirement?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 1027  (Topic 4)  —  Multiple Choice  [V80.2]

Which solution is appropriate when mitigating password attacks where the attacker was able to sniff the

clear-text password of the system administrator?

A.  next-generation firewall to keep stateful packet inspection

B.  multifactor authentication using two separate authentication sources

C.  ACL to restrict incoming Telnet sessions "admin" accounts

D.  IPS with a block list of known attack vectors

Answer: A

──────────────────────────────────────────────────

## Question 1035  (Topic 4)  —  Multiple Choice  [V80.2]

What is a function of Cisco Advanced Malware Protection for a Next-Generation IPS?

A.  authorizing potentially compromised wireless traffic

B.  inspecting specific files and file types for malware

C.  authenticating end users

D.  URL filtering

Answer: B

──────────────────────────────────────────────────

## Question 1038  (Topic 4)  —  Multiple Choice  [V80.2]

Which enhancements were implemented as part of WPA3?

A.  802.1x authentication and AES-128 encryption

B.  TKIP encryption improving WEP and per-packet keying

C.  AES-64 m personal mode and AES-128 in enterprise mode

D.  forward secrecy and SAE in personal mode for secure initial key exchange

Answer: D

──────────────────────────────────────────────────

## Question 1046  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. What is the cause of the issue?

A.  STP

B.  port security

C.  wrong cable type

D.  shutdown command

Answer: C

──────────────────────────────────────────────────

## Question 1057  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The network engineer is configuring a new WLAN and is told to use a setup password for authentication

instead of the RADIUS servers. Which additional set of tasks must the engineer perform to complete the

configuration?

A.  Disable PMF Enable PSK Enable 802.1x

B.  Select WPA Policy Enable CCKM Enable PSK

C.  Select WPA Policy Select WPA2 Policy Enable FT PSK

D.  Select WPA2 Policy Disable PMF Enable PSK

Answer: D

──────────────────────────────────────────────────

## Question 1068  (Topic 5)  —  Multiple Choice  [V80.2]

Why are API keys used to enforce rate limiting?

A.  to uniquely identify clients to monitor their usage patterns

B.  to encrypt data to prevent excessive usage

C.  to contain embedded permissions that automatically expire

D.  to track the geographical location of each request

Answer: A

──────────────────────────────────────────────────

## Question 1079  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A network engineer is configuring a wireless LAN with Web Passthrough Layer 3 Web

Policy. Which action must the engineer take to complete the configuration?

A.  Set the Layer 2 Security to 802.1X.

B.  Enable TKIP and CCMP256 WPA2 Encryption.

C.  Set the Layer 2 Security to None.

D.  Enable the WPA Policy.

Answer: C

──────────────────────────────────────────────────

## Question 1104  (Topic 5)  —  Multiple Choice  [V80.2]

How are API keys used to enforce rate limiting?

A.  To define the network path the API request should take

B.  To uniquely identify each client application

C.  To specify the type of data format the client prefers to receive

D.  To encrypt data sent in the API request

Answer: B

──────────────────────────────────────────────────

## Question 1123  (Topic 5)  —  Multiple Choice  [V80.2]

What is a characteristic of an SSID in wireless networks?

A.  provides protection against spyware

B.  eliminates network piggybacking

C.  associates a name to a wireless network

D.  allows easy file sharing between endpoints

Answer: C

──────────────────────────────────────────────────

## Question 1133  (Topic 5)  —  Multiple Choice  [V80.2]

A network security team noticed that an increasing number of employees are becoming victims of phishing

attacks. Which security program should be implemented to mitigate the problem?

A.  Physical access control

B.  Software firewall enabled on all PCs

C.  Email system patches

D.  User awareness training

Answer: D

──────────────────────────────────────────────────

## Question 1147  (Topic 5)  —  Multiple Choice  [V80.2]

Which command must be configured to enable port security with a manually assigned MAC address of

aabb.cc00.1234 for a VoIP handset on VLAN 4?

A.  switchport port-security mac-address aabb.cc00.1234

B.  switchport port-security mac-address aabb.cc00.1234 vlan 4

C.  mac-address-table static aabb.cc00.1234 vlan 4 interface fa0/1

D.  switchport port-security mac-address sticky

Answer: B

──────────────────────────────────────────────────

## Question 1148  (Topic 5)  —  Multiple Choice  [V80.2]

What is an advantage of using SDN versus traditional networking when it comes to security?

A.  SDN security is managed near the perimeter of the network with firewalls, VPNs, and IPS, and traditional

networking security policies are created based on telemetry data.

B.  SDN exposes an API to configure locally per device for security policies, and traditional networking uses

northbound API for network admin interface for configuring security policies.

C.  SDN creates a unified control point making security policies consistent across all

devices, and traditional networking must be configured device by device, leaving room for error.

D.  SDN devices communicate with each other to establish a security policy, and in traditional networking,

devices communicate upstream to a central location to establish a security policy.

Answer: C

──────────────────────────────────────────────────

## Question 1149  (Topic 5)  —  Multiple Choice  [V80.2]

What is a characteristic of private IPv4 addressing?

A.  Reduces the forwarding table on network routers

B.  Used on the external interface of a firewall

C.  Used by ISPs when only one IP is needed to connect to the internet

D.  Address space which is isolated from the internet

Answer: D

──────────────────────────────────────────────────

## Question 1157  (Topic 5)  —  Multiple Choice  [V80.2]

What is a function of a Next-Generation IPS?

A.  Integrates with a RADIUS server to enforce Layer 2 device authentication rules

B.  Serves as a controller within a controller-based network

C.  Makes forwarding decisions based on learned MAC addresses

D.  Correlates user activity with network events

Answer: D

──────────────────────────────────────────────────

## Question 1161  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Users will be using a preconfigured secret key and SSID and must have a secured key hashing algorithm

configured. The AAA server must not be used for the user authentication method. Which action completes

the task?

A.  Enable AutoConfig iPSK.

B.  SetCCMP128(AES).

C.  Configure PSK Format HEX with key string.

D.  Configure PSK-SHA2.

Answer: D

──────────────────────────────────────────────────

## Question 1169  (Topic 5)  —  Multiple Choice  [V80.2]

A network engineer starts to implement a new wireless LAN by configuring the authentication server and

creating the dynamic interface. What must be performed next to complete the basic configuration?

A.  Create the new WLAN and bind the dynamic interface to it.

B.  Install the management interface and add the management IP.

C.  Enable Telnet and RADIUS access on the management interface.

D.  Configure high availability and redundancy for the access points.

Answer: A

──────────────────────────────────────────────────

## Question 1171  (Topic 5)  —  Multiple Choice  [V80.2]

Which action implements physical access control as part of the security program of an organization?

A.  configuring a password for the console port

B.  configuring enable passwords on network devices

C.  backing up syslogs at a remote location

D.  setting up IP cameras to monitor key infrastructure

Answer: D

──────────────────────────────────────────────────

## Question 1184  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A network engineer is configuring a WLAN to use a WPA2 PSK and allow only specific

clients to join. Which two actions must be taken to complete the process? (Choose two)

A.  Enable the MAC Filtering option

B.  Enable the OSEN Policy option

C.  Enable the CCKM option for Authentication Key Management

D.  Enable the WPA2 Policy option

E.  Enable the 802.1X option for Authentication Key Management

Answer: A,D

──────────────────────────────────────────────────

## Question 1185  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A VTY password has been set to Labtest32! for remote access. Which commands are

required to allow only SSH access and to hide passwords in the running configuration?

A.  SW1#(config-line)#login localSW1#(config-line)#exitSW1(config)#crypto key generate rsa

B.  SW1#(config-line)#login localSW1#(config-line)#exitSW1(config)#enable secret test!2E

C.  SW1#(config-line)#transport input sshSW1#(config-line)#exitSW1(config)#service password-encryption

D.  SW1#(config-line)#exitSW1(config)#aaa new-model

Answer: C

──────────────────────────────────────────────────

## Question 1186  (Topic 5)  —  Multiple Choice  [V80.2]

How are API keys used to enforce rate limiting?

A.  to specify the type of data format the client prefers to receive

B.  to define the network path the API request should take

C.  to encrypt data sent in the API request

D.  to uniquely identify each client application

Answer: D

──────────────────────────────────────────────────

## Question 1199  (Topic 5)  —  Multiple Choice  [V80.2]

Which protocol should be used to transfer large files on a company intranet that allows UDP 69 through the

firewall?

A.  FTP

B.  TFTP

C.  REST API

D.  SMTP

Answer: A

──────────────────────────────────────────────────

## Question 1219  (Topic 5)  —  Multiple Choice  [V80.2]

What is an advantage of using SDN versus traditional networking when it comes to security?

A.  Security is managed near the perimeter of the network with firewalls, VPNs, and IPS.

B.  Devices communicate with each other to establish a security policy.

C.  It creates a unified control point making security policies consistent across all devices.

D.  It exposes an API to configure locally per device for security policies.

Answer: C

──────────────────────────────────────────────────

## Question 1221  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An administrator is configuring a new WLAN for a wireless network that has these

requirements:

Dual-band clients that connect to the WLAN must be directed to the 5-GHz spectrum. Wireless clients on

this WLAN must be able to apply VLAN settings from RADIUS attributes.

Which two actions meet these requirements? (Choose two.)

A.  Enable the Aironet IE option.

B.  Enable the Coverage Hole Detection option.

C.  Set the MFP Client Protection option to Required

D.  Enable the client band select option.

E.  Enable the allow AAA Override option

Answer: D,E

──────────────────────────────────────────────────

## Question 1234  (Topic 5)  —  Multiple Choice  [V80.2]

What is a characteristic of encryption in wireless networks?

A.  uses a unidirectional handshake for authentication

B.  Intercepts data threats before they attack a network

C.  uses integrity checks to identify forgery attacks

D.  prevents intercepted data from being easily read

Answer: D

──────────────────────────────────────────────────

## Question 1235  (Topic 5)  —  Multiple Choice  [V80.2]

An organization developed new security policies and decided to print the policies and distribute them to all

personnel so that employees review and apply the policies. Which element of a security program is the

organization implementing?

A.  Asset identification

B.  User training

C.  Physical access control

D.  Vulnerability control

Answer: B