# CCNA 200-301
OSPF

Open Shortest Path First routing protocol

81 Questions

Multiple Choice: 80 | Drag & Drop: 1

Sources: V7.1 (18) | V80.2 (63)

Topic 1: 13 | Topic 2: 30 | Topic 3: 13 | Topic 4: 18 | Topic 5: 7

## Question 7  (Topic 3)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

An engineer assumes a configuration task from a peer Router A must

establish an OSPF neighbor relationship with neighbor 172 1 1 1 The

output displays the status of the adjacency after 2 hours. What is the

next step in the configuration process for the routers to establish an

adjacency?

A.  Configure router A to use the same MTU size as router B.

B.  Set the router B OSPF ID to a nonhost address.

C.  Configure a point-to-point link between router A and router B.

D.  Set the router B OSPF ID to the same value as its IP address

Answer: B

──────────────────────────────────────────────────

## Question 20  (Topic 3)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Which route must be configured on R1 so that OSPF routing is used when

OSPF is up. but the server is still reachable when OSPF goes down?

A.  ip route 10.1.1.10 255.255.255.255 172.16.2.2 100

B.  ip route 10.1.1.0 255.255.255.0 gi0/1 125

C.  ip route 10.1.1.0 255.255.255.0 172.16.2.2 100

D.  ip route 10.1.1.10 255.255.255.255 gi0/0 125

Answer: D

──────────────────────────────────────────────────

## Question 46  (Topic 3)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Which configuration allows routers R14 and R86 to form an OSPFv2

adjacency while acting as a central point for exchanging OSPF

information between routers? A)

[See Exhibit Image]

B\)

[See Exhibit Image]

[See Exhibit Image]C)

D\)

[See Exhibit Image]

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 71  (Topic 2)  —  Multiple Choice  [V7.1]

Which networking function occurs on the data plane?

A.  forwarding remote client/server traffic

B.  facilitates spanning-tree elections

C.  processing inbound SSH management traffic

D.  sending and receiving OSPF Hello packets

Answer: A

──────────────────────────────────────────────────

## Question 135  (Topic 2)  —  Multiple Choice  [V7.1]

A user configured OSPF in a single area between two routers A serial

interface connecting R1 and R2 is running encapsulation PPP By default

which OSPF network type is seen on this interface when the user types

show ip ospf interface on R1 or R2?

A.  port-to-multipoint

B.  broadcast

C.  point-to-point

D.  nonbroadcast

Answer: C

Explanation:

The default OSPF network type for HDLC and PPP on Serial link is

point-to- point (while the default OSPF network type for Ethernet link

is Broadcast).

──────────────────────────────────────────────────

## Question 139  (Topic 2)  —  Multiple Choice  [V7.1]

An engineer must configure an OSPF neighbor relationship between

router R1 and R3 The authentication configuration has been configured

and the connecting interfaces are in the same 192.168 1.0/30 sublet.

What are the next two steps to complete the configuration? (Choose

two.)

A.  configure the hello and dead timers to match on both sides

B.  configure the same process ID for the router OSPF process

C.  configure the same router ID on both routing processes

D.  Configure the interfaces as OSPF active on both sides.

E.  configure both interfaces with the same area ID

Answer: AE

──────────────────────────────────────────────────

## Question 148  (Topic 2)  —  Multiple Choice  [V7.1]

When OSPF learns multiple paths to a network, how does it select a

route?

A.  It multiple the active K value by 256 to calculate the route with

the lowest metric.

B.  For each existing interface, it adds the metric from the source

router to the destination to calculate the route with the lowest

bandwidth.

C.  It divides a reference bandwidth of 100 Mbps by the actual bandwidth

of the existing interface to calculate the router with the lowest

cost.

D.  It count the number of hops between the source router and the

destination to determine the router with the lowest metric

Answer: C

──────────────────────────────────────────────────

## Question 192  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

The default-information originate command is configured under the R1

OSPF configuration After testing workstations on VLAN 20 at Site B

cannot reach a DNS server on the Internet Which action corrects the

configuration issue?

A.  Add the default-information originate command onR2

B.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.18 command on R1

C.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.2 command on R2

D.  Add the always keyword to the default-information originate command

on R1

Answer: B

──────────────────────────────────────────────────

## Question 197  (Topic 2)  —  Multiple Choice  [V7.1]

A user configured OSPF and advertised the Gigabit Ethernet interface

in OSPF By default, which type of OSPF network does this interface

belong to?

A.  point-to-multipoint

B.  point-to-point

C.  broadcast

D.  nonbroadcast

Answer: C

Explanation:

[https://www.oreilly.com/library/view/cisco-ios-](http://www.oreilly.com/library/view/cisco-ios-)

cookbook/0596527225/ch08s15.html

The Broadcast network type is the default for an OSPF enabled ethernet

interface (while Point-toPoint is the default OSPF network type for

Serial interface with HDLC and PPP encapsulation).

──────────────────────────────────────────────────

## Question 200  (Topic 2)  —  Multiple Choice  [V7.1]

What is the effect when loopback interfaces and the configured router

ID are absent during the OSPF Process configuration?

A.  No router ID is set, and the OSPF protocol does not run.

B.  The highest up/up physical interface IP address is selected as the

router ID.

C.  The lowest IP address is incremented by 1 and selected as the router

ID.

D.  The router ID 0.0.0.0 is selected and placed in the OSPF process.

Answer: B

──────────────────────────────────────────────────

## Question 204  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

The show ip ospf interface command has been executed on R1 How is OSPF

configured?

A.  The interface is not participating in OSPF

B.  A point-to-point network type is configured

C.  The default Hello and Dead timers are in use

D.  There are six OSPF neighbors on this interface

Answer: C

Explanation:

[https://www.cisco.com/c/en/us/support/docs/ip/open-shortest-path-first-](http://www.cisco.com/c/en/us/support/docs/ip/open-shortest-path-first-)

ospf/13689-17.html

──────────────────────────────────────────────────

## Question 215  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Router R2 is configured with multiple routes to reach network 10 1.1

0/24 from router R1. What protocol is chosen by router R2 to reach the

destination network

10.1 1 0/24?

A.  eBGP

B.  static

C.  OSPF

D.  EIGRP

Answer: B

──────────────────────────────────────────────────

## Question 242  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

Which configuration issue is preventing the OSPF neighbor relationship

from being established between the two routers?

A.  R2 is using the passive-interface default command

B.  R1 has an incorrect network command for interface Gi1/0

C.  R2 should have its network command in area 1

D.  R1 interface Gil/0 has a larger MTU size

Answer: D

──────────────────────────────────────────────────

## Question 257  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

What action establishes the OSPF neighbor relationship without forming

an adjacency?

A.  modify hello interval

B.  modify process ID

C.  modify priority

D.  modify network type

Answer: A

──────────────────────────────────────────────────

## Question 291  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

What does router R1 use as its OSPF router-ID?

A.  10.10.1.10

B.  10.10.10.20

C.  172.16.15.10

D.  192.168.0.1

Answer: C

Explanation:

OSPF uses the following criteria to select the router ID:1. Manual

configuration of the router ID (via the "router-id x.x.x.x" command

under OSPF router configuration mode).2. Highest IP address on a

loopback interface.3. Highest IP address on a non-loopback and active

(no shutdown) interface.

──────────────────────────────────────────────────

## Question 338  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Router R4 is dynamically learning the path to the server. If R4 is

connected to R1 via OSPF Area 20, to R2 v2ia R2 BGP, and to R3 via

EIGRP 777, which path is installed in the routing table of R4?

A.  the path through R1, because the OSPF administrative distance is 110

B.  the path through R2. because the IBGP administrative distance is 200

C.  the path through R2 because the EBGP administrative distance is 20

D.  the path through R3. because the EIGRP administrative distance is

lower than OSPF and BGP

Answer: C

──────────────────────────────────────────────────

## Question 370  (Topic 1)  —  Multiple Choice  [V7.1]

Which two minimum parameters must be configured on an active interface

to enable OSPFv2 to operate? (Choose two)

A.  OSPF area

B.  OSPF MD5 authentication key

C.  iPv6 address

D.  OSPf process ID

E.  OSPf stub flag

Answer: AD

──────────────────────────────────────────────────

## Question 382  (Topic 1)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

If OSPF is running on this network, how does Router 2 handle traffic

from Site B to 10.10.13/25 at Site A?

A.  It sends packets out of interface Fa0/2 only.

B.  It sends packets out of interface Fa0/1 only.

C.  It cannot send packets to 10.10.13 128/25

D.  It load-balances traffic out of Fa0/1 and Fa0/2

Answer: C

Explanation:

Router2 does not have an entry for the subnet 10.10.13.128/25. It only

has an entry for 10.10.13.0/25, which ranges from 10.10.13.0 to

10.10.13.127. https://study-ccna.com/administrative-distance-metric/

──────────────────────────────────────────────────

## Question 20  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R4 is dynamically learning the path to the server. If R4 is connected to R1 via OSPF Area 20, to R2

v2ia R2 BGP, and to R3 via EIGRP 777, which path is installed in the routing table of R4?

A.  the path through R1, because the OSPF administrative distance is 110

B.  the path through R2. because the IBGP administrative distance is 200

C.  the path through R2 because the EBGP administrative distance is 20

D.  the path through R3. because the EIGRP administrative distance is lower than OSPF and BGP

Answer: C

──────────────────────────────────────────────────

## Question 29  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

If OSPF Is running on this network, how does Router2 handle traffic from Site B to 10.10.13.128/25 at Site

A?

A.  It load-balances traffic out of Fa0/1 and Fa0/2.

B.  It is unreachable and discards the traffic.

C.  It sends packets out of interface FaO/2.

D.  It sends packets out of interface Fa0/1.

Answer: B

──────────────────────────────────────────────────

## Question 79  (Topic 1)  —  Multiple Choice  [V80.2]

Which network action occurs within the data plane?

A.  compare the destination IP address to the IP routing table.

B.  run routing protocols (OSPF, EIGRP, RIP, BGP)

C.  make a configuration change from an incoming NETCONF RPC

D.  reply to an incoming ICMP echo request

Answer: A

──────────────────────────────────────────────────

## Question 95  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

R5 is the current DR on the network, and R4 is the BDR. Their interfaces are flapping, so a network

engineer wants the OSPF network to elect a different DR and BDR. Which set of configurations must the

engineer implement?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 118  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A router reserved these five routes from different routing information sources. Which two routes does the

router install in its routing table? (Choose two)

A.  RIP route 10.0.0.0/30

B.  iBGP route 10.0.0.0/30

C.  OSPF route 10.0.0.0/30

D.  EIGRP route 10.0.0.1/32

E.  OSPF route 10.0.0.0/16

Answer: C,D

──────────────────────────────────────────────────

## Question 166  (Topic 1)  —  Drag & Drop  [V80.2]

A network engineer is configuring an OSPFv2 neighbor adjacency Drag and drop the parameters from the

left onto their required categories on the right. Not all parameters are used

──────────────────────────────────────────────────

## Question 217  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

How does the router manage traffic to 192.168.12.16?

A.  It selects the RIP route because it has the longest prefix inclusive of the destination address.

B.  It chooses the OSPF route because it has the longest prefix inclusive of the destination address.

C.  it load-balances traffic between all three routes

D.  It chooses the EIGRP route because it has the lowest administrative distance

Answer: A

──────────────────────────────────────────────────

## Question 222  (Topic 1)  —  Multiple Choice  [V80.2]

Which two minimum parameters must be configured on an active interface to enable OSPFv2 to operate?

(Choose two)

A.  OSPF area

B.  OSPF MD5 authentication key

C.  iPv6 address

D.  OSPf process ID

E.  OSPf stub flag

Answer: A,D

──────────────────────────────────────────────────

## Question 226  (Topic 1)  —  Multiple Choice  [V80.2]

What is the primary purpose of a First Hop Redundancy Protocol?

A.  It allows directly connected neighbors to share configuration information.

B.  It allows a router to use bridge priorities to create multiple loop-free paths to a single destination.

C.  It reduces routing failures by allowing Layer 3 load balancing between OSPF neighbors that have the

same link metric.

D.  It reduces routing failures by allowing more than one router to represent itself, as the

default gateway of a network.

Answer: D

──────────────────────────────────────────────────

## Question 237  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

If OSPF is running on this network, how does Router 2 handle traffic from Site B to 10.10.13/25 at Site A?

A.  It sends packets out of interface Fa0/2 only.

B.  It sends packets out of interface Fa0/1 only.

C.  It cannot send packets to 10.10.13 128/25

D.  It load-balances traffic out of Fa0/1 and Fa0/2

Answer: C Topic 2, Exam Pool B

──────────────────────────────────────────────────

## Question 240  (Topic 2)  —  Multiple Choice  [V80.2]

When OSPF learns multiple paths to a network, how does it select a route?

A.  It multiple the active K value by 256 to calculate the route with the lowest metric.

B.  For each existing interface, it adds the metric from the source router to the destination to calculate the

route with the lowest bandwidth.

C.  It divides a reference bandwidth of 100 Mbps by the actual bandwidth of the existing interface to

calculate the router with the lowest cost.

D.  It count the number of hops between the source router and the destination to determine the router with

the lowest metric

Answer: C

──────────────────────────────────────────────────

## Question 262  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What does router R1 use as its OSPF router-ID?

A.  10.10.1.10

B.  10.10.10.20

C.  172.16.15.10

D.  192.168.0.1

Answer: C

──────────────────────────────────────────────────

## Question 266  (Topic 2)  —  Multiple Choice  [V80.2]

An engineer must configure an OSPF neighbor relationship between router R1 and R3 The authentication

configuration has been configured and the connecting interfaces are in the same 192.168 1.0/30 sublet.

What are the next two steps to complete the configuration? (Choose two.)

A.  configure the hello and dead timers to match on both sides

B.  configure the same process ID for the router OSPF process

C.  configure the same router ID on both routing processes

D.  Configure the interfaces as OSPF active on both sides.

E.  configure both interfaces with the same area ID

Answer: A,E

──────────────────────────────────────────────────

## Question 268  (Topic 2)  —  Multiple Choice  [V80.2]

An engineer configured an OSPF neighbor as a designated router. Which state verifies the designated

router is in the proper mode?

A.  Exchange

B.  2-way

C.  Full

D.  Init

Answer: C

──────────────────────────────────────────────────

## Question 269  (Topic 2)  —  Multiple Choice  [V80.2]

Which networking function occurs on the data plane?

A.  forwarding remote client/server traffic

B.  facilitates spanning-tree elections

C.  processing inbound SSH management traffic

D.  sending and receiving OSPF Hello packets

Answer: A

──────────────────────────────────────────────────

## Question 284  (Topic 2)  —  Multiple Choice  [V80.2]

R1 has learned route 192.168.12.0/24 via IS-IS. OSPF, RIP. and Internal EIGRP Under normal operating

conditions, which routing protocol is installed in the routing table?

A.  IS-IS

B.  RIP

C.  Internal EIGRP

D.  OSPF

Answer: C

──────────────────────────────────────────────────

## Question 332  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

How does router R1 handle traffic to 192.168.10.16?

A.  It selects the IS-IS route because it has the shortest prefix inclusive of the destination address.

B.  It selects the EIGRP route because it has the lowest administrative distance.

C.  It selects the OSPF route because it has the lowest cost.

D.  It selects the RIP route because it has the longest prefix inclusive of the destination address.

Answer: D

──────────────────────────────────────────────────

## Question 337  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

If configuring a static default route on the router with the ip route 0.0.0.0 0.0.0.0 10.13.0.1 120 command

how does the router respond?

A.  It ignores the new static route until the existing OSPF default route is removed

B.  It immediately replaces the existing OSPF route in the routing table with the newly configured static route

C.  It starts load-balancing traffic between the two default routes

D.  It starts sending traffic without a specific matching entry in the routing table to GigabitEthernet0/1

Answer: A

──────────────────────────────────────────────────

## Question 354  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The default-information originate command is configured under the R1 OSPF configuration After testing

workstations on VLAN 20 at Site B cannot reach a DNS server on the Internet Which action corrects the

configuration issue?

A.  Add the default-information originate command onR2

B.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.18 command on R1

C.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.2 command on R2

D.  Add the always keyword to the default-information originate command on R1

Answer: B

──────────────────────────────────────────────────

## Question 362  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What action establishes the OSPF neighbor relationship without forming an adjacency?

A.  modify hello interval

B.  modify process ID

C.  modify priority

D.  modify network type

Answer: A

──────────────────────────────────────────────────

## Question 374  (Topic 2)  —  Multiple Choice  [V80.2]

What is the effect when loopback interfaces and the configured router ID are absent during the OSPF

Process configuration?

A.  No router ID is set, and the OSPF protocol does not run.

B.  The highest up/up physical interface IP address is selected as the router ID.

C.  The lowest IP address is incremented by 1 and selected as the router ID.

D.  The router ID 0.0.0.0 is selected and placed in the OSPF process.

Answer: B

──────────────────────────────────────────────────

## Question 387  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R2 is configured with multiple routes to reach network 10 1.1 0/24 from router R1. What protocol is

chosen by router R2 to reach the destination network 10.1 1 0/24?

A.  eBGP

B.  static

C.  OSPF

D.  EIGRP

Answer: B

──────────────────────────────────────────────────

## Question 391  (Topic 2)  —  Multiple Choice  [V80.2]

A user configured OSPF and advertised the Gigabit Ethernet interface in OSPF By default, which type of

OSPF network does this interface belong to?

A.  point-to-multipoint

B.  point-to-point

C.  broadcast

D.  nonbroadcast

Answer: C

──────────────────────────────────────────────────

## Question 399  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which configuration issue is preventing the OSPF neighbor relationship from being established between

the two routers?

A.  R2 is using the passive-interface default command

B.  R1 has an incorrect network command for interface Gi1/0

C.  R2 should have its network command in area 1

D.  R1 interface Gil/0 has a larger MTU size

Answer: D

──────────────────────────────────────────────────

## Question 409  (Topic 2)  —  Multiple Choice  [V80.2]

Router A learns the same route from two different neighbors, one of the neighbor routers is an OSPF

neighbor and the other is an EIGRP neighbor. What is the administrative distance of the route that will be

installed in the routing table?

A.  20

B.  90

C.  110

D.  115

Answer: B

──────────────────────────────────────────────────

## Question 418  (Topic 2)  —  Multiple Choice  [V80.2]

A user configured OSPF in a single area between two routers A serial interface connecting R1 and R2 is

running encapsulation PPP By default which OSPF network type is seen on this interface when the user

types show ip ospf interface on R1 or R2?

A.  port-to-multipoint

B.  broadcast

C.  point-to-point

D.  nonbroadcast

Answer: C

──────────────────────────────────────────────────

## Question 424  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The show ip ospf interface command has been executed on R1 How is OSPF configured?

A.  The interface is not participating in OSPF

B.  A point-to-point network type is configured

C.  The default Hello and Dead timers are in use

D.  There are six OSPF neighbors on this interface

Answer: C

──────────────────────────────────────────────────

## Question 425  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit. After the configuration is applied, the two routers fail to establish an OSPF neighbor

relationship. what is the reason for the problem?

A.  The OSPF router IDs are mismatched.

B.  Router2 is using the default hello timer.

C.  The network statement on Router1 is misconfigured.

D.  The OSPF process IDs are mismatched.

Answer: B

──────────────────────────────────────────────────

## Question 444  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which configuration allows routers R14 and R86 to form an OSPFv2 adjacency while acting as a central

point for exchanging OSPF information between routers?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 455  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which action must be taken to ensure that router A is elected as the DR for OSPF area 0?

A.  Configure the OSPF priority on router A with the lowest value between the three routers.

B.  Configure router B and router C as OSPF neighbors of router A.

C.  Configure the router A interfaces with the highest OSPF priority value within the area.

D.  Configure router A with a fixed OSPF router ID

Answer: C

──────────────────────────────────────────────────

## Question 460  (Topic 3)  —  Multiple Choice  [V80.2]

What is one reason to implement LAG on a Cisco WLC?

A.  to increase security and encrypt management frames

B.  to provide link redundancy and load balancing

C.  to allow for stateful and link-state failover

D.  to enable connected switch ports to failover and use different VLANs

Answer: B

──────────────────────────────────────────────────

## Question 463  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R1 resides in OSPF Area 0. After updating the R1 configuration to influence the paths that it will use

to direct traffic, an engineer verified that each of the four Gigabit interfaces has the same route to

10.10.0.0/16. Which interface will R1 choose to send traffic to reach the route?

A.  GigabitEthernet0/0

B.  GigabltEthornet0/1

C.  GigabitEthernet0/2

D.  GigabitEthernet0/3

Answer: B

──────────────────────────────────────────────────

## Question 489  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer assumes a configuration task from a peer Router A must establish an OSPF neighbor

relationship with neighbor 172 1 1 1 The output displays the status of the adjacency after 2 hours. What is

the next step in the configuration process for the routers to establish an adjacency?

A.  Configure router A to use the same MTU size as router B.

B.  Set the router B OSPF ID to a nonhost address.

C.  Configure a point-to-point link between router A and router B.

D.  Set the router B OSPF ID to the same value as its IP address

Answer: A

──────────────────────────────────────────────────

## Question 490  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Routers R1 and R3 have the default configuration The router R2 priority is set to 99 Which commands on

R3 configure it as the DR in the 10.0 4.0/24 network?

A.  R3(config)#interface Gig0/1 R3(config-if)#ip ospf priority 100

B.  R3(config)#interface Gig0/0 R3(config-if)#ip ospf priority 100

C.  R3(config)#interface Gig0/0 R3(config-if)i=ip ospf priority 1

D.  R3(config)#interface Gig0/1 R3(config-if)#ip ospf priority 0

Answer: B

──────────────────────────────────────────────────

## Question 520  (Topic 3)  —  Multiple Choice  [V80.2]

OSPF must be configured between routers R1 and R2. Which OSPF configuration must be applied to

router R1 to avoid a DR/BDR election?

A.  router ospf 1network 192.168.1.1 0.0.0.0 area 0interface e1/1ip address 192.168.1.1 255.255.255.252ip

ospf network broadcast

B.  router ospf 1network 192.168.1.1 0.0.0.0 area 0interface e1/1ip address 192.168.1.1 255.255.255.252ip

ospf network point-to-point

C.  router ospf 1network 192.168.1.1 0.0.0.0 area 0interface e1/1ip address 192.168.1.1 255.255.255.252ip

ospf cost 0

D.  router ospf 1network 192.168.1.1 0.0.0.0 area 0hello interval 15interface e1/1Ip address 192.168.1.1

255.255.255.252

Answer: B

──────────────────────────────────────────────────

## Question 521  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

R1 learns all routes via OSPF Which command configures a backup static route on R1 to reach the 192

168.20.0/24 network via R3?

A.  R1(config)#ip route 192.168.20.0 255.255.0.0 192.168.30.2

B.  R1(config)#ip route 192.168.20.0 255.255.255.0 192.168.30.2 90

C.  R1(config)#ip route 192.168.20.0 255.255.255.0 192.168.30.2 111

D.  R1(config)#ip route 192.168.20.0 255.255.255.0 192.168.30.2

Answer: C

──────────────────────────────────────────────────

## Question 525  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which route must be configured on R1 so that OSPF routing is used when OSPF is up. but the server is still

reachable when OSPF goes down?

A.  ip route 10.1.1.10 255.255.255.255 172.16.2.2 100

B.  ip route 10.1.1.0 255.255.255.0 gi0/1 125

C.  ip route 10.1.1.0 255.255.255.0 172.16.2.2 100

D.  ip route 10.1.1.10 255.255.255.255 gi0/0 125

Answer: D

──────────────────────────────────────────────────

## Question 528  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which command configures OSPF on the point-to-point link between routers R1 and R2?

A.  router-id 10.0.0.15

B.  neighbor 10.1.2.0 cost 180

C.  ipospf priority 100

D.  network 10.0.0.0 0.0.0.255 area 0

Answer: D

──────────────────────────────────────────────────

## Question 558  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An OSPF neighbor relationship must be configured using these guidelines:

• R1 is only permitted to establish a neighbor with R2

• R1 will never participate in DR elections

• R1 will use a router-id of 101.1.1. Which configuration must be used?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 562  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An engineer just installed network 10.120.10.0/24. Which configuration must be applied

to the R14 router to add the new network to its OSPF routing table?

A.  router ospf 100network 10.120.10.0 255.255.255.0 area 0

B.  router ospf 120network 10.120.10.0 255.255.255.0 area 0ip route 10.120.10.0 255.255.255.0 fa0/1

C.  router ospf 100 area 0network 10.120.10.0 0.0.0.255

Answer: B

──────────────────────────────────────────────────

## Question 572  (Topic 4)  —  Multiple Choice  [V80.2]

A router has two static routes to the same destination network under the same OSPF process. How does

the router forward packets to the destination if the next-hop devices are different?

A.  The router chooses the route with the oldest age.

B.  The router load-balances traffic over all routes to the destination.

C.  The router chooses the next hop with the lowest MAC address.

D.  The router chooses the next hop with the lowest IP address.

Answer: B

──────────────────────────────────────────────────

## Question 581  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

How must OSPF be configured on the GigabitEthernet0/0 interface of the neighbor device to achieve.

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: A

──────────────────────────────────────────────────

## Question 606  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

R1 has taken the DROTHER role in the OSPF DR/BDR election process. Which configuration must an

engineer implement so that R1 is elected as the DR?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 656  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An engineer is asked to confère router R1 so that it forms an OSPF single-area

neighbor relationship with R2. Which command sequence must be implemented to configure the router?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 674  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router OldR is replacing another router on the network with the intention of having OldR and R2 exchange

routes_ After the engineer applied the initial OSPF

configuration: the routes were still missing on both devices. Which command sequence must be issued

before the clear IP ospf process command is entered to enable the neighbor relationship?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 697  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R1 receives static routing updates from routers A. B, C, and D. The network engineer wants R1 to

advertise static routes in OSPF area 1. Which nary address must be advertised in OSPF?

A.  10.1.40.0/25

B.  10.1.40.0/24

C.  10.1.40.0/23

D.  10.1.41.0/25

Answer: C

──────────────────────────────────────────────────

## Question 708  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. R1 has taken the DROTHER role in the OSPF DR/BDR election process. Which

configuration must an engineer implement so that R1 is elected as the DR?

A.  R1(config)#interface FastEthernet 0/0R1(config-if)#ip ospf priority 1R1#clear ip ospf process

B.  R2(config)#interface FastEthernet 0/2R2(config-if)#ip ospf priority 1R2#clear ip ospf process

C.  R1(config)#interface FastEthernet 0/0R1(config-if)#ip ospf priority 200R1#clear ip ospf process

D.  R3(config)#interface FastEthernet 0/1R3(config-if)#ip ospf priority 200R3#clear ip ospf process

Answer: C

──────────────────────────────────────────────────

## Question 713  (Topic 4)  —  Multiple Choice  [V80.2]

A user configured OSPF and advertised the Gigabit Ethernet interface in OSPF. By default, to which type of

OSPF network does this interface belong?

A.  point-to-point

B.  non-broadcast

C.  point-to-multipoint

D.  broadcast

Answer: D

──────────────────────────────────────────────────

## Question 742  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The network engineer is configuring router R2 as a replacement router on the network After the initial

configuration is applied it is determined that R2 failed to show R1 as a neighbor Which configuration must

be applied to R2 to complete the OSPF configuration and enable it to establish the neighbor relationship

with R1?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 775  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer executes the show ip route command on router D. What is the next hop to network

192.168 1 0/24 and why?

A.  The next hop is 10.0.2.1 because it uses distance vector routing

B.  The next hop is 10.0.2.1 because it is a link-state routing protocol

C.  The next hop is 10.0.0.1 because it has a better administrative distance

D.  The next hop is 10.0.0.1 because it has a higher metric.

Answer: B

──────────────────────────────────────────────────

## Question 780  (Topic 4)  —  Multiple Choice  [V80.2]

R1 has learned route 192.168.12.0/24 via IS-IS, OSPF, RIP, and Internal EIGRP. Under normal operating

conditions, which routing protocol's route will be installed in the routing table?

A.  IS-IS

B.  RIP

C.  OSPF

D.  Internal EIGRP

Answer: D

──────────────────────────────────────────────────

## Question 829  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

All interfaces are configured with duplex auto and ip ospf network broadcast. Which configuration allows

routers R14 and R86 to form an OSPFv2 adjacency and act as a central point for exchanging OSPF

information between routers?

A.  Option A

B.  Option B

C.  Option C

D.  option D

Answer: B

──────────────────────────────────────────────────

## Question 831  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer started to configure two directly-connected routers as shown. Which command

sequence must the engineer configure on R2 so that the two routers become OSPF neighbors?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 882  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which action must be taken to ensure that router A is elected as the DR for OSPF area 0?

A.  Configure the router A interfaces with the highest OSPF priority value within the area.

B.  Configure router B and router C as OSPF neighbors of router A.

C.  Configure the OSPF priority on router A with the lowest value between the three routers.

D.  Configure router A with a fixed OSPF router ID.

Answer: A

──────────────────────────────────────────────────

## Question 944  (Topic 4)  —  Multiple Choice  [V80.2]

A Cisco engineer notices thai two OSPF neighbors are connected using a crossover Ethernet cable. The

neighbors are taking too long to become fully adjacent. Which command must be issued under the interface

configuration on each router to reduce the time required for the adjacency to reach the FULL state?

A.  ip ospf network broadcast

B.  ip ospf dead-interval 40

C.  ip ospf network point-to-point

D.  ip ospf priority 0

Answer: C

──────────────────────────────────────────────────

## Question 945  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The show ip ospf interface command has been executed on R1. How is OSPF configured?

A.  A point-to-point network type is configured.

B.  The default Hello and Dead timers are in use.

C.  There are six OSPF neighbors on this interface.

D.  The interface is not participating in OSPF.

Answer: B

──────────────────────────────────────────────────

## Question 1074  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. OSPF neighbors routers A, B, C, and D are sending a route for 10.227.150.160/27.

When the current route for 10.227.150.160/27 becomes unavailable, which cost will router Y use to route

traffic to 10.227.150.160/27?

A.  Cost 20

B.  Cost 30

C.  Cost 40

D.  Cost 50

Answer: A

──────────────────────────────────────────────────

## Question 1083  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Of the routes learned with dynamic routing protocols, which has the least preferred

metric?

A.  EIGRP

B.  OSPF

C.  Local

D.  RIP

Answer: D

──────────────────────────────────────────────────

## Question 1112  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. How must OSPF be configured on the GigabitEthernet0/0 interface of the neighbor

device to achieve the desired neighbor relationship?

A.  Router(config)#interface GigabitEthernet 0/0Router(config-if)#ip ospf network point-to- point

B.  Router(config)#interface GigabitEthernet 0/0Router(config-if)#ip ospf cost 5

C.  Router(config)#interface GigabitEthernet 0/0Router(config-if)#ip ospf 1 area 2

D.  Router(config)#interface GigabitEthernet 0/0Router(config-if)#ip ospf priority 1

Answer: A

──────────────────────────────────────────────────

## Question 1164  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Which routes are configured with their default administrative distances?

A.  EIGRP

B.  OSPF

C.  RIP

D.  Local

Answer: B

──────────────────────────────────────────────────

## Question 1166  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Of the routes learned with dynamic routing protocols, which has the least preferred metric?

A.  Local

B.  EIGRP

C.  OSPF

D.  RIP

Answer: D

──────────────────────────────────────────────────

## Question 1172  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Which routes are configured with their default administrative distances?

A.  Local

B.  EIGRP

C.  RIP

D.  OSPF

Answer: D

──────────────────────────────────────────────────

## Question 1212  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. OSPF neighbors routers A, B, C, and D are sending a route for 10.227.150.160/27.

When the current route for 10.227.150.160/27 becomes unavailable, which cost will router Y use to route

traffic to 10.227.150.160/27?

A.  cost 20

B.  cost 30

C.  cost 40

D.  cost 50

Answer: C