# CCNA 200-301
PoE

Power over Ethernet standards

7 Questions

Multiple Choice: 7 | Drag & Drop: 0

Sources: V7.1 (2) | V80.2 (5)

Topic 2: 2 | Topic 3: 2 | Topic 4: 3

## Question 68  (Topic 3)  —  Multiple Choice  [V7.1]

Which PoE mode enables powered-device detection and guarantees power

when the device is detected?

A.  dynamic

B.  static

C.  active

D.  auto

Answer: B

──────────────────────────────────────────────────

## Question 171  (Topic 2)  —  Multiple Choice  [V7.1]

Which action is taken by a switch port enabled for PoE power

classification override?

A.  When a powered device begins drawing power from a PoE switch port a

syslog message is generated

B.  As power usage on a PoE switch port is checked data flow to the

connected device is temporarily paused

C.  If a switch determines that a device is using less than the minimum

configured power it assumes the device has failed and disconnects

D.  Should a monitored port exceeds the maximum administrative value for

power, the port is shutdown and err-disabled

Answer: D

Explanation:

Reference:

[https://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst6500/ios/12-2SX/configuration/guide/book/power_over_ethernet.pdf](http://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst6500/ios/12-2SX/configuration/guide/book/power_over_ethernet.pdf)

PoE monitoring and policing compares the power consumption on ports

with the administrative maximum value (either a configured maximum

value or the port's default value). If the power consumption on a

monitored port exceeds the administrative maximum value, the following

actions occur:-- A syslog message is issued.-- The monitored port is

shut down and error-disabled.-- The allocated power is freed.

──────────────────────────────────────────────────

## Question 429  (Topic 2)  —  Multiple Choice  [V80.2]

Which action is taken by a switch port enabled for PoE power classification override?

A.  When a powered device begins drawing power from a PoE switch port a syslog message is generated

B.  As power usage on a PoE switch port is checked data flow to the connected device is temporarily

paused

C.  If a switch determines that a device is using less than the minimum configured power it assumes the

device has failed and disconnects

D.  Should a monitored port exceeds the maximum administrative value for power, the port is shutdown and

err-disabled

Answer: D

──────────────────────────────────────────────────

## Question 453  (Topic 3)  —  Multiple Choice  [V80.2]

Which PoE mode enables powered-device detection and guarantees power when the device is detected?

A.  dynamic

B.  static

C.  active

D.  auto

Answer: B

──────────────────────────────────────────────────

## Question 711  (Topic 4)  —  Multiple Choice  [V80.2]

What are two lacts that differentiate optical-fiber cabling from copper cabling? (Choose two.)

A.  It is less expensive when purchasing patch cables.

B.  It has a greater sensitivity to changes in temperature and moisture.

C.  It provides greater throughput options.

D.  It carries signals for longer distances.

E.  It carries electrical current further distances for PoE devices.

Answer: C,D

──────────────────────────────────────────────────

## Question 892  (Topic 4)  —  Multiple Choice  [V80.2]

What are two facts that differentiate optical fiber cabling from copper cabling? (Choose two.)

A.  It has a greater sensitivity to changes in temperature and moisture.

B.  It carries signals for longer distances.

C.  It carries electrical current further distances for PoE devices.

D.  It is less expensive when purchasing patch cables.

E.  It provides greater throughput options.

Answer: A,B

──────────────────────────────────────────────────

## Question 947  (Topic 4)  —  Multiple Choice  [V80.2]

What is an advantage of using auto mode versus static mode for power allocation when an access point is

connected to a PoE switch port?

A.  All four pairs of the cable are used

B.  It detects the device is a powered device

C.  The default level is used for the access point

D.  Power policing is enabled at the same time

Answer: D