# CCNA 200-301
SNMP Syslog NetFlow

SNMP, Syslog, NetFlow network monitoring

51 Questions

Multiple Choice: 48 | Drag & Drop: 3

Sources: V7.1 (11) | V80.2 (40)

Topic 1: 18 | Topic 2: 12 | Topic 4: 15 | Topic 5: 6

## Question 171  (Topic 2)  —  Multiple Choice  [V7.1]

Which action is taken by a switch port enabled for PoE power

classification override?

A.  When a powered device begins drawing power from a PoE switch port a

syslog message is generated

B.  As power usage on a PoE switch port is checked data flow to the

connected device is temporarily paused

C.  If a switch determines that a device is using less than the minimum

configured power it assumes the device has failed and disconnects

D.  Should a monitored port exceeds the maximum administrative value for

power, the port is shutdown and err-disabled

Answer: D

Explanation:

Reference:

[https://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst6500/ios/12-2SX/configuration/guide/book/power_over_ethernet.pdf](http://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst6500/ios/12-2SX/configuration/guide/book/power_over_ethernet.pdf)

PoE monitoring and policing compares the power consumption on ports

with the administrative maximum value (either a configured maximum

value or the port's default value). If the power consumption on a

monitored port exceeds the administrative maximum value, the following

actions occur:-- A syslog message is issued.-- The monitored port is

shut down and error-disabled.-- The allocated power is freed.

──────────────────────────────────────────────────

## Question 218  (Topic 2)  —  Multiple Choice  [V7.1]

Which condition must be met before an NMS handles an SNMP trap from an

agent?

A.  The NMS software must be loaded with the MIB associated with the

trap.

B.  The NMS must be configured on the same router as the SNMP agent

C.  The NMS must receive a trap and an inform message from the SNMP

agent within a configured interval

D.  The NMS must receive the same trap from two different SNMP agents to

verify that it is reliable.

Answer: A

──────────────────────────────────────────────────

## Question 231  (Topic 2)  —  Multiple Choice  [V7.1]

What Is a syslog facility?

A.  Host that is configured for the system to send log messages

B.  password that authenticates a Network Management System to receive

log messages

C.  group of log messages associated with the configured severity level

D.  set of values that represent the processes that can generate a log

message

Answer: C

Explanation:

Cisco Community -- Difference between logging level and logging

facility Post by ahmednaas

"The logging facility command basically tells the syslog server where

to put the log message. You configure the syslog server with something

like: local7.debug /var/adm/local7.log

Now, when you use the "logging facility local7" on your device, all

messages with severity "debug" or greater should be saved in

/var/adm/local7.log." Example: on a switch, any process (CDP, SNMP,

etc.) can generate a log message. On a syslog server, the logging

facility is the place where all received messages with the same

priority level are stored.

──────────────────────────────────────────────────

## Question 262  (Topic 2)  —  Multiple Choice  [V7.1]

What benefit does controller-based networking provide versus

traditional networking?

A.  moves from a two-tier to a three-tier network architecture to

provide maximum redundancy

B.  provides an added layer of security to protect from DDoS attacks

C.  allows configuration and monitoring of the network from one

centralized port

D.  combines control and data plane functionality on a single device to

minimize latency

Answer: C

──────────────────────────────────────────────────

## Question 328  (Topic 1)  —  Multiple Choice  [V7.1]

An engineering team asks an implementer to configure syslog for

warning conditions and error conditions. Which command does the

implementer configure to achieve the desired result?

A.  logging trap 5

B.  logging trap 2

C.  logging trap 4

D.  logging trap 3

Answer: C

──────────────────────────────────────────────────

## Question 441  (Topic 1)  —  Multiple Choice  [V7.1]

A network engineer must back up 20 network router configurations

globally within a customer environment. Which protocol allows the

engineer to perform this function using the Cisco IOS MIB?

A.  CDP

B.  SNMP

C.  SMTP

D.  ARP

Answer: B

Explanation:

SNMP is an application-layer protocol that provides a message format

for communication between SNMP managers and agents. SNMP provides a

standardized framework and a common language used for the monitoring

and management of devices in a network.The SNMP framework has three

parts:+ An SNMP manager+ An SNMP agent+ A Management Information Base

(MIB)The Management Information Base (MIB) is a virtual information

storage area for network management information, which consists of

collections of managed objects.With SNMP, the network administrator

can send commands to multiple routers to do the backup

──────────────────────────────────────────────────

## Question 537  (Topic 1)  —  Multiple Choice  [V7.1]

How does Cisco DNA Center gather data from the network?

A.  Network devices use different services like SNMP, syslog, and

streaming telemetry to send data to the controller

B.  Devices establish an iPsec tunnel to exchange data with the

controller

C.  Devices use the call-home protocol to periodically send data to the

controller.

D.  The Cisco CU Analyzer tool gathers data from each licensed network

device and streams it to the controller.

Answer: A

──────────────────────────────────────────────────

## Question 577  (Topic 1)  —  Multiple Choice  [V7.1]

What facilitates a Telnet connection between devices by entering the

device name?

A.  SNMP

B.  DNS lookup

C.  syslog

D.  NTP

Answer: B

──────────────────────────────────────────────────

## Question 586  (Topic 1)  —  Multiple Choice  [V7.1]

If a notice-level messaging is sent to a syslog server, which event

has occurred?

A.  A network device has restarted

B.  An ARP inspection has failed

C.  A routing instance has flapped

D.  A debug operation is running

Answer: C

Explanation:

Usually no action is required when a route flaps so it generates the

notification syslog level message (level 5).

──────────────────────────────────────────────────

## Question 590  (Topic 1)  —  Multiple Choice  [V7.1]

What protocol allows an engineer to back up 20 network router

configurations globally while using the copy function?

A.  SMTP

B.  SNMP

C.  TCP

D.  FTP

Answer: B

──────────────────────────────────────────────────

## Question 600  (Topic 1)  —  Multiple Choice  [V7.1]

On workstations running Microsoft Windows, which protocol provides the

default gateway for the device?

A.  DHCP

B.  STP

C.  SNMP

D.  DNS

Answer: A

──────────────────────────────────────────────────

## Question 21  (Topic 1)  —  Multiple Choice  [V80.2]

A network engineer must back up 20 network router configurations globally within a customer environment.

Which protocol allows the engineer to perform this function using the Cisco IOS MIB?

A.  CDP

B.  SNMP

C.  SMTP

D.  ARP

Answer: B

──────────────────────────────────────────────────

## Question 28  (Topic 1)  —  Multiple Choice  [V80.2]

What are two fundamentals of virtualization? (choose two)

A.  The environment must be configured with one hypervisor that serves solely as a network manager to

monitor SNMP traffic

B.  It allows logical network devices to move traffic between virtual machines and the rest of the physical

network

C.  It allows multiple operating systems and applications to run independently on one physical server.

D.  It allows a physical router to directly connect NICs from each virtual machine into the network

E.  It requires that some servers, virtual machines and network gear reside on the Internet

Answer: B,C

──────────────────────────────────────────────────

## Question 44  (Topic 1)  —  Multiple Choice  [V80.2]

What event has occurred if a router sends a notice level message to a syslog server?

A.  A TCP connection has been torn down

B.  An ICMP connection has been built

C.  An interface line has changed status

D.  A certificate has expired.

Answer: C

──────────────────────────────────────────────────

## Question 49  (Topic 1)  —  Multiple Choice  [V80.2]

What protocol allows an engineer to back up 20 network router configurations globally while using the copy

function?

A.  SMTP

B.  SNMP

C.  TCP

D.  FTP

Answer: B

──────────────────────────────────────────────────

## Question 64  (Topic 1)  —  Multiple Choice  [V80.2]

What facilitates a Telnet connection between devices by entering the device name?

A.  SNMP

B.  DNS lookup

C.  syslog

D.  NTP

Answer: B

──────────────────────────────────────────────────

## Question 71  (Topic 1)  —  Multiple Choice  [V80.2]

How does Cisco DNA Center gather data from the network?

A.  Network devices use different services like SNMP, syslog, and streaming telemetry to send data to the

controller

B.  Devices establish an iPsec tunnel to exchange data with the controller

C.  Devices use the call-home protocol to periodically send data to the controller.

D.  The Cisco CU Analyzer tool gathers data from each licensed network device and streams it to the

controller.

Answer: A

──────────────────────────────────────────────────

## Question 72  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the SNMP manager and agent identifier commands from the left onto the functions on the

right

──────────────────────────────────────────────────

## Question 114  (Topic 1)  —  Multiple Choice  [V80.2]

An engineering team asks an implementer to configure syslog for warning conditions and error conditions.

Which command does the implementer configure to achieve the desired result?

A.  logging trap 5

B.  logging trap 2

C.  logging trap 4

D.  logging trap 3

Answer: C

──────────────────────────────────────────────────

## Question 127  (Topic 1)  —  Multiple Choice  [V80.2]

On workstations running Microsoft Windows, which protocol provides the default gateway for the device?

A.  DHCP

B.  STP

C.  SNMP

D.  DNS

Answer: A

──────────────────────────────────────────────────

## Question 176  (Topic 1)  —  Multiple Choice  [V80.2]

If a notice-level messaging is sent to a syslog server, which event has occurred?

A.  A network device has restarted

B.  An ARP inspection has failed

C.  A routing instance has flapped

D.  A debug operation is running

Answer: C

──────────────────────────────────────────────────

## Question 186  (Topic 1)  —  Drag & Drop  [V80.2]

Drag and drop the SNMP components from the left onto the descriptions on the right.

──────────────────────────────────────────────────

## Question 271  (Topic 2)  —  Multiple Choice  [V80.2]

Which condition must be met before an NMS handles an SNMP trap from an agent?

A.  The NMS software must be loaded with the MIB associated with the trap.

B.  The NMS must be configured on the same router as the SNMP agent

C.  The NMS must receive a trap and an inform message from the SNMP agent within a configured interval

D.  The NMS must receive the same trap from two different SNMP agents to verify that it is reliable.

Answer: A

──────────────────────────────────────────────────

## Question 273  (Topic 2)  —  Multiple Choice  [V80.2]

What is a role of access points in an enterprise network?

A.  connect wireless devices to a wired network

B.  support secure user logins to devices or the network

C.  integrate with SNMP in preventing DDoS attacks

D.  serve as a first line of defense in an enterprise network

Answer: A

──────────────────────────────────────────────────

## Question 327  (Topic 2)  —  Multiple Choice  [V80.2]

What benefit does controller-based networking provide versus traditional networking?

A.  moves from a two-tier to a three-tier network architecture to provide maximum redundancy

B.  provides an added layer of security to protect from DDoS attacks

C.  allows configuration and monitoring of the network from one centralized port

D.  combines control and data plane functionality on a single device to minimize latency

Answer: C

──────────────────────────────────────────────────

## Question 336  (Topic 2)  —  Multiple Choice  [V80.2]

Which function does an SNMP agent perform?

A.  it sends information about MIB variables in response to requests from the NMS

B.  it requests information from remote network nodes about catastrophic system events.

C.  it manages routing between Layer 3 devices in a network

D.  it coordinates user authentication between a network device and a TACACS+ or RADIUS server

Answer: A

──────────────────────────────────────────────────

## Question 358  (Topic 2)  —  Multiple Choice  [V80.2]

When deploying syslog, which severity level logs informational message?

A.  0

B.  2

C.  4

D.  6

Answer: D

──────────────────────────────────────────────────

## Question 415  (Topic 2)  —  Multiple Choice  [V80.2]

What Is a syslog facility?

A.  Host that is configured for the system to send log messages

B.  password that authenticates a Network Management System to receive log messages

C.  group of log messages associated with the configured severity level

D.  set of values that represent the processes that can generate a log message

Answer: C

──────────────────────────────────────────────────

## Question 429  (Topic 2)  —  Multiple Choice  [V80.2]

Which action is taken by a switch port enabled for PoE power classification override?

A.  When a powered device begins drawing power from a PoE switch port a syslog message is generated

B.  As power usage on a PoE switch port is checked data flow to the connected device is temporarily

paused

C.  If a switch determines that a device is using less than the minimum configured power it assumes the

device has failed and disconnects

D.  Should a monitored port exceeds the maximum administrative value for power, the port is shutdown and

err-disabled

Answer: D

──────────────────────────────────────────────────

## Question 436  (Topic 2)  —  Multiple Choice  [V80.2]

Which technology must be implemented to configure network device monitoring with the highest security?

A.  IP SLA

B.  syslog

C.  NetFlow

D.  SNMPv3

Answer: C

──────────────────────────────────────────────────

## Question 542  (Topic 4)  —  Multiple Choice  [V80.2]

What describes a northbound REST API for SON?

A.  application-facing interface for SNMP GET requests

B.  network-element-facing interface for GET POST PUT and DELETE methods

C.  network-element-facing interface for the control and data planes

D.  application-facing interface far GET, POST, PUT, and DELETE methods

Answer: D

──────────────────────────────────────────────────

## Question 568  (Topic 4)  —  Multiple Choice  [V80.2]

Which protocol does a REST API use to communicate?

A.  SSH

B.  SNMP

C.  HTTP

D.  STP

Answer: C

──────────────────────────────────────────────────

## Question 595  (Topic 4)  —  Multiple Choice  [V80.2]

What is the role of syslog level 7 in network device health monitoring?

A.  It provides information about error conditions visible on the network device.

B.  It shares normal operational messages from the network equipment.

C.  It sends outputs from various debug commands on the device.

D.  It warns about emergency conditions on the network appliance.

Answer: C

──────────────────────────────────────────────────

## Question 614  (Topic 4)  —  Multiple Choice  [V80.2]

What is the difference between SNMP traps and SNMP polling?

A.  SNMP traps are used for proactive monitoring, and SNMP polling is used for reactive monitoring.

B.  SNMP traps send periodic updates via the MIB, and SNMP polling sends data on demand.

C.  SNMP traps are initiated by the network management system, and network devices initiate SNMP

polling.

D.  SNMP traps are initiated using a push model at the network device, and SNMP polling is initiated at the

server.

Answer: B

──────────────────────────────────────────────────

## Question 630  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the SNMP components from the left onto the description on the right.

──────────────────────────────────────────────────

## Question 635  (Topic 4)  —  Multiple Choice  [V80.2]

Which event has occurred if a router sends a notice level message to a syslog server?

A.  An ICMP connection has been built

B.  A certificate has expired

C.  An interface line has changed status

D.  A TCP connection has been torn down

Answer: C

──────────────────────────────────────────────────

## Question 652  (Topic 4)  —  Multiple Choice  [V80.2]

What is the role of SNMP in the network?

A.  to monitor network devices and functions using a TCP underlay that operates on the presentation layer

B.  to collect data directly from network devices using an SSL underlay that operates on the transport layer

C.  to monitor and manage network devices using a UDP underlay that operates on the application layer

D.  to collect telemetry and critical information from network devices using an SSH underlay that operates

on the network layer

Answer: C

──────────────────────────────────────────────────

## Question 677  (Topic 4)  —  Multiple Choice  [V80.2]

Which protocol does Ansible use to push modules to nodes in a network?

A.  SSH

B.  SNMP

C.  Kerberos

D.  Telnet

Answer: A

──────────────────────────────────────────────────

## Question 681  (Topic 4)  —  Multiple Choice  [V80.2]

Which protocol is implemented when an organization must verify network performance, troubleshoot issues,

and use an agent to communicate between monitoring tools and end devices?

A.  SNMP

B.  FTP

C.  NTP

D.  NFS

Answer: D

──────────────────────────────────────────────────

## Question 682  (Topic 4)  —  Multiple Choice  [V80.2]

Which syslog severity level is considered the most severe and results in the system being considered

unusable?

A.  Alert

B.  Error

C.  Emergency

D.  Critical

Answer: C

──────────────────────────────────────────────────

## Question 783  (Topic 4)  —  Multiple Choice  [V80.2]

Which command implies the use of SNMPv3?

A.  snmp-server host

B.  snmp-server community

C.  snmp-server enable traps

D.  snmp-server user

Answer: B

──────────────────────────────────────────────────

## Question 801  (Topic 4)  —  Multiple Choice  [V80.2]

What is the role of community strings in SNMP operations?

A.  It serves as a sequence tag on SNMP traffic messages.

B.  It serves as a password lo protect access to MIB objects.

C.  It passes the Active Directory username and password that are required for device access

D.  It translates alphanumeric MIB output values to numeric values.

Answer: B

──────────────────────────────────────────────────

## Question 858  (Topic 4)  —  Multiple Choice  [V80.2]

A network administrator wants the syslog server to filter incoming messages into different files based on

their Importance. Which filtering criteria must be used?

A.  level

B.  message body

C.  process ID

D.  facility

Answer: A

──────────────────────────────────────────────────

## Question 906  (Topic 4)  —  Multiple Choice  [V80.2]

How does authentication differ from authorization?

A.  Authentication verifies the identity of a person accessing a network, and authorization determines what

resource a user can access.

B.  Authentication is used to record what resource a user accesses, and authorization is used to determine

what resources a user can access

C.  Authentication is used to determine what resources a user is allowed to access, and authorization is

used to track what equipment is allowed access to the network

D.  Authentication is used to verify a person's identity, and authorization is used to create syslog messages

for logins.

Answer: A

──────────────────────────────────────────────────

## Question 1023  (Topic 4)  —  Multiple Choice  [V80.2]

What is the purpose of configuring different levels of syslog for different devices on the network?

A.  to rate-limit messages for different seventy levels from each device

B.  to set the severity of syslog messages from each device

C.  to identify the source from which each syslog message originated

D.  to control the number of syslog messages from different devices that are stored locally

Answer: B

──────────────────────────────────────────────────

## Question 1087  (Topic 5)  —  Multiple Choice  [V80.2]

Which two transport layer protocols carry syslog messages? (Choose two.)

A.  UDP

B.  RTP

C.  TCP

D.  IP

E.  ARP

Answer: A,C

──────────────────────────────────────────────────

## Question 1093  (Topic 5)  —  Multiple Choice  [V80.2]

Which fact must the engineer consider when implementing syslog on a new network?

A.  Syslog defines the software or hardware component that triggered the message.

B.  There are 16 different logging levels (0-15).

C.  By default, all message levels are sent to the syslog server.

D.  The logging level defines the severity of a particular message.

Answer: D

──────────────────────────────────────────────────

## Question 1165  (Topic 5)  —  Multiple Choice  [V80.2]

Which role does AI play in monitoring network data flow?

A.  It analyzes patterns for anomaly detection.

B.  It exclusively predicts device malfunctions.

C.  It simplifies traffic route mapping.

D.  It enhances data packet delivery speeds.

Answer: A

──────────────────────────────────────────────────

## Question 1201  (Topic 5)  —  Multiple Choice  [V80.2]

Which syslog message logging level displays interface line protocol up/down events?

A.  Informational

B.  debugging

C.  alerts

D.  notifications

Answer: D

──────────────────────────────────────────────────

## Question 1233  (Topic 5)  —  Multiple Choice  [V80.2]

How does automation leverage data models to reduce the operational complexity of a managed network?

A.  Reduces the response time for specific requests to devices with many interfaces

B.  Categorizes traffic and provides insights

C.  Allows the controller to be vendor-agnostic

D.  Streamlines monitoring using SNMP and other polling tools

Answer: C

──────────────────────────────────────────────────

## Question 1237  (Topic 5)  —  Multiple Choice  [V80.2]

Which action prevents debug messages from being sent via syslog while allowing other messages when an

abnormally high number of syslog messages are generated by a device with the debug process turned on?

A.  Use an access list to filter out the syslog messages.

B.  Turn off the logging monitor in global configuration mode.

C.  Disable logging to the console.

D.  Set the logging trap severity level to informational.

Answer: D