# CCNA 200-301
Static and Default Routing

Static routes, default routes, routing tables, and administrative distance

131 Questions

Multiple Choice: 128 | Drag & Drop: 3

Sources: V7.1 (17) | V80.2 (114)

Topic 1: 25 | Topic 2: 24 | Topic 3: 14 | Topic 4: 48 | Topic 5: 20

## Question 9  (Topic 3)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

A static route must be configured on R14 to forward traffic for the

172 21 34 0/25 network that resides on R86 Which command must be used

to fulfill the request?

A.  ip route 172.21.34.0 255.255.255.192 10.73.65.65

B.  ip route 172.21.34.0 255.255.255.0 10.73.65.65

C.  ip route 172.21.34.0 255.255.128.0 10.73.65.64

D.  ip route 172.21.34.0 255.255.255.128 10.73.65.66

Answer: D

──────────────────────────────────────────────────

## Question 20  (Topic 3)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Which route must be configured on R1 so that OSPF routing is used when

OSPF is up. but the server is still reachable when OSPF goes down?

A.  ip route 10.1.1.10 255.255.255.255 172.16.2.2 100

B.  ip route 10.1.1.0 255.255.255.0 gi0/1 125

C.  ip route 10.1.1.0 255.255.255.0 172.16.2.2 100

D.  ip route 10.1.1.10 255.255.255.255 gi0/0 125

Answer: D

──────────────────────────────────────────────────

## Question 88  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

Which route type is configured to reach the internet?

A.  host route

B.  default route

C.  floating static route

D.  network route

Answer: B

──────────────────────────────────────────────────

## Question 104  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Router R1 is running three different routing protocols. Which route

characteristic is used by the router to forward the packet that it

receives for destination IP 172.16.32.1?

A.  longest prefix

B.  metric

C.  cost

D.  administrative distance

Answer: A

Explanation:

https://learningnetwork.cisco.com/s/question/0D53i00000KszSlCAJ/administrative-

distance-vs-longest-match-rule

──────────────────────────────────────────────────

## Question 116  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Which command configures a floating static route to provide a backup

to the primary link?

A.  ip route 0.0.0.0 0.0.0.0 209.165.202.131

B.  ip route 209.165.201.0 255.255.255.224 209.165.202.130

C.  ip route 0.0.0.0 0.0.0.0 209.165.200.224

D.  ip route 209.165.200.224 255.255.255.224 209.165.202.129 254

Answer: D

──────────────────────────────────────────────────

## Question 175  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Which two prefixes are included in this routing table entry? (Choose

two.)

A.  192.168.1.17

B.  192.168.1.61

C.  192.168.1.64

D.  192.168.1.127

E.  192.168.1.254

Answer: BC

──────────────────────────────────────────────────

## Question 181  (Topic 2)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 2)

[See Exhibit Image]Refer to the exhibit.

Drag and drop the networking parameters from the left onto the correct

values on the right.

[See Exhibit Image]

A.  Mastered

B.  Not Mastered

Answer: A

Explanation:

NIC vendor OUI --\> 00:0C:22NIC MAC address --\>

00:0C:22:83:79:A3default gateway --\>192.168.1.193host IP address --\>

192.168.1.200subnet mask --\> 255.255.255.192

The "ip route" and "ip addr show eth1" are Linux commands.+ "ip

route": display the routing table+ "ip addr show eth1": get depth

information (only on eth1 interface) about your network interfaces

like IP Address, MAC Address information

──────────────────────────────────────────────────

## Question 182  (Topic 2)  —  Multiple Choice  [V7.1]

What are two benefits of FHRPs? (Choose two.)

A.  They prevent (oops in the Layer 2 network.

B.  They allow encrypted traffic.

C.  They are able to bundle muftlple ports to increase bandwidth

D.  They enable automatic failover of the default gateway.

E.  They allow multiple devices lo serve as a single virtual gateway for

clients in the network

Answer: DE

──────────────────────────────────────────────────

## Question 192  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

The default-information originate command is configured under the R1

OSPF configuration After testing workstations on VLAN 20 at Site B

cannot reach a DNS server on the Internet Which action corrects the

configuration issue?

A.  Add the default-information originate command onR2

B.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.18 command on R1

C.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.2 command on R2

D.  Add the always keyword to the default-information originate command

on R1

Answer: B

──────────────────────────────────────────────────

## Question 210  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

An engineer configured the New York router with state routes that

point to the Atlanta and Washington sites. When command must be

configured on the Atlanta and Washington routers so that both sites

are able to reach the loopback2 interface on the New York router?

A.  ipv6 route ::/0 Serial 0/0/1

B.  ipv6 route 0/0 Serial 0/0/0

C.  ipv6 route ::/0 Serial 0/0/0

D.  ip route 0.0.0.0.0.0.0.0 Serial 0/0/0

E.  ipv6 route ::/0 2000::2

Answer: C

──────────────────────────────────────────────────

## Question 324  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]

Refer to the exhibit Router R1 Fa0/0 is unable to ping router R3

Fa0\'1. Which action must be taken in router R1 to help resolve the

configuration issue?

A.  set the default network as 20.20.20.0/24

B.  set the default gateway as 20.20.20.2

C.  configure a static route with Fa0/1 as the egress interface to reach

the 20.20.20.0/24 network

D.  configure a static route with 10.10.10.2 as the next hop to reach

the 20.20.20.0/24 network

Answer: D

──────────────────────────────────────────────────

## Question 338  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

Router R4 is dynamically learning the path to the server. If R4 is

connected to R1 via OSPF Area 20, to R2 v2ia R2 BGP, and to R3 via

EIGRP 777, which path is installed in the routing table of R4?

A.  the path through R1, because the OSPF administrative distance is 110

B.  the path through R2. because the IBGP administrative distance is 200

C.  the path through R2 because the EBGP administrative distance is 20

D.  the path through R3. because the EIGRP administrative distance is

lower than OSPF and BGP

Answer: C

──────────────────────────────────────────────────

## Question 383  (Topic 1)  —  Multiple Choice  [V7.1]

How does HSRP provide first hop redundancy?

A.  It load-balances traffic by assigning the same metric value to more

than one route to the same destination m the IP routing table.

B.  It load-balances Layer 2 traffic along the path by flooding traffic

out all interfaces configured with the same VLAN.

C.  It forwards multiple packets to the same destination over different

routed links n the data path

D.  It uses a shared virtual MAC and a virtual IP address to a group of

routers that serve as the default gateway for hosts on a LAN

Answer: D

Explanation:

[https://www.cisco.com/c/en/us/td/docs/ios-xml/ios/ipapp_fhrp/configuration/xe-16/fhp-xe-16-book/fhp-hsrp-mgo.html](http://www.cisco.com/c/en/us/td/docs/ios-xml/ios/ipapp_fhrp/configuration/xe-16/fhp-xe-16-book/fhp-hsrp-mgo.html)

──────────────────────────────────────────────────

## Question 445  (Topic 1)  —  Multiple Choice  [V7.1]

When a floating static route is configured, which action ensures that

the backup route is used when the primary route fails?

A.  The floating static route must have a higher administrative distance

than the primary route so it is used as a backup

B.  The administrative distance must be higher on the primary route so

that the backup route becomes secondary.

C.  The floating static route must have a lower administrative distance

than the primary route so it is used as a backup

D.  The default-information originate command must be configured for the

route to be installed into the routing table

Answer: A

──────────────────────────────────────────────────

## Question 524  (Topic 1)  —  Multiple Choice  [V7.1]

By default, how Does EIGRP determine the metric of a route for the

routing table?

A.  it uses the bandwidth and delay values of the path to calculate the

route metric

B.  it uses a default metric of 10 for all routes that are learned by

the router

C.  it uses a reference Bandwidth and the actual bandwidth of the

connected link to calculate the route metric

D.  it counts the number of hops between the receiving and destination

routers and uses that value as the metric

Answer: A

──────────────────────────────────────────────────

## Question 573  (Topic 1)  —  Multiple Choice  [V7.1]

Router R1 must send all traffic without a matching routing-table entry

to 192.168.1.1. Which configuration accomplishes this task?

[See Exhibit Image]

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 600  (Topic 1)  —  Multiple Choice  [V7.1]

On workstations running Microsoft Windows, which protocol provides the

default gateway for the device?

A.  DHCP

B.  STP

C.  SNMP

D.  DNS

Answer: A

──────────────────────────────────────────────────

## Question 5  (Topic 1)  —  Multiple Choice  [V80.2]

By default, how Does EIGRP determine the metric of a route for the routing table?

A.  it uses the bandwidth and delay values of the path to calculate the route metric

B.  it uses a default metric of 10 for all routes that are learned by the router

C.  it uses a reference Bandwidth and the actual bandwidth of the connected link to calculate the route

metric

D.  it counts the number of hops between the receiving and destination routers and uses that value as the

metric

Answer: A

──────────────────────────────────────────────────

## Question 20  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R4 is dynamically learning the path to the server. If R4 is connected to R1 via OSPF Area 20, to R2

v2ia R2 BGP, and to R3 via EIGRP 777, which path is installed in the routing table of R4?

A.  the path through R1, because the OSPF administrative distance is 110

B.  the path through R2. because the IBGP administrative distance is 200

C.  the path through R2 because the EBGP administrative distance is 20

D.  the path through R3. because the EIGRP administrative distance is lower than OSPF and BGP

Answer: C

──────────────────────────────────────────────────

## Question 35  (Topic 1)  —  Multiple Choice  [V80.2]

Which two outcomes are predictable behaviors for HSRP? (Choose two.)

A.  The two routers synchronize configurations to provide consistent packet forwarding

B.  The two routers negotiate one router as the active router and the other as the standby router

C.  Each router has a different IP address, both routers act as the default gateway on the LAN, and traffic is

load-balanced between them

D.  The two routers share a virtual IP address that is used as the default gateway for devices on the LAN

E.  The two routers share the same interface IP address and default gateway traffic is load- balanced

between them

Answer: B,D

──────────────────────────────────────────────────

## Question 65  (Topic 1)  —  Multiple Choice  [V80.2]

What is a benefit of VRRP?

A.  It provides traffic load balancing to destinations that are more than two hops from the source.

B.  It provides the default gateway redundancy on a LAN using two or more routers.

C.  It allows neighbors to share routing table information between each other.

D.  It prevents loops in a Layer 2 LAN by forwarding all traffic to a root bridge, which then makes the final

forwarding decision.

Answer: B

──────────────────────────────────────────────────

## Question 78  (Topic 1)  —  Multiple Choice  [V80.2]

Router R1 must send all traffic without a matching routing-table entry to 192.168.1.1. Which configuration

accomplishes this task?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 79  (Topic 1)  —  Multiple Choice  [V80.2]

Which network action occurs within the data plane?

A.  compare the destination IP address to the IP routing table.

B.  run routing protocols (OSPF, EIGRP, RIP, BGP)

C.  make a configuration change from an incoming NETCONF RPC

D.  reply to an incoming ICMP echo request

Answer: A

──────────────────────────────────────────────────

## Question 117  (Topic 1)  —  Multiple Choice  [V80.2]

When DHCP is configured on a router, which command must be entered so the default gateway is

automatically distributed?

A.  default-router

B.  default-gateway

C.  ip helper-address

D.  dns-server

Answer: A

──────────────────────────────────────────────────

## Question 118  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A router reserved these five routes from different routing information sources. Which two routes does the

router install in its routing table? (Choose two)

A.  RIP route 10.0.0.0/30

B.  iBGP route 10.0.0.0/30

C.  OSPF route 10.0.0.0/30

D.  EIGRP route 10.0.0.1/32

E.  OSPF route 10.0.0.0/16

Answer: C,D

──────────────────────────────────────────────────

## Question 125  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which type of route does R1 use to reach host 10.10.13.10/32?

A.  floating static route

B.  host route

C.  default route

D.  network route

Answer: D

──────────────────────────────────────────────────

## Question 127  (Topic 1)  —  Multiple Choice  [V80.2]

On workstations running Microsoft Windows, which protocol provides the default gateway for the device?

A.  DHCP

B.  STP

C.  SNMP

D.  DNS

Answer: A

──────────────────────────────────────────────────

## Question 135  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit Router R1 Fa0/0 is unable to ping router R3 Fa0'1. Which action must be taken in router

R1 to help resolve the configuration issue?

A.  set the default network as 20.20.20.0/24

B.  set the default gateway as 20.20.20.2

C.  configure a static route with Fa0/1 as the egress interface to reach the 20.20.20.0/24 network

D.  configure a static route with 10.10.10.2 as the next hop to reach the 20.20.20.0/24 network

Answer: D

──────────────────────────────────────────────────

## Question 149  (Topic 1)  —  Multiple Choice  [V80.2]

What is the purpose of using First Hop Redundancy Protocol in a specific subnet?

A.  Filter traffic based on destination IP addressing

B.  Sends the default route to the hosts on a network

C.  ensures a loop-free physical topology

D.  forwards multicast hello messages between routers

Answer: D

──────────────────────────────────────────────────

## Question 156  (Topic 1)  —  Multiple Choice  [V80.2]

When a floating static route is configured, which action ensures that the backup route is used when the

primary route fails?

A.  The floating static route must have a higher administrative distance than the primary route so it is used

as a backup

B.  The administrative distance must be higher on the primary route so that the backup route becomes

secondary.

C.  The floating static route must have a lower administrative distance than the primary route so it is used as

a backup

D.  The default-information originate command must be configured for the route to be installed into the

routing table

Answer: A

──────────────────────────────────────────────────

## Question 158  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which command provides this output?

A.  show ip route

B.  show ip interface

C.  show interface

D.  show cdp neighbor

Answer: D

──────────────────────────────────────────────────

## Question 162  (Topic 1)  —  Multiple Choice  [V80.2]

How does HSRP provide first hop redundancy?

A.  It load-balances traffic by assigning the same metric value to more than one route to the same

destination m the IP routing table.

B.  It load-balances Layer 2 traffic along the path by flooding traffic out all interfaces configured with the

same VLAN.

C.  It forwards multiple packets to the same destination over different routed links n the data path

D.  It uses a shared virtual MAC and a virtual IP address to a group of routers that serve as the default

gateway for hosts on a LAN

Answer: D

──────────────────────────────────────────────────

## Question 164  (Topic 1)  —  Multiple Choice  [V80.2]

Which attribute does a router use to select the best path when two or more different routes to the same

destination exist from two different routing protocols.

A.  dual algorithm

B.  metric

C.  administrative distance

D.  hop count

Answer: C

──────────────────────────────────────────────────

## Question 217  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

How does the router manage traffic to 192.168.12.16?

A.  It selects the RIP route because it has the longest prefix inclusive of the destination address.

B.  It chooses the OSPF route because it has the longest prefix inclusive of the destination address.

C.  it load-balances traffic between all three routes

D.  It chooses the EIGRP route because it has the lowest administrative distance

Answer: A

──────────────────────────────────────────────────

## Question 226  (Topic 1)  —  Multiple Choice  [V80.2]

What is the primary purpose of a First Hop Redundancy Protocol?

A.  It allows directly connected neighbors to share configuration information.

B.  It allows a router to use bridge priorities to create multiple loop-free paths to a single destination.

C.  It reduces routing failures by allowing Layer 3 load balancing between OSPF neighbors that have the

same link metric.

D.  It reduces routing failures by allowing more than one router to represent itself, as the

default gateway of a network.

Answer: D

──────────────────────────────────────────────────

## Question 252  (Topic 2)  —  Multiple Choice  [V80.2]

R1 has learned route 10.10.10.0/24 via numerous routing protocols. Which route is installed?

A.  route with the lowest cost

B.  route with the next hop that has the highest IP

C.  route with the shortest prefix length

D.  route with the lowest administrative distance

Answer: D

──────────────────────────────────────────────────

## Question 284  (Topic 2)  —  Multiple Choice  [V80.2]

R1 has learned route 192.168.12.0/24 via IS-IS. OSPF, RIP. and Internal EIGRP Under normal operating

conditions, which routing protocol is installed in the routing table?

A.  IS-IS

B.  RIP

C.  Internal EIGRP

D.  OSPF

Answer: C

──────────────────────────────────────────────────

## Question 295  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which route type is configured to reach the internet?

A.  host route

B.  default route

C.  floating static route

D.  network route

Answer: B

──────────────────────────────────────────────────

## Question 322  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which two prefixes are included in this routing table entry? (Choose two.)

A.  192.168.1.17

B.  192.168.1.61

C.  192.168.1.64

D.  192.168.1.127

E.  192.168.1.254

Answer: B,C

──────────────────────────────────────────────────

## Question 332  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

How does router R1 handle traffic to 192.168.10.16?

A.  It selects the IS-IS route because it has the shortest prefix inclusive of the destination address.

B.  It selects the EIGRP route because it has the lowest administrative distance.

C.  It selects the OSPF route because it has the lowest cost.

D.  It selects the RIP route because it has the longest prefix inclusive of the destination address.

Answer: D

──────────────────────────────────────────────────

## Question 335  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R1 is running three different routing protocols. Which route characteristic is used by the router to

forward the packet that it receives for destination IP 172.16.32.1?

A.  longest prefix

B.  metric

C.  cost

D.  administrative distance

Answer: A

──────────────────────────────────────────────────

## Question 337  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

If configuring a static default route on the router with the ip route 0.0.0.0 0.0.0.0 10.13.0.1 120 command

how does the router respond?

A.  It ignores the new static route until the existing OSPF default route is removed

B.  It immediately replaces the existing OSPF route in the routing table with the newly configured static route

C.  It starts load-balancing traffic between the two default routes

D.  It starts sending traffic without a specific matching entry in the routing table to GigabitEthernet0/1

Answer: A

──────────────────────────────────────────────────

## Question 340  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which command configures a floating static route to provide a backup to the primary link?

A.  ip route 0.0.0.0 0.0.0.0 209.165.202.131

B.  ip route 209.165.201.0 255.255.255.224 209.165.202.130

C.  ip route 0.0.0.0 0.0.0.0 209.165.200.224

D.  ip route 209.165.200.224 255.255.255.224 209.165.202.129 254

Answer: D

──────────────────────────────────────────────────

## Question 344  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is bringing up a new circuit to the MPLS provider on the Gi0/1 interface of Router1 The new

circuit uses eBGP and teams the route to VLAN25 from the BGP path What s the expected behavior for the

traffic flow for route 10.10.13.0/25?

A.  Traffic to 10.10.13.0.25 is load balanced out of multiple interfaces

B.  Route 10.10.13.0/25 is updated in the routing table as being learned from interface Gi0/1.

C.  Traffic to 10.10.13.0/25 is asymmeteical

D.  Route 10.10.13.0/25 learned via the GiO/0 interface remains in the routing table

Answer: D

──────────────────────────────────────────────────

## Question 354  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The default-information originate command is configured under the R1 OSPF configuration After testing

workstations on VLAN 20 at Site B cannot reach a DNS server on the Internet Which action corrects the

configuration issue?

A.  Add the default-information originate command onR2

B.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.18 command on R1

C.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.2 command on R2

D.  Add the always keyword to the default-information originate command on R1

Answer: B

──────────────────────────────────────────────────

## Question 386  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer configured the New York router with state routes that point to the Atlanta and Washington sites.

When command must be configured on the Atlanta and Washington routers so that both sites are able to

reach the loopback2 interface on the New York router?

A.  ipv6 route ::/0 Serial 0/0/1

B.  ipv6 route 0/0 Serial 0/0/0

C.  ipv6 route ::/0 Serial 0/0/0

D.  ip route 0.0.0.0.0.0.0.0 Serial 0/0/0

E.  ipv6 route ::/0 2000::2

Answer: C

──────────────────────────────────────────────────

## Question 389  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R1 Fa0/0 is unable ping router R3 Fa0/1.

Which action must be taken in router R1 to help resolve the configuration issue?

A.  set the default network as 20.20.20.0/24

B.  set the default gateway as 20.20.20.2

C.  configure a static route with Fa0/1 as the egress interface to reach the 20.20.20.0/24 network

D.  configure a static route with 10.10.10.2 as the next hop to reach the 20.20.20.0/24 network

Answer: D

──────────────────────────────────────────────────

## Question 392  (Topic 2)  —  Multiple Choice  [V80.2]

Which two outcomes are predictable behaviors for HSRP? (Choose two)

A.  The two routers share a virtual IP address that is used as the default gateway for devices on the LAN.

B.  The two routers negotiate one router as the active router and the other as the standby router

C.  Each router has a different IP address both routers act as the default gateway on the LAN, and traffic is

load balanced between them.D The two routers synchronize configurations to provide consistent packet

forwarding

D.  The two routed share the same IP address, and default gateway traffic is load-balanced between them

Answer: A,B

──────────────────────────────────────────────────

## Question 407  (Topic 2)  —  Multiple Choice  [V80.2]

What are two benefits of FHRPs? (Choose two.)

A.  They prevent (oops in the Layer 2 network.

B.  They allow encrypted traffic.

C.  They are able to bundle muftlple ports to increase bandwidth

D.  They enable automatic failover of the default gateway.

E.  They allow multiple devices lo serve as a single virtual gateway for clients in the network

Answer: D,E

──────────────────────────────────────────────────

## Question 409  (Topic 2)  —  Multiple Choice  [V80.2]

Router A learns the same route from two different neighbors, one of the neighbor routers is an OSPF

neighbor and the other is an EIGRP neighbor. What is the administrative distance of the route that will be

installed in the routing table?

A.  20

B.  90

C.  110

D.  115

Answer: B

──────────────────────────────────────────────────

## Question 417  (Topic 2)  —  Multiple Choice  [V80.2]

A packet is destined for 10.10.1.22. Which static route does the router choose to forward the packet?

A.  ip route 10.10.1.0 255.255.255.240 10.10.255.1

B.  ip route 10.10.1.16 255.255.255.252 10.10.255.1

C.  ip route 10.10.1.20 255.255.255.252 10.10.255.1

D.  ip route 10.10.1.20 255.255.255.254 10.10.255.1

Answer: C

──────────────────────────────────────────────────

## Question 457  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A static route must be configured on R14 to forward traffic for the 172 21 34 0/25 network that resides on

R86 Which command must be used to fulfill the request?

A.  ip route 172.21.34.0 255.255.255.192 10.73.65.65

B.  ip route 172.21.34.0 255.255.255.0 10.73.65.65

C.  ip route 172.21.34.0 255.255.128.0 10.73.65.64

D.  ip route 172.21.34.0 255.255.255.128 10.73.65.66

Answer: D

──────────────────────────────────────────────────

## Question 472  (Topic 3)  —  Multiple Choice  [V80.2]

Which action is taken by the data plane within a network device?

A.  forwards traffic to the next hop

B.  constructs a routing table based on a routing protocol

C.  provides CLI access to the network device

D.  looks up an egress interface in the forwarding information base

Answer: A

──────────────────────────────────────────────────

## Question 473  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Packets received by the router from BGP enter via a serial interface at 209.165.201.10. Each route is

present within the routing table. Which interface is used to forward traffic with a destination IP of

10.10.10.24?

A.  F0/10

B.  F0/11

C.  F0/12

D.  F0/13

Answer: B

──────────────────────────────────────────────────

## Question 477  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A company is configuring a failover plan and must implement the default routes in such a way that a floating

static route will assume traffic forwarding when the primary link goes down. Which primary route

configuration must be used?

A.  ip route 0.0.0.0 0.0.0.0 192.168.0.2 GigabitEthernetl/0

B.  ip route 0.0.0.0 0.0.0.0 192.168.0.2 tracked

C.  ip route 0.0.0.0 0.0.0.0 192.168.0.2 floating

D.  ip route 0.0.0.0 0.0.0.0 192.168.0.2

Answer: B

──────────────────────────────────────────────────

## Question 484  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

All traffic enters the CPE router from interface Serial0/3 with an IP address of 192 168 50 1 Web traffic from

the WAN is destined for a LAN network where servers are load-balanced An IP packet with a destination

address of the HTTP virtual IP of 192 1681 250 must be forwarded Which routing table entry does the

router use?

A.  192.168.1.0/24 via 192.168.12.2

B.  192.168.1.128/25 via 192.168.13.3

C.  192.168.1.192/26 via 192.168.14.4

D.  192.168.1.224/27 via 192.168.15.5

Answer: B

──────────────────────────────────────────────────

## Question 497  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which command must be issued to enable a floating static default route on router A?

A.  lp route 0.0.0.0 0.0.0.0 192.168.1.2

B.  ip default-gateway 192.168.2.1

C.  ip route 0.0.0.0 0.0.0.0 192.168.2.1 10

D.  ip route 0.0.0.0 0.0.0.0 192.168.1.2 10

Answer: D

──────────────────────────────────────────────────

## Question 499  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is updating the R1 configuration to connect a new server to the management network. The

PCs on the management network must be blocked from pinging the default gateway of the new server.

Which command must be configured on R1 to complete the task?

A.  R1(config)#lp route 172.16.2.2 255.255.255.248 gi0/1

B.  R1(config)#jp route 172.16.2.2 255.255.255.255 gi0/0

C.  R1(config>#ip route 172.16.2.0 255.255.255.0 192.168.1.15

D.  R1(conflg)#ip route 172.16.2.0 255.255.255.0 192.168.1.5

Answer: C

──────────────────────────────────────────────────

## Question 515  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R1 currently is configured to use R3 as the primary route to the Internet, and the route uses the

default administrative distance settings. A network engineer must configure R1 so that it uses R2 as a

backup, but only if R3 goes down. Which command must the engineer configure on R1 so that it correctly

uses R2 as a backup route, without changing the administrative distance configuration on the link to R3?

A.  ip route 0.0.0.0 0.0.0.0 g0/1 1

B.  ip route 0.0.0.0 0.0.0.0 209.165.201.5 10

C.  ip route 0.0.0.0 0.0.0.0 209.165.200.226 1

D.  ip route 0,0.0.0 0.0.0.0 g0/1 6

Answer: C

──────────────────────────────────────────────────

## Question 521  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

R1 learns all routes via OSPF Which command configures a backup static route on R1 to reach the 192

168.20.0/24 network via R3?

A.  R1(config)#ip route 192.168.20.0 255.255.0.0 192.168.30.2

B.  R1(config)#ip route 192.168.20.0 255.255.255.0 192.168.30.2 90

C.  R1(config)#ip route 192.168.20.0 255.255.255.0 192.168.30.2 111

D.  R1(config)#ip route 192.168.20.0 255.255.255.0 192.168.30.2

Answer: C

──────────────────────────────────────────────────

## Question 524  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Packets received by the router from BGP enter via a serial interface at 209 165 201 1 Each route is present

within the routing table Which interface is used to forward traffic with a destination IP of 10.1.1.19?

A.  F0/4

B.  F0/0

C.  F0/1

D.  F0/3

Answer: B

──────────────────────────────────────────────────

## Question 525  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which route must be configured on R1 so that OSPF routing is used when OSPF is up. but the server is still

reachable when OSPF goes down?

A.  ip route 10.1.1.10 255.255.255.255 172.16.2.2 100

B.  ip route 10.1.1.0 255.255.255.0 gi0/1 125

C.  ip route 10.1.1.0 255.255.255.0 172.16.2.2 100

D.  ip route 10.1.1.10 255.255.255.255 gi0/0 125

Answer: D

──────────────────────────────────────────────────

## Question 538  (Topic 3)  —  Multiple Choice  [V80.2]

What are two benefits of FHRPs? (Choose two.)

A.  They enable automatic failover of the default gateway.

B.  They allow multiple devices to serve as a single virtual gateway for clients in the network.

C.  They are able to bundle multiple ports to increase bandwidth.

D.  They prevent loops in the Layer 2 network.

E.  They allow encrypted traffic.

Answer: A,B Topic 4, Exam Pool D (NEW)

──────────────────────────────────────────────────

## Question 560  (Topic 4)  —  Drag & Drop  [V80.2]

Refer to the exhibit.

The Router1 routing table has multiple methods to reach 10.10.10.0/24 as shown. The default

Administrative Distance is used. Drag and drop the network conditions from the left onto the routing

methods that Router1 uses on the right.

──────────────────────────────────────────────────

## Question 562  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An engineer just installed network 10.120.10.0/24. Which configuration must be applied

to the R14 router to add the new network to its OSPF routing table?

A.  router ospf 100network 10.120.10.0 255.255.255.0 area 0

B.  router ospf 120network 10.120.10.0 255.255.255.0 area 0ip route 10.120.10.0 255.255.255.0 fa0/1

C.  router ospf 100 area 0network 10.120.10.0 0.0.0.255

Answer: B

──────────────────────────────────────────────────

## Question 563  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer must configure a floating static route on an external EIGRP network. The

destination subnet is the /29 on the LAN Interface of R86. Which command must be executed on R14?

A.  ip route 10.80.65.0.255.255.248.0.10.73.65.66.1

B.  ip route 10.80.65.0.255.255.255..240 fa0/1 89

C.  ip route 10.80.65.0.255.255.248.0.10.73.65.66.171

D.  ip route 10.80.65.0.0.0.224.10.80.65.0. 255

Answer: C

──────────────────────────────────────────────────

## Question 575  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer is updating the configuration on router R1 to connect a new branch office to the

company network R2 has been configured correctly. Which command must the engineer configure so that

devices at the new site communicate with the main office?

A.  ip route 172.25.25 0 255 255 255.0 192.168.2.1

B.  ip route 172.25.25 1 255 255 255 255 g0/1

C.  ip route 172.25.25.0.255.255.255.0.192.168.2.2

Answer: C

──────────────────────────────────────────────────

## Question 586  (Topic 4)  —  Multiple Choice  [V80.2]

What does the implementation of a first-hop redundancy protocol protect against on a network?

A.  root-bridge loss

B.  spanning-tree loops

C.  default gateway failure

D.  BGP neighbor flapping

Answer: C

──────────────────────────────────────────────────

## Question 591  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer executed the script and added commands that were not necessary for SSH and now must

remove the commands.

A.  metric

B.  cost

C.  longest prefix

D.  administrative distance

Answer: D

──────────────────────────────────────────────────

## Question 600  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. User traffic originating within site 0 is failing to reach an application hosted on IP

address 192.168 0 10. Which is located within site A What is determined by the routing table?

A.  The default gateway for site B is configured incorrectly

B.  The lack of a default route prevents delivery of the traffic

C.  The traffic is blocked by an implicit deny in an ACL on router2

D.  The traffic to 192 168 010 requires a static route to be configured in router 1.

Answer: B

──────────────────────────────────────────────────

## Question 603  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Routers R1 and R2 are configured with RIP as the dynamic routing protocol. A network engineer must

configure R1 with a floating static route to serve as a backup route to network 192.168.23.0. Which

command must the engineer configure on R1?

A.  ip route 192.168.23.0 255.255.255.255 192.168.13.3 121

B.  ip route 192.168.23.0 255.255.255.0 192.168.13.3 100

C.  ip route 192.168.23.0 265.255.255.0 192.168.13.3 121

D.  ip route 192.168.23.0 255.255.255.0 192.168.13.3

Answer: C

──────────────────────────────────────────────────

## Question 607  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

After configuring a new static route on the CPE. the engineer entered this series of commands to verify that

the new configuration is operating normally When is the static default route installed into the routing table?

A.  when 203 0 113.1 is no longer reachable as a next hop

B.  when the default route learned over external BGP becomes invalid

C.  when a route to 203.0 113 1 is learned via BGP

D.  when the default route over external BGP changes its next hop

Answer: A

──────────────────────────────────────────────────

## Question 615  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. The user has connectivity to devices on network 192.168.3 0/24 but cannot reach users

on the network 10.10.1.0724.

What is the first step to verify connectivity?

A.  Is the internet reachable?

B.  Is the default gateway reachable?

C.  Is the DNS server reachable?

Answer: B

──────────────────────────────────────────────────

## Question 617  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. How will the device handle a packet destined to IP address 100.100.100.100?

A.  If will choose the route with the longest match.O 100.100.100.100'32 (110/21) via 192.168.1.1. 00:05:57.

EmernetO/1.

B.  It will always prefer the static route over dynamic routes and choose the routeS 100.100.0.0/16(1/0] via

192.168.4.1.

C.  It will choose the route with the highest metric.D 100.100.100.0/24 (90/435200) via 192.168.2.1.

00:00:13. EthernetO/2.

D.  It will choose the route with the lowest metric,R 100.0.0.0/8 [120/2] via 192.168.3.1. 00:00:13.

EthernetO/3.

Answer: A

──────────────────────────────────────────────────

## Question 659  (Topic 4)  —  Drag & Drop  [V80.2]

An engineer must configure a core router with a floating static default route to the backup router at

10.200.0.2.

──────────────────────────────────────────────────

## Question 664  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Traffic from R1 to the 10.10.2.0/24 subnet uses 192.168.1.2 as its next hop. An network engineer wants to

update the R1 configuration so that traffic with destination 10.10.2.1 passes through router R3, and all other

traffic to the 10.10.20/24 subnet passes through r2. Which command must be used?

A.  Ip route 10.10.2.1 255.255.255.255 192.168.1.4 115

B.  Ip route 10.10.2.0 255.255.255.0 192.168.1.4 100

C.  Ip route 10.10.2.0 255.255.255.0 192.168.1.4 115

D.  Ip route 10.10.2.1 255.255.255.255 192.168.1.4 100

Answer: D

──────────────────────────────────────────────────

## Question 666  (Topic 4)  —  Multiple Choice  [V80.2]

A HCP pool has been created with the name CONTROL. The pool uses the next to last usable IP address

as the default gateway for the DHCP clients. The server is located at 172.16 32.15. What is the step in the

process for clients on the 192.168.52.0/24 subnet to reach the DHCP server?

A.  ip forward-protocol udp 137

B.  ip default-network 192.168.52.253

C.  ip helper-address 172.16.32.15

D.  ip default-gateway 192.168.52.253

Answer: C

──────────────────────────────────────────────────

## Question 693  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

R1 has just received a packet from host A that is destined to host B. Which route in the routing table is used

by R1 to reac B?

A.  10.10.13.0/25 [108/0] via 10.10.10.10

B.  10.10.13.0/25 [110/2] via 10.10.10.2

C.  10.10.13.0/25 [110/2] via 10.10.10.6

D.  10.10.13.0/25 [1/0] via 10.10.10.2

Answer: D

──────────────────────────────────────────────────

## Question 704  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which configuration parameter is preventing host C from reaching the internet?

A.  automatic DNS

B.  default gateway

C.  IP network mask

D.  IP address assignment

Answer: B

──────────────────────────────────────────────────

## Question 729  (Topic 4)  —  Multiple Choice  [V80.2]

A router received three destination prefixes:10.0.0/18, and 10.0.0/24. When the show ip route command is

executed, which output does it return?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: A

──────────────────────────────────────────────────

## Question 735  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A newly configured PC fails to connect to the internet using TCP port 80 to www cisco com Which setting

must be modified for the connection to work?

A.  Subnet Mask

B.  DNS Servers

C.  Default Gateway

D.  DHCP Server

Answer: B

──────────────────────────────────────────────────

## Question 748  (Topic 4)  —  Multiple Choice  [V80.2]

What is the purpose of using First Hop Redundancy Protocol on a specific subnet?

A.  ensures a loop-free physical topology

B.  filters traffic based on destination IP addressing

C.  sends the default route to the hosts on a network

D.  forwards multicast hello messages between routers

Answer: D

──────────────────────────────────────────────────

## Question 759  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R14 is in the process of being configured. Which configuration must be used to establish a host

route to PC 10?

A.  ip route 10.80.65.10 255.255.255.254 10.80.65.1

B.  ip route 10.8065.10 255.255.255.255 10.73.65.66

C.  ip route 1073.65.65 255.0.0.0 10.80.65.10

D.  ip route 10.73.65.66 0.0.0.255 10.80.65.10

Answer: B

──────────────────────────────────────────────────

## Question 762  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A packet sourced from 10.10.10.32 is destined for the Internet. What is the

administrative distance for the destination route?

A.  0

B.  1

C.  2

D.  32

Answer: B

──────────────────────────────────────────────────

## Question 770  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which IP route command created the best path for a packet destined for 10.10.10.3?

A.  ip route 10.10.0.0 255.255.252.0 g0/0

B.  ip route 10.0.0.0 255.0.0.0 g0/0

C.  ip route 10.10.10.1 255.255.255.255 g0/0

D.  ip route 10.10.10.0 255.255.255.240 g0/0

Answer: D

──────────────────────────────────────────────────

## Question 772  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which two values does router R1 use to identify valid routes for the R3 loopback address 1.1.1.3/32?

(Choose two.)

A.  lowest cost to teach the next hop

B.  highest metric

C.  highest administrative distance

D.  lowest metric

E.  lowest administrative distance

Answer: D,E

──────────────────────────────────────────────────

## Question 775  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer executes the show ip route command on router D. What is the next hop to network

192.168 1 0/24 and why?

A.  The next hop is 10.0.2.1 because it uses distance vector routing

B.  The next hop is 10.0.2.1 because it is a link-state routing protocol

C.  The next hop is 10.0.0.1 because it has a better administrative distance

D.  The next hop is 10.0.0.1 because it has a higher metric.

Answer: B

──────────────────────────────────────────────────

## Question 780  (Topic 4)  —  Multiple Choice  [V80.2]

R1 has learned route 192.168.12.0/24 via IS-IS, OSPF, RIP, and Internal EIGRP. Under normal operating

conditions, which routing protocol's route will be installed in the routing table?

A.  IS-IS

B.  RIP

C.  OSPF

D.  Internal EIGRP

Answer: D

──────────────────────────────────────────────────

## Question 820  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What does route 10.0.1.3/32 represent in the routing table?

A.  the 10.0.0.0 network

B.  a single destination address

C.  the source 10.0.1.100

D.  all hosts in the 10.0.1.0 subnet

Answer: A

──────────────────────────────────────────────────

## Question 828  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Routers R1 and R2 are configured with RIP as the dynamic routing protocol. A network engineer must

configure R1 with a floating static route to serve as a backup route to network 192.168.23. Which command

must the engineer configure on R1?

A.  ip route 192.168.23.0 255.255.255.0 192.168.13.3 100

B.  ip route 192.168.23.0 255.255.255.0 192.168.13.3 121

C.  ip route 192.168.23.0 255.255.255.255 192.168.13.3 121

D.  ip route 192.168.23.0 255.255.255.0 192.168.13.3

Answer: B

──────────────────────────────────────────────────

## Question 833  (Topic 4)  —  Multiple Choice  [V80.2]

Which Windows command is used instead of the route print command to display the contents of the IP

routing table?

A.  netstat-n

B.  ipconfig

C.  ifconfig

D.  netstat-r

Answer: D

──────────────────────────────────────────────────

## Question 834  (Topic 4)  —  Multiple Choice  [V80.2]

Which type of address is shared by routers in a HSRP implementation and used by hosts on the subnet as

their default gateway address?

A.  multicast address

B.  loopback IP address

C.  virtual IP address

D.  broadcast address

Answer: C

──────────────────────────────────────────────────

## Question 836  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The administrator must configure a floating sialic default route that points to 2001:db8:1234:2::1 and

replaces the current default route only if it fails. Which command must the engineer configure on the CPE?

A.  ipv6 route ::/0 2001:db8:1234:2::1 3

B.  ipv6 route ::/128 2001 :db8:1234:2::1 3

C.  ipv6 route ::/0 2001:db8:1234:2::1 1

D.  ipv6 route ::/0 2001:db8:1234:2::1 2

Answer: B

──────────────────────────────────────────────────

## Question 843  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R1 is added to the network and configured with the 10.0.0.64/26 and 10.0.20.0/24 subnets.

However, traffic destined for the LAN on R3 is not accessible. Which command when executed on R1

defines a static route to reach the R3 LAN?

A.  ip route 10.0.15.0 255.255.255.0 10.0.20.3

B.  ip route 10.0.15.0 255.255.255.0 10.0.20.1

C.  ip route 10.0.0.64 255.255.255.192 10.0.20.3

D.  ip route 10.0.15.0 255.255.255.192 10.0.20.1

Answer: A

──────────────────────────────────────────────────

## Question 852  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is checking the routing table in the main router to identify the path to a server on the network.

Which route does the router use to reach the server at 192.168.2.2?

A.  S 192.168.0.0/20 [1/0] via 10.1.1.1

B.  S 192.168.2.0/29 [1/0] via 10.1.1.1

C.  S 192.168.2.0/28 [1/0] via 10.1.1.1

D.  S 192.168.1.0/30 [1/0] via 10.1.1.1

Answer: B

──────────────────────────────────────────────────

## Question 857  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The image server and client A are running an application that transfers an extremely high volume of data

between the two. An engineer is configuring a dedicated circuit between R1 and R2. Which set of

commands must the engineer apply to the routers so that only traffic between the image server and client A

is forces to use the new circuit?

A.  R1(config)#ip route 10.10.13.10 255.255.255.255 10.10.10.6R2(config)#ip route 192.168.0.100

255.255.255.255 10.10.10.5

B.  R1(config)#ip route 10.10.13.10 255.255.255.128 10.10.10.6R2(config)#lp route 192.168.0.100

255.255.255.0 10.10.10.5

C.  R1(config)#ip route 10.10.13.10 255.255.255.252 10.10.10.6R2(config)#tp route 192.168.0.100

255.255.255.252 10.10.10.5

D.  R1(config)#ip route 10.10.13.10 255.255.255.255 10.10.10.2R2(config)#ip route 192.168.0.100

255.255.255.255 10.10.10.1

Answer: D

──────────────────────────────────────────────────

## Question 859  (Topic 4)  —  Multiple Choice  [V80.2]

Why is a first-hop redundancy protocol implemented?

A.  to protect against default gateway failures

B.  to prevent loops in a network

C.  to enable multiple switches to operate as a single unit

D.  to provide load-sharing for a multilink segment

Answer: A

──────────────────────────────────────────────────

## Question 865  (Topic 4)  —  Multiple Choice  [V80.2]

What are two purposes of HSRP? (Choose two.)

A.  It groups two or more routers to operate as one virtual router.

B.  It improves network availability by providing redundant gateways.

C.  It passes configuration information to hosts in a TCP/IP network.

D.  It helps hosts on the network to reach remote subnets without a default gateway.

E.  It provides a mechanism for diskless clients to autoconfigure their IP parameters during boot.

Answer: A,B

──────────────────────────────────────────────────

## Question 874  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. The router R1 is in the process of being configured. Routers R2 and R3 are configured

correctly for the new environment. Which two commands must be configured on R1 for PC1 to

communicate to all PCs on the 10.10.10.0/24 network? (Choose two.)

A.  ip route 10.10.10.0 255.255.255.0 192.168.2.3

B.  ip route 10.10.10.10 255.255.255.255 192.168.2.2

C.  ip route 10.10.10.10 255.255.255.255 g0/1

D.  ip route 10.10.10.8 255.255.255.248 g0/1

E.  ip route 10.10.10.0 255.255.255.248 192.168.2.2

Answer: A,E

──────────────────────────────────────────────────

## Question 893  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer must configure router R1 with a host route to the server. Which command must the

engineer configure?

A.  R1(conftg)#lp route 10.10.10.0 255.255.255.0 192.168.0.2

B.  R1(Config)#lp route 10.10.10.10 265.255.255.255 192 168.0.2

C.  R1(config)#ip route 192.168.0.2 255.255.255.255 10.10.10.10

D.  R1(config)3|p route 0.0.0.0 0.0 0.0 192 168.0.2

Answer: B

──────────────────────────────────────────────────

## Question 900  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer must configure a core router with a floating static default route to the backup router at

10.200.0.2. Which command meets the requirements?

A.  ip route 0.0.0.0 0.0.0.0 10.200.0.2 1

B.  Ip route 0.0.0.0 0.0.0.0 10.200.0.2 floating

C.  ip route 0.0.0.0 0.0.0.0 10.200.0.2

D.  Ip route 0.0.0.0 0.0.0.0 10.200.0.2 10

Answer: D

──────────────────────────────────────────────────

## Question 908  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer is configuring router R1 with an IPv6 static route for prefix 2019:C15C:0CAF:E001::/64. The

next hop must be 2019:C15C:0CAF:E002::1 The route must be reachable via the R1 Gigabit 0/0 interface.

Which command configures the designated route?

A.  R1(config)#ipv6 route 2019:C15C:0CAF:E001::/64 2019:C15C:0CAF:E002::1

B.  R1(config-if)#ipv6 route 2019:C15C:0CAF:E001::/64 2019:C15C:0CAF:E002::1

C.  R1(config-if)#ip route 2019:C15C:0CAF:E001::/64 GigabitEthernet0/0

D.  R1(config)#ip route 2019:C15C:0CAF:E001::/64 GigabitEthernet0/0

Answer: C

──────────────────────────────────────────────────

## Question 939  (Topic 4)  —  Multiple Choice  [V80.2]

A packet from a company s branch office is destined to host 172.31.0.1 at headquarters. The sending

router has three possible matches in its routing table for the packet prefixes:

* 172. 31.0 .0/16.72.31.0.0724. and 172.31 0 0/25.

How does the router handle the packet?

A.  It sends the traffic via prefix 172.31.0.0/16

B.  It sends the traffic via the default gateway 0.0.0.070.

C.  It sends the traffic via prefix 172.31.0.0/24

D.  It sends the traffic via prefix 172.31.0.0/25

Answer: D

──────────────────────────────────────────────────

## Question 948  (Topic 4)  —  Multiple Choice  [V80.2]

The clients and OHCP server reside on different subnets. Which command must be used to forward

requests and replies between clients on the 10.10.0.1/24 subnet and the DHCP server at 192.168.10.1?

A.  ip route 192.168.10.1

B.  ip default-gateway 192.168.10.1

C.  ip helper-address 192.168.10.1

D.  ip dhcp address 192.168.10.1

Answer: C

──────────────────────────────────────────────────

## Question 951  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Router R1 must be configured to reach the 10.0.3.0/24 network from the 10.0.1.0/24 segment.

Which command must be used to configure the route?

A.  ip route 10.0.3.0 0.255255.255 10.0.4.2

B.  route add 10.0.3.0 mask 255.255.255.0 10.0.4.3

C.  Ip route 10.0.3.0 255.255.255.0 10.0.4.3

D.  route add 10.0.3.0 0.255.255.255 10.0.4.2

Answer: C

──────────────────────────────────────────────────

## Question 966  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Traffic from R1 to the 10.10.2.0/24 subnet uses 192.168.1.2 as its next hop. A network

engineer wants to update the R1 configuration so that traffic with destination 10.10.2.1 passes through

router R3, and all other traffic to the 10.10.2.0/24 subnet passes through R2.

Which command must be used?

A.  ip route 10.10.2.0 255.255.255.0 192.168.1.4 100

B.  ip route 10.10.2.0 255.255.255.0 192.168.1.4 115

C.  ip route 10.10.2.1 255.255.255.255 192.168.1.4 100

D.  ip route 10.10.2.1 255.255.255.255 192.168.1.4 115

Answer: D

──────────────────────────────────────────────────

## Question 978  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What does route 10.0.1.3/32 represent in the routing table?

A.  a single destination address

B.  the source 10.0.1.100

C.  all hosts in the 10.0.1.0 subnet

D.  the 10.0.0.0 network

Answer: A

──────────────────────────────────────────────────

## Question 985  (Topic 4)  —  Multiple Choice  [V80.2]

A DHCP pool has been created with the name NOCC. The pool is using 192.168.20.0/24 and must use the

next to last usable IP address as the default gateway for the DHCP clients. What is the next step in the

process?

A.  default-router192.168.20.253

B.  network 192.168.20.254 255.255.255.0 secondary

C.  ip default-gateway 0.0.0.0 0.0.0.0 192.168.20.253

D.  next-server 192.168.20.254

Answer: A

──────────────────────────────────────────────────

## Question 1034  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which two values does router R1 use to determine the best path to reach destinations in network 1,0.0.0/8?

(Choose two.)

A.  longest prefix match

B.  highest administrative distance

C.  highest metric

D.  lowest metric

E.  lowest cost to reach the next hop

Answer: A,D

──────────────────────────────────────────────────

## Question 1054  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The New York router must be configured so that traffic to 2000::1 is sent primarily via the Atlanta site, with a

secondary path via Washington that has an administrative distance of 2. Which two commands must be

configured on the New York router? (Choose two.)

A.  ipv6 route 2000::1/128 2012::1

B.  ipv6 route 2000::1/128 2023::2 5

C.  ipv6 route 2000::1/128 2012::1 5

D.  ipv6 route 2000::1/128 2023::3 2

E.  ipv6 route 2000::1/128 2012::2

Answer: D,E

──────────────────────────────────────────────────

## Question 1055  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which command must be enable a floating default route on router A?

A.  ip route 0.0.0.0 0.0.0.0 192.168.1.2

B.  ip default-gateway 192.168.2.1

C.  ip route 0.0.0.0 0.0.0.0 192.168.1.2 10

D.  ip route 0.0.0.0 0.0.0.0 192.168.2.1 10

Answer: C

──────────────────────────────────────────────────

## Question 1082  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Routers R1 and R2 have been configured with their respective LAN interfaces. The two

circuits are operational and reachable across the WAN. Which command set establishes failover

redundancy if the primary circuit goes down?

A.  Ri(config)#ip route 0.0.0.0 0.0.0.0 10.10.10.62R2(config)#ip route 0.0.0.0 0.0.0.0 10.10.10.5 2

B.  Ri(config)#ip route 10.10.13.10 255.255.255.255 10.10.10.6R2(config)#ip route 192.168.0.100

255.255.255.255 10.10.10.5

C.  Ri(config)#ip route 0.0.0.0 0.0.0.0 10.10.10.6R2(config)#ip route 0.0.0.0 0.0.0.0 10.10.10.5

D.  Ri(config)#ip route 10.10.13.10 255.255.255.255 10.10.10.2R2(config)#ip route 192.168.0.100

255.255.255.255 10.10.10.1

Answer: A

──────────────────────────────────────────────────

## Question 1113  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. What is preventing host A from reaching the internet?

A.  The domain name server is unreachable.

B.  LAN and WAN network segments are different.

C.  IP address assignment is incorrect.

D.  The default gateway should be the first usable IP address.

Answer: C

──────────────────────────────────────────────────

## Question 1118  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. During initial configuration testing, the Windows workstation PC1 cannot connect with

the 172.16.2.0/24 network. Which set of actions corrects the configuration?

A.  Change the IP address to 172.16.1.6 and change the subnet mask to 255.255.255.248.

B.  Change the IP address to 172.16.1.9 and change the default gateway to 172.16.1.7.

C.  Change the IP address to 172.16.1.9 and change the DNS server to 172.16.1.12 only.

D.  Change the IP address to 172.16.1.6 and change the DNS servers to 172.18.1.12 and 172.16.1.13.

Answer: B

──────────────────────────────────────────────────

## Question 1134  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Which next hop is used to route packets to the application server at 152.168.32.85?

A.  10.10.1.2

B.  10.10.2.2

C.  10.10.3.2

D.  10.10.4.2

Answer: B

──────────────────────────────────────────────────

## Question 1150  (Topic 5)  —  Multiple Choice  [V80.2]

What is a reason to implement HSRP on a network?

A.  to Identify the closest hop to the default gateway In a LAN network

B.  to provide load balancing over multiple gateways in a LAN network

C.  to optimally route traffic based on the forwarding capacity of the edge routing devices in the LAN network

D.  to ensure that user traffic in a LAN rapidly recovers from the failure of an edge routing device

Answer: D

──────────────────────────────────────────────────

## Question 1152  (Topic 5)  —  Multiple Choice  [V80.2]

Which key function is provided by the data plane?

A.  Making routing decisions

B.  Originating packets

C.  Forwarding traffic to the next hop

D.  Exchanging routing table data

Answer: C

──────────────────────────────────────────────────

## Question 1155  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. What is the administrative distance for the advertised prefix that includes the host IP

address 192.168.20.1?

A.  1

B.  24

C.  192.168.10.2

D.  0

Answer: A

──────────────────────────────────────────────────

## Question 1158  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. What is the value of the administrative distance for the default gateway?

A.  10

B.  0

C.  1

D.  110

Answer: C

──────────────────────────────────────────────────

## Question 1159  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A static route must be configured on R14 to forward traffic for the 172.21.34.0/25

network that resides on R86. Which command must be used to fulfill the request?

A.  ip route 172.21.34.0 255.255.255.192 10.73.65.65

B.  ip route 172.21.34.0 255.255.255.0 10.73.65.65

C.  ip route 172.21.34.0 255.255.255.128 10.73.65.66

D.  ip route 172.21.34.0 255.255.128.0 10.73.65.64

Answer: C

──────────────────────────────────────────────────

## Question 1170  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Inter-VLAN routing is configured on SW1. Client A is running Linux as an OS in VLAN

10 with a default gateway IP 10.0.0.1 but cannot ping client B in VLAN 20 running Windows. What action

must be taken to verify that client A has the correct IP settings?

A.  Run the ipconfig command on client A and ensure that the IP address is within the host range of 10.0.0.1

- 10.0.255.254.

B.  Run the ifconfig command on client A to confirm that its IP and subnet mask fall within 255.254.0.0.

C.  Run the ipconfig command on client A to confirm that the correct 10.0.0.1 default gateway is used.

D.  Run the ifconfig command on client A to confirm that the subnet mask is set to 255.255.128.0.

Answer: B

──────────────────────────────────────────────────

## Question 1174  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Which route type is configured to reach the Internet?

A.  host route

B.  floating static route

C.  default route

D.  network route

Answer: C

──────────────────────────────────────────────────

## Question 1179  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. What is the administrative distance for the advertised prefix that includes the host IP

address 192.168.20.1?

A.  0

B.  192.168.10.2

C.  24

D.  1

Answer: D

──────────────────────────────────────────────────

## Question 1202  (Topic 5)  —  Multiple Choice  [V80.2]

A default route must be configured to temporarily use a local broadband modem with the IP address of

192.168.1.1 until a WAN circuit is installed. The WAN circuit will use an external routing protocol that

exchanges network prefixes between two separate autonomous systems on the Internet. Only a default

route will be received by the ISP Which configuration must be applied to allow the new WAN circuit to take

precedence when It Is In place?

A.  Ip route 0.0.0.0 0.0.0.0 192.168.1.1

B.  Ip route 0.0.0.0 0.0.0.0 192.168.1.1 25

C.  Ip route 0.0.0.0 0.0.0.0 192.168.1.1 track 1

D.  Ip route 0.0.0.0 0.0.0.0 192.168.1.1 20

Answer: B

──────────────────────────────────────────────────

## Question 1203  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. How will router R1 handle packets destined to 192.168.64.22?

A.  It will use the static route to 10.1.1.1.

B.  It will use the route with the highest AD and highest destination IP.

C.  It will route the packets to 10.1.1.2.

D.  It will drop the packets.

Answer: D

──────────────────────────────────────────────────

## Question 1206  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. During initial configuration testing, the Windows workstation PC1 cannot connect with

the 172.16.2.0/24 network.

Which set of actions corrects the configuration?

A.  Change the IP address to 172.16.1.6 and change the subnet mask to 255.255.255.248.

B.  Change the IP address to 172.16.1.6 and change the DNS servers to 172.16.1.12 and 172.16.1.13.

C.  Change the IP address to 172.16.1.9 and change the default gateway to 172.16.1.7.

D.  Change the IP address to 172.16.1.9 and change the DNS server to 172.16.1.12 only.

Answer: A

──────────────────────────────────────────────────

## Question 1216  (Topic 5)  —  Multiple Choice  [V80.2]

Which type of protocol is VRRP?

A.  uses dynamic IP address assignment

B.  allows two or more routers to act as a default gateway

C.  uses a destination IP address 224.0.0.102 for router-to-router communication

D.  uses Cisco-proprietary First Hop Redundancy Protocol

Answer: D

──────────────────────────────────────────────────

## Question 1227  (Topic 5)  —  Multiple Choice  [V80.2]

When deploying a new network that includes both Cisco and third-party network devices, which

redundancy protocol avoids the interruption of network traffic if the default gateway router fails?

A.  FHRP

B.  HSRP

C.  GLBP

D.  VRRP

Answer: D

──────────────────────────────────────────────────

## Question 1228  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A secondary route is required on router R1 to pass traffic to the LAN network on R2 if

the primary link fails. Which command must be entered to configure the router?

A.  ip route 10.0.2.0 255.255.255.240 10.0.0.6 91

B.  ip route 10.0.2.0 255.255.255.248 10.0.0.6 91

C.  ip route 10.0.2.0 255.255.255.248 null0 93

D.  ip route 10.0.2.0 255.255.255.240 10.0.0.7 92

Answer: B

──────────────────────────────────────────────────

## Question 1231  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A secondary route is required on router R1 to pass traffic to the LAN network on R2 if

the primary link fails. Which command must be entered to configure the router?

A.  ip route 10.0.2.0 255.255.255.240 10.0.0.7 92

B.  ip route 10.0.2.0 255.255.255.248 10.0.0.6 91

C.  ip route 10.0.2.0 256.255.255.240 10.0.0.6 91

D.  ip route 10.0.2.0 255.255.255.248 null0 93

Answer: B

──────────────────────────────────────────────────

## Question 1232  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. What is the administrative distance for the advertised prefix that includes the host IP

address 10.30.0.1?

A.  10.0.0.2

B.  110

C.  30

D.  2

Answer: B