# CCNA 200-301
TCP UDP and Transport

TCP, UDP, and transport layer concepts

25 Questions

Multiple Choice: 19 | Drag & Drop: 6

Sources: V7.1 (2) | V80.2 (23)

Topic 1: 7 | Topic 3: 1 | Topic 4: 12 | Topic 5: 5

## Question 368  (Topic 1)  —  Multiple Choice  [V7.1]

How do TCP and UDP differ in the way they provide reliability for

delivery of packets?

A.  TCP is a connectionless protocol that does not provide reliable

delivery of data, UDP is a connection-oriented protocol that uses

sequencing to provide reliable delivery.

B.  TCP does not guarantee delivery or error checking to ensure that

there is no corruption of data UDP provides message acknowledgement

and retransmits data if lost.

C.  TCP provides flow control to avoid overwhelming a receiver by

sending too many packets at once, UDP sends packets to the receiver

in a continuous stream without checking for sequencing

D.  TCP uses windowing to deliver packets reliably; UDP provides

reliable message transfer between hosts by establishing a three-way

handshake

Answer: C

──────────────────────────────────────────────────

## Question 476  (Topic 1)  —  Multiple Choice  [V7.1]

How do TCP and UDP differ in the way that they establish a connection

between two endpoints?

A.  TCP uses synchronization packets, and UDP uses acknowledgment

packets.

B.  UDP uses SYN, SYN ACK and FIN bits in the frame header while TCP

uses SYN, SYN ACK and ACK bits

C.  UDP provides reliable message transfer and TCP is a connectionless

protocol

D.  TCP uses the three-way handshake and UDP does not guarantee message

delivery

Answer: D

──────────────────────────────────────────────────

## Question 3  (Topic 1)  —  Multiple Choice  [V80.2]

What is the difference regarding reliability and communication type between TCP and UDP?

A.  TCP is reliable and is a connection-oriented protocol UDP is not reliable and is a connectionless protocol

B.  TCP is not reliable and is a connection-oriented protocol; UDP is reliable and is a connectionless

protocol

C.  TCP is not reliable and is a connectionless protocol; UDP is reliable and is a connection- oriented

protocol

D.  TCP is reliable and is a connectionless protocol; UDP is not reliable and is a connection- oriented

protocol

Answer: A

──────────────────────────────────────────────────

## Question 60  (Topic 1)  —  Multiple Choice  [V80.2]

How do TCP and UDP differ in the way they provide reliability for delivery of packets?

A.  TCP is a connectionless protocol that does not provide reliable delivery of data, UDP is a

connection-oriented protocol that uses sequencing to provide reliable delivery.

B.  TCP does not guarantee delivery or error checking to ensure that there is no corruption of data UDP

provides message acknowledgement and retransmits data if lost.

C.  TCP provides flow control to avoid overwhelming a receiver by sending too many packets at once, UDP

sends packets to the receiver in a continuous stream without checking for sequencing

D.  TCP uses windowing to deliver packets reliably; UDP provides reliable message transfer between hosts

by establishing a three-way handshake

Answer: C

──────────────────────────────────────────────────

## Question 92  (Topic 1)  —  Multiple Choice  [V80.2]

What is the difference in data transmission delivery and reliability between TCP and UDP?

A.  TCP transmits data at a higher rate and ensures packet delivery. UDP retransmits lost data to ensure

applications receive the data on the remote end.

B.  UDP sets up a connection between both devices before transmitting data. TCP uses the three-way

handshake to transmit data with a reliable connection.

C.  UDP is used for multicast and broadcast communication. TCP is used for unicast communication and

transmits data at a higher rate with error checking.

D.  TCP requires the connection to be established before transmitting data. UDP transmits data at a higher

rate without ensuring packet delivery.

Answer: D

──────────────────────────────────────────────────

## Question 104  (Topic 1)  —  Multiple Choice  [V80.2]

How do TCP and UDP differ in the way that they establish a connection between two endpoints?

A.  TCP uses synchronization packets, and UDP uses acknowledgment packets.

B.  UDP uses SYN, SYN ACK and FIN bits in the frame header while TCP uses SYN, SYN ACK and ACK

bits

C.  UDP provides reliable message transfer and TCP is a connectionless protocol

D.  TCP uses the three-way handshake and UDP does not guarantee message delivery

Answer: D

──────────────────────────────────────────────────

## Question 160  (Topic 1)  —  Multiple Choice  [V80.2]

How do TCP and UDP differ in the way they guarantee packet delivery?

A.  TCP uses checksum, acknowledgement, and retransmissions, and UDP uses checksums only.

B.  TCP uses two-dimensional parity checks, checksums, and cyclic redundancy checks and UDP uses

retransmissions only.

C.  TCP uses checksum, parity checks, and retransmissions, and UDP uses acknowledgements only.

D.  TCP uses retransmissions, acknowledgement and parity checks and UDP uses cyclic redundancy

checks only.

Answer: A

──────────────────────────────────────────────────

## Question 475  (Topic 3)  —  Drag & Drop  [V80.2]

Drag and drop the TCP or UDP details from the left onto their corresponding protocols on the right.

──────────────────────────────────────────────────

## Question 574  (Topic 4)  —  Multiple Choice  [V80.2]

Why is TCP desired over UDP for application that require extensive error checking, such as HTTPS?

A.  UDP operates without acknowledgments, and TCP sends an acknowledgment for every packet

received.

B.  UDP reliably guarantees delivery of all packets, and TCP drops packets under heavy load.

C.  UDP uses flow control mechanisms for the delivery of packets, and TCP uses congestion control for

efficient packet delivery.

D.  UDP uses sequencing data tor packets to arrive in order, and TCP offers trie capability

to receive packets in random order.

Answer: A

──────────────────────────────────────────────────

## Question 593  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the TCP and UDP characteristics from the lett onto the corresponding protocols on the right.

──────────────────────────────────────────────────

## Question 628  (Topic 4)  —  Multiple Choice  [V80.2]

Under which condition is TCP preferred over UDP?

A.  UDP is used when low latency is optimal, and TCP is used when latency is tolerable.

B.  TCP is used when dropped data is more acceptable, and UDP is used when data is accepted outof-order.

C.  TCP is used when data reliability is critical, and UDP is used when missing packets are acceptable.

D.  UDP is used when data is highly interactive, and TCP is used when data is time- sensitive.

Answer: C

──────────────────────────────────────────────────

## Question 634  (Topic 4)  —  Multiple Choice  [V80.2]

How do TCP and UDP fit into a query-response model?

A.  TCP establishes a connection prior to sending data, and UDP sends immediately.

B.  TCP uses error detection for packets, and UDP uses error recovery.

C.  TCP avoids using sequencing, and UDP avoids using acknowledgments.

D.  TCP encourages out-of-order packet delivery, and UDP prevents re-ordering.

Answer: A

──────────────────────────────────────────────────

## Question 652  (Topic 4)  —  Multiple Choice  [V80.2]

What is the role of SNMP in the network?

A.  to monitor network devices and functions using a TCP underlay that operates on the presentation layer

B.  to collect data directly from network devices using an SSL underlay that operates on the transport layer

C.  to monitor and manage network devices using a UDP underlay that operates on the application layer

D.  to collect telemetry and critical information from network devices using an SSH underlay that operates

on the network layer

Answer: C

──────────────────────────────────────────────────

## Question 752  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the TCP and UDP characteristics from the left onto the supporting protocols on the right. Not

all options are used.

──────────────────────────────────────────────────

## Question 766  (Topic 4)  —  Multiple Choice  [V80.2]

Why is UDP more suitable than TCP for applications that require low latency, such as VoIP?

A.  TCP uses congestion control for efficient packet delivery, and UDP uses flow control mechanisms for the

delivery of packets.

B.  TCP sends an acknowledgment for every packet that is received, and UDP operates without

acknowledgments.

C.  UDP reliably guarantees delivery of all packets, and TCP drops packets under heavy load.

D.  UDP uses sequencing data for packets to arrive in order, and TCP offers the capability to receive

packets in random order.

Answer: B

──────────────────────────────────────────────────

## Question 817  (Topic 4)  —  Multiple Choice  [V80.2]

Why is UDP more suitable than TCP tor applications that require low latency, such as VoIP?

A.  UDP reliably guarantees delivery of all packets and TCP drops packets under heavy load.

B.  TCP sends an acknowledgment for every packet that is received and UDP operates without

acknowledgments.

C.  UDP uses sequencing data for packets to arrive in order, and TCP offers the capability

to receive packets in random order.

D.  TCP uses congestion control for efficient packet delivery and UDP uses flow control mechanisms for the

delivery of packets.

Answer: B

──────────────────────────────────────────────────

## Question 861  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. This ACL is configured to allow client access only to HTTP, HTTPS, and DNS services

via UDP. The new administrator wants to add TCP access to the DNS service. Which configuration updates

the ACL efficiently?

A.  ip access-list extended Services35 permit tcp 10.0.0.0 0.255.255.255 host 198.51.100.11 eq domain

B.  no ip access-list extended Servicesip access-list extended Services30 permit tcp 10.0.0.0 0.255.255.255

host 198.51.100.11 eq domain

C.  ip access-list extended Servicespermit tcp 10.0.0.0 0.255.255.255 host 198.51.100.11 eq domain

D.  no ip access-list extended Servicesip access-list extended Servicespermit udp 10.0.0.0 0.255.255.255

any eq 53permit tcp 10.0.0.0 0.255.255.255 host 198.51.100.11 eq domaindeny ip any any log

Answer: A

──────────────────────────────────────────────────

## Question 984  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the TCP or UDP details from the left onto their corresponding protocols on the right.

──────────────────────────────────────────────────

## Question 1001  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the TCP or UDP details from the left onto their corresponding protocols on the right.

──────────────────────────────────────────────────

## Question 1014  (Topic 4)  —  Drag & Drop  [V80.2]

Drag and drop the characteristics of transport layer protocols from the left onto the corresponding protocols

on the right.

──────────────────────────────────────────────────

## Question 1087  (Topic 5)  —  Multiple Choice  [V80.2]

Which two transport layer protocols carry syslog messages? (Choose two.)

A.  UDP

B.  RTP

C.  TCP

D.  IP

E.  ARP

Answer: A,C

──────────────────────────────────────────────────

## Question 1122  (Topic 5)  —  Multiple Choice  [V80.2]

What is the difference between the TCP and UDP protocols?

A.  TCP ensures ordered, reliable data delivery, and UDP offers low latency and high throughput.

B.  TCP is used for transmitting data over the internet, and UDP is used for transmitting data over a local

network.

C.  TCP manages multicast and broadcast data transfers, and UDP only handles unicast communications.

D.  TCP is used to ensure data integrity in a file transfer, and UDP is used to broadcast a message to

multiple recipients.

Answer: A

──────────────────────────────────────────────────

## Question 1127  (Topic 5)  —  Multiple Choice  [V80.2]

Which default condition must be considered when an encrypted mobility tunnel is used between two Cisco

WLCs?

A.  TCP port 443 and UDP 21 are used.

B.  Control and data traffic encryption are enabled.

C.  The tunnel uses the IPsec protocol for encapsulation.

D.  The tunnel uses the EolP protocol to transmit data traffic.

Answer: A

──────────────────────────────────────────────────

## Question 1137  (Topic 5)  —  Multiple Choice  [V80.2]

What is a characteristic of a Layer 2 switch?

A.  Maintains stateful transaction information

B.  Responsible for sending data in a particular sequence

C.  Makes forwarding decisions based on MAC addresses

D.  Filters based on a transport layer protocol

Answer: C

──────────────────────────────────────────────────

## Question 1229  (Topic 5)  —  Multiple Choice  [V80.2]

What differentiates the TCP and UDP protocols?

A.  TCP tracks segments being transmitted or received by assigning segment numbers, and UDP adjusts

data flow according to network conditions.

B.  TCP establishes a connection with the device on the other end before transferring, and UDP transfers

without establishing a connection.

C.  TCP sends data at a constant rate with error checking on upper protocol layers, and UDP provides

error-checking and sequencing.

D.  TCP immediately transmits data without waiting for a handshake, and UDP awaits a response from the

receiver before sending additional data.

Answer: B