# CCNA 200-301
VLANs and Trunking

VLANs, trunking, VTP, and inter-VLAN routing

138 Questions

Multiple Choice: 137 | Drag & Drop: 1

Sources: V7.1 (23) | V80.2 (115)

Topic 1: 39 | Topic 2: 29 | Topic 3: 20 | Topic 4: 39 | Topic 5: 11

## Question 1  (Topic 3)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

The DHCP server and clients are connected to the same switch. What is

the next step to complete the DHCP configuration to allow clients on

VLAN 1 to receive addresses from the DHCP server?

A.  Configure the ip dhcp snooping trust command on the interlace that

is connected to the DHCP client.

B.  Configure the ip dhcp relay information option command on the

interface that is connected to the DHCP client.

C.  Configure the ip dhcp snooping trust command on the interface that

is connected to the DHCP server.

D.  Configure the Ip dhcp relay information option command on the

interface that is connected to the DHCP server.

Answer: C

──────────────────────────────────────────────────

## Question 5  (Topic 3)  —  Multiple Choice  [V7.1]

Which interface mode must be configured to connect the lightweight APs

in a centralized architecture?

A.  WLAN dynamic

B.  management

C.  trunk

D.  access

Answer: D

──────────────────────────────────────────────────

## Question 8  (Topic 3)  —  Multiple Choice  [V7.1]

###

A network engineer must configure two new subnets using the address

block 10 70 128 0/19 to meet these requirements:

-   The first subnet must support 24 hosts

-   The second subnet must support 472 hosts

-   Both subnets must use the longest subnet mask possible from the

address block Which two configurations must be used to configure the

new subnets and meet a

requirement to use the first available address in each subnet for the

router interfaces? (Choose two )

A.  interface vlan 1234ip address 10.70.159.1 255.255.254.0

B.  interface vlan 1148ip address 10.70.148.1 255.255.254.0

C.  interface vlan 4722ip address 10.70.133.17 255.255.255.192

D.  interface vlan 3002ip address 10.70.147.17 255.255.255.224

E.  interface vlan 155ip address 10.70.155.65 255.255.255.224

Answer: BD

──────────────────────────────────────────────────

## Question 26  (Topic 3)  —  Multiple Choice  [V7.1]

How does Rapid PVST+ create a fast loop-free network topology?

A.  lt requires multiple links between core switches

B.  It generates one spanning-tree instance for each VLAN

C.  It maps multiple VLANs into the same spanning-tree instance

D.  It uses multiple active paths between end stations.

Answer: A

──────────────────────────────────────────────────

## Question 37  (Topic 3)  —  Multiple Choice  [V7.1]

Which two network actions occur within the data plane? (Choose two.)

A.  Add or remove an 802.1Q trunking header.

B.  Make a configuration change from an incoming NETCONF RPC.

C.  Run routing protocols.

D.  Match the destination MAC address to the MAC address table.

E.  Reply to an incoming ICMP echo request.

Answer: BD

──────────────────────────────────────────────────

## Question 41  (Topic 3)  —  Multiple Choice  [V7.1]

A network engineer must implement an IPv6 configuration on the vlan

2000 interface to create a routable locally-unique unicast address

that is blocked from being advertised to the internet. Which

configuration must the engineer apply?

A.  interface vlan 2000ipv6 address ffc0:0000:aaaa::1234:2343/64

B.  interface vlan 2000Ipv6 address

fc00:0000:aaaa:a15d:1234:2343:8aca/64

C.  interface vlan 2000ipv6 address fe80;0000:aaaa::1234:2343/64

D.  interface vlan 2000ipv6 address fd00::1234:2343/64

Answer: B

──────────────────────────────────────────────────

## Question 125  (Topic 2)  —  Multiple Choice  [V7.1]

Which function is performed by DHCP snooping?

A.  propagates VLAN information between switches

B.  listens to multicast traffic for packet forwarding

C.  provides DDoS mitigation

D.  rate-limits certain traffic

Answer: A

──────────────────────────────────────────────────

## Question 136  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

An administrator is tasked with configuring a voice VLAN. What is the

expected outcome when a Cisco phone is connected to the

GigabitEfriemet3/1/4 port on a switch?

A.  The phone and a workstation that is connected to the phone do not

have VLAN connectivity

B.  The phone and a workstation that is connected to the phone send and

receive data in VLAN 50.

C.  The phone sends and receives data in VLAN 50, but a workstation

connected to the phone has no VLAN connectivity

D.  The phone sends and receives data in VLAN 50, but a workstation

connected to the phone sends and receives data in VLAN 1

Answer: D

──────────────────────────────────────────────────

## Question 192  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

The default-information originate command is configured under the R1

OSPF configuration After testing workstations on VLAN 20 at Site B

cannot reach a DNS server on the Internet Which action corrects the

configuration issue?

A.  Add the default-information originate command onR2

B.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.18 command on R1

C.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.2 command on R2

D.  Add the always keyword to the default-information originate command

on R1

Answer: B

──────────────────────────────────────────────────

## Question 252  (Topic 2)  —  Multiple Choice  [V7.1]

Refer to the exhibit.

[See Exhibit Image]

Only four switches are participating in the VLAN spanning-tree

process. Branch-1 priority 614440

Branch-2: priority 39082416

Branch-3: priority 0 Branch-4: root primary

Which switch becomes the permanent root bridge for VLAN 5?

A.  Branch-1

B.  Branch-2

C.  Branch-3

D.  Branch-4

Answer: C

Explanation:

Dynamic ARP inspection is an ingress security feature; it does not

perform any egress checking.

──────────────────────────────────────────────────

## Question 254  (Topic 2)  —  Multiple Choice  [V7.1]

[See Exhibit Image][See Exhibit Image]Refer to the exhibit.

A network administrator assumes a task to complete the connectivity

between PC A and the File Server. Switch A and Switch B have been

partially configured with VLAN 10, 11, 12, and 13. What is the next

step in the configuration?

A.  Add PC A to VLAN 10 and the File Server to VLAN 11 fa VLAN

segmentation

B.  Add VLAN 13 to the trunk links on Switch A and Switch B for VLAN

propagation

C.  Add a router on a stick between Switch A and Switch B allowing for

Inter-VLAN routing.

D.  Add PC A to the same subnet as the Fie Server allowing for

intra-VLAN communication.

Answer: B

──────────────────────────────────────────────────

## Question 265  (Topic 2)  —  Multiple Choice  [V7.1]

-   (Topic 2) Refer to Exhibit.

[See Exhibit Image]

How does SW2 interact with other switches in this VTP domain?

A.  It processes VTP updates from any VTP clients on the network on its

access ports.

B.  It receives updates from all VTP servers and forwards all locally

configured VLANs out all trunk ports

C.  It forwards only the VTP advertisements that it receives on its

trunk ports.

D.  It transmits and processes VTP updates from any VTP Clients on the

network on its trunk ports

Answer: C

Explanation:

Reference:

[https://www.cisco.com/c/en/us/support/docs/lan-switching/vtp/10558-21.html](http://www.cisco.com/c/en/us/support/docs/lan-switching/vtp/10558-21.html)

The VTP mode of SW2 is transparent so it only forwards the VTP updates

it receives to its trunk links without processing them.

──────────────────────────────────────────────────

## Question 320  (Topic 1)  —  Multiple Choice  [V7.1]

When using Rapid PVST+, which command guarantees the switch is always

the root bridge for VLAN 200?

A.  spanning -tree vlan 200 priority 614440

B.  spanning -tree vlan 200 priority 38572422

C.  spanning -tree vlan 200 priority 0

D.  spanning -tree vlan 200 root primary

Answer: C

──────────────────────────────────────────────────

## Question 333  (Topic 1)  —  Multiple Choice  [V7.1]

Two switches are connected and using Cisco Dynamic Trunking Protocol

SW1 is set to Dynamic Desirable What is the result of this

configuration?

A.  The link is in a down state.

B.  The link is in an error disables state

C.  The link is becomes an access port.

D.  The link becomes a trunk port.

Answer: D

──────────────────────────────────────────────────

## Question 344  (Topic 1)  —  Drag & Drop  [V7.1]

DRAG DROP - (Topic 1)

[See Exhibit Image]Drag and drop the threat-mitigation techniques from

the left onto the types of threat or attack they mitigate on the

right.

A.  Mastered

B.  Not Mastered

Answer: A

Explanation:

[See Exhibit Image]Double-Tagging attack:

In this attack, the attacking computer generates frames with two

802.1Q tags. The first tag matches the native VLAN of the trunk port

(VLAN 10 in this case), and the second matches the VLAN of a host it

wants to attack (VLAN 20).When the packet from the attacker reaches

Switch A, Switch A only sees the first VLAN 10 and it matches with its

native VLAN 10 so this VLAN tag is removed. Switch A forwards the

frame out all links with the same native VLAN 10. Switch B receives

the frame with an tag of VLAN 20 so it removes this tag and forwards

out to the Victim computer.Note: This attack only works if the trunk

(between two switches) has the same native VLAN as the attacker.To

mitigate this type of attack, you can use VLAN access control lists

(VACLs, which applies to all traffic within a VLAN. We can use VACL to

drop attacker traffic to specific victims/servers) or implement

Private VLANs.ARP attack (like ARP poisoning/spoofing) is a type of

attack in which a malicious actor sends falsified ARP messages over a

local area network as ARP allows a gratuitous reply from a host even

if an ARP request was not received. This results in the linking of an

attacker's MAC address with the IP address of a legitimate computer or

server on the network. This is an attack based on ARP which is at

Layer 2.Dynamic ARP inspection (DAI) is a security feature that

validates ARP packets in a network which can be used to mitigate this

type of attack.

──────────────────────────────────────────────────

## Question 353  (Topic 1)  —  Multiple Choice  [V7.1]

What are network endpoints?

A.  act as routers to connect a user to the service prowler network

B.  a threat to the network if they are compromised

C.  support inter-VLAN connectivity

D.  enforce policies for campus-wide traffic going to the internet

Answer: B

──────────────────────────────────────────────────

## Question 383  (Topic 1)  —  Multiple Choice  [V7.1]

How does HSRP provide first hop redundancy?

A.  It load-balances traffic by assigning the same metric value to more

than one route to the same destination m the IP routing table.

B.  It load-balances Layer 2 traffic along the path by flooding traffic

out all interfaces configured with the same VLAN.

C.  It forwards multiple packets to the same destination over different

routed links n the data path

D.  It uses a shared virtual MAC and a virtual IP address to a group of

routers that serve as the default gateway for hosts on a LAN

Answer: D

Explanation:

[https://www.cisco.com/c/en/us/td/docs/ios-xml/ios/ipapp_fhrp/configuration/xe-16/fhp-xe-16-book/fhp-hsrp-mgo.html](http://www.cisco.com/c/en/us/td/docs/ios-xml/ios/ipapp_fhrp/configuration/xe-16/fhp-xe-16-book/fhp-hsrp-mgo.html)

──────────────────────────────────────────────────

## Question 392  (Topic 1)  —  Multiple Choice  [V7.1]

Which command on a port enters the forwarding state immediately when a

PC is connected to it?

A.  switch(config)#spanning-tree portfast default

B.  switch(config)#spanning-tree portfast bpduguard default

C.  switch(config-if)#spanning-tree portfast trunk

D.  switch(config-if)#no spanning-tree portfast

Answer: C

──────────────────────────────────────────────────

## Question 456  (Topic 1)  —  Multiple Choice  [V7.1]

An engineer is asked to protect unused ports that are configured in

the default VLAN on a switch. Which two steps will fulfill the

request? (Choose two)

A.  Configure the ports in an EtherChannel.

B.  Administratively shut down the ports

C.  Configure the port type as access and place in VLAN 99

D.  Configure the ports as trunk ports

E.  Enable the Cisco Discovery Protocol

Answer: BC

──────────────────────────────────────────────────

## Question 489  (Topic 1)  —  Multiple Choice  [V7.1]

[See Exhibit Image]Refer to the exhibit.

A network engineer must configured communication between PC A and the

File Server. To prevent interruption for any other communications,

which command must be configured?

A.  Switch trunk allowed vlan 12

B.  Switchport trunk allowed vlan none

C.  Switchport trunk allowed vlan add 13

D.  Switchport trunk allowed vlan remove 10-11

Answer: C

──────────────────────────────────────────────────

## Question 516  (Topic 1)  —  Multiple Choice  [V7.1]

Which configuration ensures that the switch is always the root for

VLAN 750?

A.  Switch(config)#spanning-tree vlan 750 priority 38003685

B.  Switch(config)#spanning-tree vlan 750 root primary

C.  Switch(config)#spanning-tree vlan 750 priority 614440

D.  Switch(config)#spanning-tree vlan 750 priority 0

Answer: D

Explanation:

Although the spanning-tree vlan 10 root primary command will ensure a

switch will have a bridge priority value lower than other bridges

introduced to the network, the spanning-tree vlan 10 priority 0

command ensures the bridge priority takes precedence over all other

priorities.

──────────────────────────────────────────────────

## Question 592  (Topic 1)  —  Multiple Choice  [V7.1]

Which command entered on a switch configured with Rapid PVST\* listens

and learns for a specific time period?

A.  switch(config)#spanning-tree vlan 1 max-age 6

B.  switch(config)#spanning-tree vlan 1 hello-time 10

C.  switch(config)#spanning-tree vlan 1 priority 4096

D.  switch(config)#spanning-tree vlan 1 forward-time 20

Answer: D

Explanation:

Forward time : Determines how long each of the listening and learning

states last before the port begins forwarding.

Switch(config)# \[ no \] spanning-tree vlan vlan_ID forward-time

forward_timeConfigures the forward time of a VLAN. The forward_time

value can be from 4 to 30 seconds.

[https://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst4500/12-2/15-02SG/configuration/guide/config/spantree.html#56177](http://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst4500/12-2/15-02SG/configuration/guide/config/spantree.html#56177)

──────────────────────────────────────────────────

## Question 594  (Topic 1)  —  Multiple Choice  [V7.1]

Where does the configuration reside when a helper address Is

configured lo support DHCP?

A.  on the router closest to the server

B.  on the router closest to the client

C.  on every router along the path

D.  on the switch trunk interface

Answer: B

──────────────────────────────────────────────────

## Question 1  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The network administrator wants VLAN 67 traffic to be untagged between Switch 1 and Switch 2 while all

other VLANs are to remain tagged.

Which command accomplishes this task?

A.  switchport access vlan 67

B.  switchport trunk allowed vlan 67

C.  switchport private-vlan association host 67

D.  switchport trunk native vlan 67

Answer: D

──────────────────────────────────────────────────

## Question 7  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the Exhibit.

After the switch configuration the ping test fails between PC A and PC B Based on the output for switch 1.

which error must be corrected?

A.  There is a native VLAN mismatch

B.  Access mode is configured on the switch ports.

C.  The PCs are m the incorrect VLAN

D.  All VLANs are not enabled on the trunk

Answer: A

──────────────────────────────────────────────────

## Question 12  (Topic 1)  —  Multiple Choice  [V80.2]

Which command on a port enters the forwarding state immediately when a PC is connected to it?

A.  switch(config)#spanning-tree portfast default

B.  switch(config)#spanning-tree portfast bpduguard default

C.  switch(config-if)#spanning-tree portfast trunk

D.  switch(config-if)#no spanning-tree portfast

Answer: C

──────────────────────────────────────────────────

## Question 23  (Topic 1)  —  Multiple Choice  [V80.2]

What is a practice that protects a network from VLAN hopping attacks?

A.  Enable dynamic ARP inspection

B.  Configure an ACL to prevent traffic from changing VLANs

C.  Change native VLAN to an unused VLAN ID

D.  Implement port security on internet-facing VLANs

Answer: C

──────────────────────────────────────────────────

## Question 24  (Topic 1)  —  Multiple Choice  [V80.2]

Which access layer threat-mitigation technique provides security based on identity?

A.  Dynamic ARP Inspection

B.  using a non-default native VLAN

C.  802.1x

D.  DHCP snooping

Answer: C

──────────────────────────────────────────────────

## Question 41  (Topic 1)  —  Multiple Choice  [V80.2]

An engineer must configure Interswitch VLAN communication between a Cisco switch and a third-party

switch. Which action should be taken?

A.  configure IEEE 802.1p

B.  configure IEEE 802.1q

C.  configure ISL

D.  configure DSCP

Answer: B

──────────────────────────────────────────────────

## Question 46  (Topic 1)  —  Multiple Choice  [V80.2]

What occurs to frames during the process of frame flooding?

A.  Frames are sent to every port on the switch in the same VLAN except from the originating port

B.  Frames are sent to every port on the switch that has a matching entry in the MAC address table.

C.  Frames are sent to all ports, including those that are assigned to other VLANs.

D.  Frames are sent to every port on the switch in the same VLAN.

Answer: A

──────────────────────────────────────────────────

## Question 58  (Topic 1)  —  Multiple Choice  [V80.2]

How are VLAN hopping attacks mitigated?

A.  enable dynamic ARP inspection

B.  manually implement trunk ports and disable DTP

C.  activate all ports and place in the default VLAN

D.  configure extended VLANs

Answer: B

──────────────────────────────────────────────────

## Question 61  (Topic 1)  —  Multiple Choice  [V80.2]

What does a switch use to build its MAC address table?

A.  VTP

B.  DTP

C.  egress traffic

D.  ingress traffic

Answer: D

──────────────────────────────────────────────────

## Question 62  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which switch becomes the root of the spanning tree for VLAN 110?

A.  Switch 1

B.  Switch 2

C.  Switch 3

D.  Switch 4

Answer: B

──────────────────────────────────────────────────

## Question 81  (Topic 1)  —  Multiple Choice  [V80.2]

What are network endpoints?

A.  act as routers to connect a user to the service prowler network

B.  a threat to the network if they are compromised

C.  support inter-VLAN connectivity

D.  enforce policies for campus-wide traffic going to the internet

Answer: B

──────────────────────────────────────────────────

## Question 94  (Topic 1)  —  Multiple Choice  [V80.2]

When using Rapid PVST+, which command guarantees the switch is always the root bridge for VLAN 200?

A.  spanning -tree vlan 200 priority 614440

B.  spanning -tree vlan 200 priority 38572422

C.  spanning -tree vlan 200 priority 0

D.  spanning -tree vlan 200 root primary

Answer: C

──────────────────────────────────────────────────

## Question 97  (Topic 1)  —  Multiple Choice  [V80.2]

After installing a new Cisco ISE server, which task must the engineer perform on the Cisco WLC to connect

wireless clients on a specific VLAN based on their credentials?

A.  Enable the allow AAA Override

B.  Enable the Even: Driven RRM.

C.  Disable the LAG Mode or Next Reboot.

D.  Enable the Authorized MIC APs against auth-list or AAA.

Answer: A

──────────────────────────────────────────────────

## Question 100  (Topic 1)  —  Multiple Choice  [V80.2]

Two switches are connected and using Cisco Dynamic Trunking Protocol SW1 is set to Dynamic Desirable

What is the result of this configuration?

A.  The link is in a down state.

B.  The link is in an error disables state

C.  The link is becomes an access port.

D.  The link becomes a trunk port.

Answer: D

──────────────────────────────────────────────────

## Question 108  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

What commands are needed to add a subinterface to Ethernet0/0 on R1 to allow for VLAN 20, with IP

address 10.20.20.1/24?

A.  R1(config)#interface ethernet0/0R1(config)#encapsulation dot1q 20R1(config)#ip address 10.20.20.1

255.255.255.0

B.  R1(config)#interface ethernet0/0.20R1(config)#encapsulation dot1q 20R1(config)#ip address 10.20.20.1

255.255.255.0

C.  R1(config)#interface ethernet0/0.20R1(config)#ip address 10.20.20.1 255.255.255.0

D.  R1(config)#interface ethernet0/0R1(config)#ip address 10.20.20.1 255.255.255.0

Answer: B

──────────────────────────────────────────────────

## Question 115  (Topic 1)  —  Multiple Choice  [V80.2]

An engineer is asked to protect unused ports that are configured in the default VLAN on a switch.

Which two steps will fulfill the request? (Choose two)

A.  Configure the ports in an EtherChannel.

B.  Administratively shut down the ports

C.  Configure the port type as access and place in VLAN 99

D.  Configure the ports as trunk ports

E.  Enable the Cisco Discovery Protocol

Answer: B,C

──────────────────────────────────────────────────

## Question 122  (Topic 1)  —  Multiple Choice  [V80.2]

What is the default behavior of a Layer 2 switch when a frame with an unknown destination MAC address is

received?

A.  The Layer 2 switch drops the received frame

B.  The Layer 2 switch floods packets to all ports except the receiving port in the given VLAN.

C.  The Layer 2 switch sends a copy of a packet to CPU for destination MAC address learning.

D.  The Layer 2 switch forwards the packet and adds the destination MAC address to its MAC address table

Answer: B

──────────────────────────────────────────────────

## Question 129  (Topic 1)  —  Multiple Choice  [V80.2]

What are two functions of a Layer 2 switch? (Choose two)

A.  acts as a central point for association and authentication servers

B.  selects the best route between networks on a WAN

C.  moves packets within a VLAN

D.  moves packets between different VLANs

E.  makes forwarding decisions based on the MAC address of a packet

Answer: A,E

──────────────────────────────────────────────────

## Question 136  (Topic 1)  —  Multiple Choice  [V80.2]

Which two command sequences must you configure on switch to establish a Layer 3 EtherChannel with an

open-standard protocol? (Choose two )

A.  interface GigabitEthernet0/0/1channel-group 10 mode on

B.  interface GigabitEthernet0/0/1channel-group 10 mode active

C.  interface GigabitEthernet0/0/1channel-group 10 mode auto

D.  interface port-channel 10switchportswitchport mode trunk

E.  interface port-channel 10no switchportip address 172.16.0.1.255.255.255.0

Answer: B,E

──────────────────────────────────────────────────

## Question 140  (Topic 1)  —  Multiple Choice  [V80.2]

An engineer needs to add an old switch back into a network. To prevent the switch from

corrupting the VLAN database which action must be taken?

A.  Add the switch in the VTP domain with a lower revision number

B.  Add the switch with DTP set to dynamic desirable

C.  Add the switch in the VTP domain with a higher revision number

D.  Add the switch with DTP set to desirable

Answer: A

──────────────────────────────────────────────────

## Question 155  (Topic 1)  —  Multiple Choice  [V80.2]

Which command entered on a switch configured with Rapid PVST* listens and learns for a specific time

period?

A.  switch(config)#spanning-tree vlan 1 max-age 6

B.  switch(config)#spanning-tree vlan 1 hello-time 10

C.  switch(config)#spanning-tree vlan 1 priority 4096

D.  switch(config)#spanning-tree vlan 1 forward-time 20

Answer: D

──────────────────────────────────────────────────

## Question 162  (Topic 1)  —  Multiple Choice  [V80.2]

How does HSRP provide first hop redundancy?

A.  It load-balances traffic by assigning the same metric value to more than one route to the same

destination m the IP routing table.

B.  It load-balances Layer 2 traffic along the path by flooding traffic out all interfaces configured with the

same VLAN.

C.  It forwards multiple packets to the same destination over different routed links n the data path

D.  It uses a shared virtual MAC and a virtual IP address to a group of routers that serve as the default

gateway for hosts on a LAN

Answer: D

──────────────────────────────────────────────────

## Question 170  (Topic 1)  —  Multiple Choice  [V80.2]

Which two components are needed to create an Ansible script that configures a VLAN on a switch?

(Choose two.)

A.  cookbook

B.  task

C.  playbook

D.  model

E.  recipe

Answer: C,D

──────────────────────────────────────────────────

## Question 172  (Topic 1)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer must configured communication between PC A and the File Server. To prevent

interruption for any other communications, which command must be configured?

A.  Switch trunk allowed vlan 12

B.  Switchport trunk allowed vlan none

C.  Switchport trunk allowed vlan add 13

D.  Switchport trunk allowed vlan remove 10-11

Answer: C

──────────────────────────────────────────────────

## Question 181  (Topic 1)  —  Multiple Choice  [V80.2]

Which configuration ensures that the switch is always the root for VLAN 750?

A.  Switch(config)#spanning-tree vlan 750 priority 38003685

B.  Switch(config)#spanning-tree vlan 750 root primary

C.  Switch(config)#spanning-tree vlan 750 priority 614440

D.  Switch(config)#spanning-tree vlan 750 priority 0

Answer: D

──────────────────────────────────────────────────

## Question 202  (Topic 1)  —  Multiple Choice  [V80.2]

A network administrator is asked to configure VLANS 2, 3 and 4 for a new implementation. Some ports

must be assigned to the new VLANS with unused remaining. Which action should be taken for the unused

ports?

A.  configure port in the native VLAN

B.  configure ports in a black hole VLAN

C.  configure in a nondefault native VLAN

D.  configure ports as access ports

Answer: B

──────────────────────────────────────────────────

## Question 204  (Topic 1)  —  Multiple Choice  [V80.2]

How is the native VLAN secured in a network?

A.  separate from other VLANs within the administrative domain

B.  give it a value in the private VLAN range

C.  assign it as VLAN 1

D.  configure it as a different VLAN ID on each end of the link

Answer: A

──────────────────────────────────────────────────

## Question 230  (Topic 1)  —  Multiple Choice  [V80.2]

Where does the configuration reside when a helper address Is configured lo support DHCP?

A.  on the router closest to the server

B.  on the router closest to the client

C.  on every router along the path

D.  on the switch trunk interface

Answer: B

──────────────────────────────────────────────────

## Question 242  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network administrator assumes a task to complete the connectivity between PC A and the File Server.

Switch A and Switch B have been partially configured with VLAN 10, 11, 12, and 13. What is the next step

in the configuration?

A.  Add PC A to VLAN 10 and the File Server to VLAN 11 fa VLAN segmentation

B.  Add VLAN 13 to the trunk links on Switch A and Switch B for VLAN propagation

C.  Add a router on a stick between Switch A and Switch B allowing for Inter-VLAN routing.

D.  Add PC A to the same subnet as the Fie Server allowing for intra-VLAN communication.

Answer: B

──────────────────────────────────────────────────

## Question 255  (Topic 2)  —  Multiple Choice  [V80.2]

What is a function of a Layer 3 switch?

A.  move frames between endpoints limited to IP addresses

B.  transmit broadcast traffic when operating in Layer 3 mode exclusively

C.  forward Ethernet frames between VLANs using only MAC addresses

D.  flood broadcast traffic within a VLAN

Answer: A

──────────────────────────────────────────────────

## Question 260  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Only four switches are participating in the VLAN spanning-tree process.

Branch-1 priority 614440

Branch-2: priority 39082416

Branch-3: priority 0

Branch-4: root primary

Which switch becomes the permanent root bridge for VLAN 5?

A.  Branch-1

B.  Branch-2

C.  Branch-3

D.  Branch-4

Answer: C

──────────────────────────────────────────────────

## Question 265  (Topic 2)  —  Multiple Choice  [V80.2]

A Cisco IP phone receive untagged data traffic from an attached PC. Which action is taken by the phone?

A.  It allows the traffic to pass through unchanged

B.  It drops the traffic

C.  It tags the traffic with the default VLAN

D.  It tags the traffic with the native VLAN

Answer: A

──────────────────────────────────────────────────

## Question 275  (Topic 2)  —  Multiple Choice  [V80.2]

Which result occurs when PortFast is enabled on an interface that is connected to another switch?

A.  Spanning tree may fail to detect a switching loop in the network that causes broadcast storms

B.  VTP is allowed to propagate VLAN configuration information from switch to switch automatically.

C.  Root port choice and spanning tree recalculation are accelerated when a switch link goes down

D.  After spanning tree converges PortFast shuts down any port that receives BPDUs.

Answer: A

──────────────────────────────────────────────────

## Question 289  (Topic 2)  —  Multiple Choice  [V80.2]

Which function is performed by DHCP snooping?

A.  propagates VLAN information between switches

B.  listens to multicast traffic for packet forwarding

C.  provides DDoS mitigation

D.  rate-limits certain traffic

Answer: A

──────────────────────────────────────────────────

## Question 297  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to Exhibit.

Which configuration must be applied to the router that configures PAT to translate all addresses in VLAN

200 while allowing devices on VLAN 100 to use their own IP addresses?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 323  (Topic 2)  —  Multiple Choice  [V80.2]

What prevents a workstation from receiving a DHCP address?

A.  DTP

B.  STP

C.  VTP

D.  802.10

Answer: B

──────────────────────────────────────────────────

## Question 326  (Topic 2)  —  Multiple Choice  [V80.2]

Which port type supports the spanning-tree portfast command without additional configuration?

A.  access ports

B.  Layer 3 main Interfaces

C.  Layer 3 suninterfaces

D.  trunk ports

Answer: A

──────────────────────────────────────────────────

## Question 328  (Topic 2)  —  Multiple Choice  [V80.2]

An engineer must configure traffic for a VLAN that is untagged by the switch as it crosses a trunk link.

Which command should be used?

A.  switchport trunk allowed vlan 10

B.  switchport trunk native vlan 10

C.  switchport mode trunk

D.  switchport trunk encapsulation dot1q

Answer: B

──────────────────────────────────────────────────

## Question 338  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which action is expected from SW1 when the untagged frame is received on the GigabitEthernet0/1

interface?

A.  The frame is processed in VLAN 5.

B.  The frame is processed in VLAN 11

C.  The frame is processed in VLAN 1

D.  The frame is dropped

Answer: A

──────────────────────────────────────────────────

## Question 346  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer must block access for all computers on VLAN 20 to the web server via HTTP All other

computers must be able to access the web server Which configuration when applied to switch A

accomplishes this task?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 348  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An administrator must configure interfaces Gi1/1 and Gi1/3 on switch SW11 PC-1 and PC- 2 must be

placed in the Data VLAN and Phone-1 must be placed in the Voice VLAN Which configuration meets these

requirements?

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 352  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An administrator is tasked with configuring a voice VLAN. What is the expected outcome when a Cisco

phone is connected to the GigabitEfriemet3/1/4 port on a switch?

A.  The phone and a workstation that is connected to the phone do not have VLAN connectivity

B.  The phone and a workstation that is connected to the phone send and receive data in VLAN 50.

C.  The phone sends and receives data in VLAN 50, but a workstation connected to the phone has no VLAN

connectivity

D.  The phone sends and receives data in VLAN 50, but a workstation connected to the phone sends and

receives data in VLAN 1

Answer: D

──────────────────────────────────────────────────

## Question 354  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The default-information originate command is configured under the R1 OSPF configuration After testing

workstations on VLAN 20 at Site B cannot reach a DNS server on the Internet Which action corrects the

configuration issue?

A.  Add the default-information originate command onR2

B.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.18 command on R1

C.  Configure the ip route 0.0.0.0 0.0.0.0 10.10.10.2 command on R2

D.  Add the always keyword to the default-information originate command on R1

Answer: B

──────────────────────────────────────────────────

## Question 356  (Topic 2)  —  Multiple Choice  [V80.2]

The service password-encryption command is entered on a router. What is the effect of this configuration?

A.  restricts unauthorized users from viewing clear-text passwords in the running configuration

B.  encrypts the password exchange when a VPN tunnel is established

C.  prevents network administrators from configuring clear-text passwords

D.  protects the VLAN database from unauthorized PC connections on the switch

Answer: A

──────────────────────────────────────────────────

## Question 364  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to Exhibit.

Which action do the switches take on the trunk link?

A.  The trunk does not form and the ports go into an err-disabled status.

B.  The trunk forms but the mismatched native VLANs are merged into a single broadcast domain.

C.  The trunk does not form, but VLAN 99 and VLAN 999 are allowed to traverse the link.

D.  The trunk forms but VLAN 99 and VLAN 999 are in a shutdown state.

Answer: B

──────────────────────────────────────────────────

## Question 366  (Topic 2)  —  Multiple Choice  [V80.2]

Which protocol requires authentication to transfer a backup configuration file from a router to a remote

server?

A.  DTP

B.  FTP

C.  SMTP

D.  TFTP

Answer: B

──────────────────────────────────────────────────

## Question 368  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which command must be executed for Gi1.1 on SW1 to become a trunk port if Gi1/1 on SW2 is configured

in desirable or trunk mode?

A.  switchport mode trunk

B.  switchport mode dot1-tunnel

C.  switchport mode dynamic auto

D.  switchport mode dynamic desirable

Answer: C

──────────────────────────────────────────────────

## Question 376  (Topic 2)  —  Multiple Choice  [V80.2]

An engineer must establish a trunk link between two switches. The neighboring switch is set to trunk or

desirable mode. What action should be taken?

A.  configure switchport nonegotiate

B.  configure switchport mode dynamic desirable

C.  configure switchport mode dynamic auto

D.  configure switchport trunk dynamic desirable

Answer: C

──────────────────────────────────────────────────

## Question 393  (Topic 2)  —  Multiple Choice  [V80.2]

A network administrator needs to aggregate 4 ports into a single logical link which must negotiate layer 2

connectivity to ports on another switch. What must be configured when using active mode on both sides of

the connection?

A.  802.1q trunks

B.  Cisco vPC

C.  LLDP

D.  LACP

Answer: D

──────────────────────────────────────────────────

## Question 428  (Topic 2)  —  Multiple Choice  [V80.2]

An engineer requires a scratch interface to actively attempt to establish a trunk link with a neighbor switch.

What command must be configured?

A.  switchport mode trunk

B.  switchport mode dynamic desirable

C.  switchport mode dynamic auto

D.  switchport nonegotiate

Answer: C

──────────────────────────────────────────────────

## Question 435  (Topic 2)  —  Multiple Choice  [V80.2]

Refer to Exhibit.

How does SW2 interact with other switches in this VTP domain?

A.  It processes VTP updates from any VTP clients on the network on its access ports.

B.  It receives updates from all VTP servers and forwards all locally configured VLANs out all trunk ports

C.  It forwards only the VTP advertisements that it receives on its trunk ports.

D.  It transmits and processes VTP updates from any VTP Clients on the network on its trunk ports

Answer: C

──────────────────────────────────────────────────

## Question 440  (Topic 3)  —  Multiple Choice  [V80.2]

Which two network actions occur within the data plane? (Choose two.)

A.  Add or remove an 802.1Q trunking header.

B.  Make a configuration change from an incoming NETCONF RPC.

C.  Run routing protocols.

D.  Match the destination MAC address to the MAC address table.

E.  Reply to an incoming ICMP echo request.

Answer: B,D

──────────────────────────────────────────────────

## Question 442  (Topic 3)  —  Multiple Choice  [V80.2]

How does Rapid PVST+ create a fast loop-free network topology?

A.  lt requires multiple links between core switches

B.  It generates one spanning-tree instance for each VLAN

C.  It maps multiple VLANs into the same spanning-tree instance

D.  It uses multiple active paths between end stations.

Answer: A

──────────────────────────────────────────────────

## Question 443  (Topic 3)  —  Multiple Choice  [V80.2]

A network engineer must configure two new subnets using the address block 10 70 128 0/19 to meet these

requirements:

• The first subnet must support 24 hosts

• The second subnet must support 472 hosts

• Both subnets must use the longest subnet mask possible from the address block Which two

configurations must be used to configure the new subnets and meet a

requirement to use the first available address in each subnet for the router interfaces? (Choose two )

A.  interface vlan 1234ip address 10.70.159.1 255.255.254.0

B.  interface vlan 1148ip address 10.70.148.1 255.255.254.0

C.  interface vlan 4722ip address 10.70.133.17 255.255.255.192

D.  interface vlan 3002ip address 10.70.147.17 255.255.255.224

E.  interface vlan 155ip address 10.70.155.65 255.255.255.224

Answer: B,D

──────────────────────────────────────────────────

## Question 481  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Users on existing VLAN 100 can reach sites on the Internet. Which action must the administrator take to

establish connectivity to the Internet for users in VLAN 200?

A.  Define a NAT pool on the router.

B.  Configure static NAT translations for VLAN 200.

C.  Configure the ip nat outside command on another interface for VLAN 200.

D.  Update the NAT INSIDF RANGFS ACL

Answer: B

──────────────────────────────────────────────────

## Question 487  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

All VLANs are present in the VLAN database. Which command sequence must be applied to complete the

configuration?

A.  Interface FastEthernet0/1 switchport trunk native vlan 10 switchport trunk allowed vlan 10,15

B.  Interface FastEthernet0/1 switchport mode trunk switchport trunk allowed vlan 10,15

C.  interface FastEthernet0/1 switchport mode access switchport voice vlan 10

D.  Interface FastEthernet0/1 switchport trunk allowed vlan add 10 vlan 10 private-vlan isolated

Answer: C

──────────────────────────────────────────────────

## Question 492  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Switch A is newly configured. All VLANs are present in the VLAN database. The IP phone and PC A on

Gi0/1 must be configured for the appropriate VLANs to establish connectivity between the PCs. Which

command set fulfills the requirement?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: A

──────────────────────────────────────────────────

## Question 496  (Topic 3)  —  Multiple Choice  [V80.2]

Which Layer 2 switch function encapsulates packets for different VLANs so that the packets traverse the

same port and maintain traffic separation between the VLANs?

A.  VLAN numbering

B.  VLAN DSCP

C.  VLAN tagging

D.  VLAN marking

Answer: C

──────────────────────────────────────────────────

## Question 498  (Topic 3)  —  Multiple Choice  [V80.2]

A Cisco engineer must configure a single switch interface to meet these requirements

• accept untagged frames and place them in VLAN 20

• accept tagged frames in VLAN 30 when CDP detects a Cisco IP phone Which command set must the

engineer apply?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 508  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which switch becomes the root of a spanning tree for VLAN 20 if all li links are of equal speed?

A.  SW1

B.  SW2

C.  SW3

D.  SW4

Answer: C

──────────────────────────────────────────────────

## Question 510  (Topic 3)  —  Multiple Choice  [V80.2]

A network engineer must implement an IPv6 configuration on the vlan 2000 interface to create a routable

locally-unique unicast address that is blocked from being advertised to the internet. Which configuration

must the engineer apply?

A.  interface vlan 2000ipv6 address ffc0:0000:aaaa::1234:2343/64

B.  interface vlan 2000Ipv6 address fc00:0000:aaaa:a15d:1234:2343:8aca/64

C.  interface vlan 2000ipv6 address fe80;0000:aaaa::1234:2343/64

D.  interface vlan 2000ipv6 address fd00::1234:2343/64

Answer: B

──────────────────────────────────────────────────

## Question 516  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The DHCP server and clients are connected to the same switch. What is the next step to complete the

DHCP configuration to allow clients on VLAN 1 to receive addresses from the

DHCP server?

A.  Configure the ip dhcp snooping trust command on the interlace that is connected to the DHCP client.

B.  Configure the ip dhcp relay information option command on the interface that is connected to the DHCP

client.

C.  Configure the ip dhcp snooping trust command on the interface that is connected to the DHCP server.

D.  Configure the Ip dhcp relay information option command on the interface that is connected to the DHCP

server.

Answer: C

──────────────────────────────────────────────────

## Question 517  (Topic 3)  —  Multiple Choice  [V80.2]

Which interface mode must be configured to connect the lightweight APs in a centralized architecture?

A.  WLAN dynamic

B.  management

C.  trunk

D.  access

Answer: D

──────────────────────────────────────────────────

## Question 523  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The following must be considered:

• SW1 is fully configured for all traffic

• The SW4 and SW9 links to SW1 have been configured

• The SW4 interface Gi0/1 and Gi0/0 on SW9 have been configured

• The remaining switches have had all VLANs adde d to their VLAN database

Which configuration establishes a successful ping from PC2 to PC7 without interruption to traffic flow

between other PCs?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 534  (Topic 3)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is asked to insert the new VLAN into the existing trunk without modifying anything previously

configured Which command accomplishes this task?

A.  switchport trunk allowed vlan 100-104

B.  switchport trunk allowed vlan add 104

C.  switchport trunk allowed vlan all

D.  switchport trunk allowed vlan 104

Answer: D

──────────────────────────────────────────────────

## Question 544  (Topic 4)  —  Multiple Choice  [V80.2]

Two switches have been implemented and all interfaces are at the default configuration level. A trunk link

must be implemented between two switches with these requirements:

• using an industry-standard trunking protocol

• permitting VLANs 1 -10 and denying other VLANs

How must the interconnecting ports be configured?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 552  (Topic 4)  —  Multiple Choice  [V80.2]

What is a specification for SSIDS?

A.  They are a Cisco proprietary security feature.

B.  They must include one number and one letter.

C.  They define the VLAN on a switch.

D.  They are case sensitive.

Answer: B

──────────────────────────────────────────────────

## Question 571  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A new VLAN and switch are added to the network. A remote engineer configures OldSwitch and must

ensure that the configuration meets these requirements:

• accommodates current configured VLANs

• expands the range to include VLAN 20

• allows for IEEE standard support for virtual LANs

Which configuration on the NewSwitch side of the link meets these requirements?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: C

──────────────────────────────────────────────────

## Question 608  (Topic 4)  —  Multiple Choice  [V80.2]

How does frame switching function on a switch?

A.  forwards frames to a neighbor port using CDP

B.  modifies frames that contain a known source VLAN

C.  inspects and drops frames from unknown destinations

D.  forwards known destinations to the destination port

Answer: D

──────────────────────────────────────────────────

## Question 616  (Topic 4)  —  Multiple Choice  [V80.2]

What is a function of MAC address learning?

A.  It is enabled by default on all VLANs and interfaces

B.  It increases the potential for MAC address flooding.

C.  It is disabled by default on all interfaces connected to trunks

D.  lt increases security on the management VLAN

Answer: A

──────────────────────────────────────────────────

## Question 623  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is updating the management access configuration of switch SW1 to allow secured, encrypted

remote configuration. Which two commands or command sequences must the engineer apply to the switch?

(Choose two.)

A.  SW1(config)#enable secret ccnaTest123

B.  SW1(config)#username NEW secret R3mote123

C.  SW1(config)#line vty 0 15 SW1(config-line)#transport input ssh

D.  SW1(config)# crypto key generate rsa

E.  SW1(config)# interface f0/1 SW1(confif-if)# switchport mode trunk

Answer: A,C

──────────────────────────────────────────────────

## Question 633  (Topic 4)  —  Multiple Choice  [V80.2]

An engineer is configuring a switch port that is connected to a VoIP handset. Which command must the

engineer configure to enable port security with a manually assigned MAC address of abod-bod on voice

VLAN 4?

A.  switchport port-security mac-address abcd.abcd.abcd

B.  switchport port-security mac-address abed.abed.abed vlan 4

C.  switchport port-security mac-address sticky abcd.abcd.abcd vlan 4

D.  switchport port-security mac-address abcd.abcd.abcd vlan voice

Answer: A

──────────────────────────────────────────────────

## Question 636  (Topic 4)  —  Multiple Choice  [V80.2]

What must be considered for a locally switched FlexConnect AP if the VLANs that are used by the AP and

client access are different?

A.  The APs must be connected to the switch with multiple links in LAG mode

B.  The switch port mode must be set to trunk

C.  The native VLAN must match the management VLAN of the AP

D.  IEEE 802.10 trunking must be disabled on the switch port.

Answer: C

──────────────────────────────────────────────────

## Question 646  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Which configuration establishes a Layer 2 LACP EtherChannel when applied to both switches?

A.  Interface range G1/1 – 1/3 switchport mode trunk channel-group 1 mode active no shutdown

B.  Interface range G1/1 – 1/3 switchport mode access channel-group 1 mode passive no shutdown

C.  Interface range G1/1 – 1/3 switchport mode trunkchannel-group 1 mode desirableno shutdown

D.  Interface range G1/1 – 1/3 switchport mode access channel-group 1 mode on no shutdown

Answer: A

──────────────────────────────────────────────────

## Question 655  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

PC A and the file server. Which commend must be configured on switch A to prevent interruption of other

communications?

A.  switch port trunk allowed vlan 12

B.  switchport trunk allowed vlan none

C.  switchport trunk allowed vlan add 13

D.  switch port trunk allowed vlan remove 10-11

Answer: C

──────────────────────────────────────────────────

## Question 662  (Topic 4)  —  Multiple Choice  [V80.2]

Which type of port is used to connect lo the wired network when an autonomous AP maps two VLANs to its

WLANs?

A.  LAG

B.  EtherChannel

C.  trunk

D.  access

Answer: D

──────────────────────────────────────────────────

## Question 669  (Topic 4)  —  Multiple Choice  [V80.2]

What is a reason to configure a trunk port that connects to a WLC distribution port?

A.  Eliminate redundancy with a link failure in the data path.

B.  Allow multiple VLAN to be used in the data path.

C.  Provide redundancy if there is a link failure for out-of-band management.

D.  Permit multiple VLANs to provide out-of-band management.

Answer: D

──────────────────────────────────────────────────

## Question 692  (Topic 4)  —  Multiple Choice  [V80.2]

The address block 192.168.32.0/24 must be subnetted into smaller networks. The engineer must meet

these requirements:

Create 8 new subnets

Each subnet must accommodate 30 hosts

Interface VLAN 10 must use the last usable IP in the first new subnet. A Layer 3 interface is used.

Which configuration must be applied to the interface?

A.  no switchportip address 192.168.32.30 255.255.255.224

B.  no switchport mode trunkip address 192.168.32.97 255.255.255.224

C.  no switchport mode accessip address 192.168.32.62 255.255.255.240

D.  switchportip address 192.168.32.65 255.255.255.240

Answer: A

──────────────────────────────────────────────────

## Question 710  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The SW1 and SW2 Gi0/0 ports have been preconfigured. An engineer is given these requirements:

• Allow all PCs to communicate with each other at Layer 3.

• Configure untagged traffic to use VLAN 5.

• Disable VLAN 1 from being used.

Which configuration set meets these requirements?

A.  SW1#interface Gi0/1switchport mode trunkswitchport trunk allowed vlan 5,7,9,108switchport trunk native

vlan 5interface Gi0/2switchport mode trunkswitchport trunk allowed vlan 5,7,9,108SW2#interface

Gi0/1switchport mode accessswitchport access vlan 7interface Gi0/7switchport mode trunkswitchport trunk

allowed vlan 7,9,108

B.  SW1#interface Gi0/1switchport mode trunkswitchport trunk allowed vlan 5,7,9,108switchport trunk native

vlan 5interface Gi0/2switchport mode accessswitchport trunk allowed vlan 7,9,108SW2#interface

Gi0/1switchport mode accessno switchport access vlan 1switchport access vlan 7interface Gi0/7switchport

mode trunkswitchport trunk allowed vlan 7,9,108switchport trunk native vlan 5

C.  SW#1 -interface Gi0/1switchport mode trunkswitchport trunk allowed vlan 5,7,9,108switchport trunk

native vlan 5interface Gi0/2switchport mode trunkswitchport trunk allowed vlan 5,7,9,108SW2#interface

Gi0/1switchport mode accessswitchport access vlan 7interface Gi0/7switchport mode trunkswitchport trunk

allowed vlan 5,7,9,108switchport trunk native vlan 5

D.  SW1#interface Gi0/1switchport mode trunkswitchport trunk allowed vian 5,7,9,108interface

Gi0/2switchport mode trunkswitchport trunk allowed vlan 7,9,108SW2#interface Gi0/1switchport mode

trunkswitchport trunk allowed vlan 7interface Gi0/7switchport mode trunkswitchport trunk allowed vlan

5,7,9,108

Answer: C

──────────────────────────────────────────────────

## Question 716  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

SW_1 and SW_12 represent two companies that are merging. They use separate network vendors. The

VLANs on both Sides have been migrated to share IP subnets. Which command sequence must be issued

on both sides to join the two companies and pass all VLANs between the companies?

A.  switchport mode trunkswitchport trunk encapsulation dot1q

B.  switchport mode trunkswitchport trunk allowed vlan allswitchport dot1q ethertype 0800

C.  switchport mode dynamic desirableswitchport trunk allowed vlan allswitchport trunk native vlan 7

D.  switchport dynamic autoswitchport nonegotiate

Answer: A

──────────────────────────────────────────────────

## Question 719  (Topic 4)  —  Multiple Choice  [V80.2]

A network engineer is upgrading a small data center to host several new applications, including server

backups that are expected to account for up to 90% of the bandwidth during peak times. The data center

connects to the MPLS network provider via a primary circuit and a secondary circuit. How does the

engineer inexpensively update the data center to avoid saturation of the primary circuit by traffic associated

with the backups?

A.  Assign traffic from the backup servers to a dedicated switch.

B.  Configure a dedicated circuit for the backup traffic.

C.  Place the backup servers in a dedicated VLAN.

D.  Advertise a more specific route for the backup traffic via the secondary circuit.

Answer: A

──────────────────────────────────────────────────

## Question 723  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

The DHCP server is configured with a DHCP pool for each of the subnets represented. Which command

must be configured on switch SW1 to allow DHCP clients on VLAN 10 to receive dynamic IP addresses

from the DHCP server?

A.  SW1(config-if)#ip helper-address 192.168.20.2

B.  SW1(config-if)#ip helper-address 192.168.20.1

C.  SW1(config-if)#ip helper-address 192.168.10.2

D.  SW1(config-if)#ip helper-address 192.168.10.1

Answer: C

──────────────────────────────────────────────────

## Question 731  (Topic 4)  —  Multiple Choice  [V80.2]

What happens when a switch receives a frame with a destination MAC address that recently aged out?

A.  The switch references the MAC address aging table for historical addresses on the port that received the

frame.

B.  The switch floods the frame to all ports in all VLANs except the port that received the frame

C.  The switch drops the frame and learns the destination MAC address again from the port that received

the frame

D.  The switch floods the frame to all ports in the VLAN except the port that received the frame.

Answer: D

──────────────────────────────────────────────────

## Question 747  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Rapid PVST+ mode is on the same VLAN on each switch. Which switch becomes the root bridge and why?

A.  SW2, because its MAC address is the highest

B.  SW3, because its priority is the highest

C.  SW4, because its priority is highest and its MAC address is lower

D.  SW1, because its priority is the lowest and its MAC address is higher

Answer: B

──────────────────────────────────────────────────

## Question 760  (Topic 4)  —  Multiple Choice  [V80.2]

Which interface IP address serves as the tunnel source for CAPWAP packets from the WLC to an AP?

A.  service

B.  trunk

C.  AP-manager

D.  virtual AP connection

Answer: C

──────────────────────────────────────────────────

## Question 779  (Topic 4)  —  Multiple Choice  [V80.2]

What is a function of an endpoint?

A.  It is used directly by an individual user to access network services

B.  It passes unicast communication between hosts in a network

C.  It transmits broadcast traffic between devices in the same VLAN

D.  It provides security between trusted and untrusted sections of the network.

Answer: A

──────────────────────────────────────────────────

## Question 789  (Topic 4)  —  Multiple Choice  [V80.2]

PC1 tries to send traffic to newly installed PC2. The PC2 MAC address is not listed in the MAC address

table of the switch, so the switch sends the packet to all ports in the same VLAN Which switching concept

does this describe?

A.  MAC address aging

B.  MAC address table

C.  frame flooding

D.  spanning-tree protocol

Answer: A

──────────────────────────────────────────────────

## Question 791  (Topic 4)  —  Multiple Choice  [V80.2]

Which port type does a lightweight AP use to connect to the wired network when it is configured in local

mode?

A.  EtherChannel

B.  LAG

C.  trunk

D.  access

Answer: C

──────────────────────────────────────────────────

## Question 798  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer is configuring a new Cisco switch NewSW, to replace SW2 The details have been provided

• Switches SW1 and SW2 are third-party devices without support for trunk ports

• The existing connections must be maintained between PC1 PC2 and PC3

• Allow the switch to pass traffic from future VLAN 10. Which configuration must be applied?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 875  (Topic 4)  —  Multiple Choice  [V80.2]

What is the default port-security behavior on a trunk link?

A.  It causes a network loop when a violation occurs.

B.  It disables the native VLAN configuration as soon as port security is enabled.

C.  It places the port in the err-disabled state if it learns more than one MAC address.

D.  It places the port in the err-disabled slate after 10 MAC addresses are statically configured.

Answer: A

──────────────────────────────────────────────────

## Question 884  (Topic 4)  —  Multiple Choice  [V80.2]

What is the operating mode and role of a backup port on a shared LAN segment in Rapid PVST+?

A.  forwarding mode and provides the lowest-cost path to the root bridge for each VLAN

B.  learning mode and provides the shortest path toward the root bridge handling traffic away from the LAN

C.  blocking mode and provides an alternate path toward the designated bridge

D.  listening mode and provides an alternate path toward the root bridge

Answer: C

──────────────────────────────────────────────────

## Question 897  (Topic 4)  —  Multiple Choice  [V80.2]

Which protocol is used in Software Defined Access (SDA) to provide a tunnel between two edge nodes in

different fabrics?

A.  Generic Router Encapsulation (GRE)

B.  Virtual Local Area Network (VLAN)

C.  Virtual Extensible LAN (VXLAN)

D.  Point-to-Point Protocol

Answer: C

──────────────────────────────────────────────────

## Question 909  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer assigns IP addressing to the current VLAN with three PCs. The configuration must also

account for the expansion of 30 additional VLANS using the same Class C subnet for subnetting and host

count. Which command set fulfills the request while reserving address space for the expected growth?

A.  Switch(config)#interface vlan 10Switch(config-if)#ip address 192.168.0.1 265 255.255.252

B.  Switch(config)#interface vlan 10Switch(config-if)#ip address 192.168.0.1 255 255.255.248

C.  Switch(config)#interface vlan 10Switch(config-if)#ip address 192.168.0.1 255 255.255.0

D.  Switch(config)#interface vlan 10Switch(config-if)#ip address 192.168.0.1 255.255.255.128

Answer: B

──────────────────────────────────────────────────

## Question 936  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An engineer is building a new Layer 2 LACP EtherChannel between SW1 and SW2.

and they executed the given show commands to verify the work Which additional task must be performed

so that the switches successfully bundle the second member in the LACP port-channel?

A.  Configure the switchport trunk allowed vlan 300 command on SW1 port-channel 1

B.  Configure the switchport trunk allowed vlan 300 command on interface Fa0/2 on SW1.

C.  Configure the switchport trunk allowtd vlan add 300 command on interface FaO 2 on SW2.

D.  Configure the switchport trunk allowtd vlan add 300 command on SW1 port-channel 1

Answer: B

──────────────────────────────────────────────────

## Question 955  (Topic 4)  —  Multiple Choice  [V80.2]

Which two protocols are used by an administrator for authentication and configuration on access points?

A.  Kerberos

B.  802.1Q

C.  802.1x

D.  TACACS+

E.  RADIUS

Answer: D,E

──────────────────────────────────────────────────

## Question 963  (Topic 4)  —  Multiple Choice  [V80.2]

Which command do you enter so that a switch configured with Rapid PVST + listens and learns for a

specific time period?

A.  switch(config)#spanning-tree vlan 1 max-age 6

B.  switch(config)#spanning-tree vlan 1 hello-time 10

C.  switch(config)#spanning-tree vlan 1 priority 4096

D.  switch(config)#spanning-tree vlan 1 forward-time 20

Answer: D

──────────────────────────────────────────────────

## Question 970  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer must configure the interface that connects to PC 1 and secure it in a way that only PC1 is

allowed to use the port No VLAN tagging can be used except for a voice VLAN. Which command sequence

must be entered to configure the switch?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: B

──────────────────────────────────────────────────

## Question 996  (Topic 4)  —  Multiple Choice  [V80.2]

What provides connection redundancy increased bandwidth and load sharing between a wireless LAN

controller and a Layer 2 switch?

A.  VLAN trunking

B.  tunneling

C.  first hop redundancy

D.  link aggregation

Answer: D

──────────────────────────────────────────────────

## Question 1008  (Topic 4)  —  Multiple Choice  [V80.2]

What does a switch do when it receives a frame whose destination MAC address is missing from the MAC

address table?

A.  It floods the frame unchanged across all remaining ports in the incoming VLAN.

B.  It appends the table with a static entry for the MAC and shuts down the port.

C.  It updates the CAM table with the destination MAC address of the frame.

D.  It changes the checksum of the frame to a value that indicates an invalid frame.

Answer: A

──────────────────────────────────────────────────

## Question 1009  (Topic 4)  —  Multiple Choice  [V80.2]

The address block 192 168 32 0/24 must be subnetted into smaller networks The engineer must meet

these requirements

• Create 8 new subnets

• Each subnet must accommodate 30 hosts

• Interface VLAN 10 must use the last usable IP in the first new subnet

• A Layer 3 interface is used

Which configuration must be applied to the interface?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 1021  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

An engineer configures interface fa0/1 on SW1 and SW2 to pass traffic from two different VLANs. For

security reasons, company policy requires the native VLAN to be set to a nondefault value. Which

configuration meets this requirement?

A)

B)

C)

D)

A.  Option A

B.  Option B

C.  Option C

D.  Option D

Answer: D

──────────────────────────────────────────────────

## Question 1024  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

All interfaces are in the same VLAN. All switches are configured with the default STP priorities. During the

STP electronics, which switch becomes the root bridge?

A.  MDF-DC-4:08:E0:19: 08:B3:19

B.  MDF-DC-3:08:0E:18::1A:3C:9D

C.  MDF-DC-08:0E:18:22:05:97

D.  MDF-DC-1:DB:E:44:02:54:79

Answer: C

──────────────────────────────────────────────────

## Question 1032  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

Host A switch interface is configured in VLAN 2. Host D sends a unicast packet destined for the IP address

of host A.

What does the switch do when it receives the frame from host D?

A.  It creates a broadcast storm.

B.  It drops the frame from the MAC table of the switch.

C.  It shuts down the source port and places It In err-disable mode.

D.  It floods the frame out of every port except the source port.

Answer: C

──────────────────────────────────────────────────

## Question 1048  (Topic 4)  —  Multiple Choice  [V80.2]

Refer to the exhibit.

A network engineer is configuring a WLAN to connect with the 172.16.10.0/24 network on VLAN 20. The

engineer wants to limit the number of devices that connect to the WLAN on the USERWL SSID to 125.

Which configuration must the engineer perform on the WLC?

A.  In the Management Software activation configuration, set the Clients value to 125.

B.  In the Controller IPv6 configuration, set the Throttle value to 125.

C.  In the WLAN configuration, set the Maximum Allowed Clients value to 125.

D.  In the Advanced configuration, set the DTIM value to 125.

Answer: C

──────────────────────────────────────────────────

## Question 1060  (Topic 5)  —  Multiple Choice  [V80.2]

Which switch port configuration must be configured when connected to an AP running in FlexConnect

mode, and the WLANs use flex local switching?

A.  access port with one VLAN

B.  trunk port with pruned VLANs

C.  Layer 3 port with an IP address

D.  tagged port with MAC Filtering enabled

Answer: B

──────────────────────────────────────────────────

## Question 1062  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An administrator must connect SW_1 and the printer to the network. SW_2 requires

DTP to be used for the connection to SW_1. The printer is configured as an access port with VLAN 5.

Which set of commands completes the connectivity?

A.  switchport mode dynamic autoswitchport private-vlan association host 5

B.  switchport mode dynamic autoswitchport trunk encapsulation negotiate

C.  switchport mode dynamic desirableswitchport trunk allowed vlan add 5

D.  switchport mode trunkswitchport trunk pruning vlan add 5

Answer: C

──────────────────────────────────────────────────

## Question 1111  (Topic 5)  —  Multiple Choice  [V80.2]

Which type of wired port is required when an AP offers one unique SSID, passes client data and

management traffic, and is in autonomous mode?

A.  Default

B.  Access

C.  Trunk

D.  LAG

Answer: B

──────────────────────────────────────────────────

## Question 1130  (Topic 5)  —  Multiple Choice  [V80.2]

How does MAC learning function?

A.  Sends frames with unknown destinations to a multicast group.

B.  Increases security on the management VLAN.

C.  Rewrites the source and destination MAC address.

D.  Associates the MAC address with the port on which it is received.

Answer: D

──────────────────────────────────────────────────

## Question 1131  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. VLAN 23 is being implemented between SW1 and SW2. The command show interface

ethernet0/0 switchport has been issued on SW1. Ethernet0/0 on SW1 is the uplink to SW2. Which

command when entered on the uplink interface allows PC 1 and PC 2 to communicate without impact to the

communication between PC 11 and PC 12?

A.  switchport trunk allowed vlan 2-1001

B.  switchport trunk allowed vlan add 23

C.  switchport trunk allowed vlan 23

D.  switchport trunk allowed vlan 22-23

Answer: B

──────────────────────────────────────────────────

## Question 1135  (Topic 5)  —  Multiple Choice  [V80.2]

Which type of wired port is required when an AP offers one unique SSID, passes client data and

management traffic, and is in autonomous mode?

A.  LAG

B.  default

C.  trunk

D.  Bopess

Answer: B

──────────────────────────────────────────────────

## Question 1147  (Topic 5)  —  Multiple Choice  [V80.2]

Which command must be configured to enable port security with a manually assigned MAC address of

aabb.cc00.1234 for a VoIP handset on VLAN 4?

A.  switchport port-security mac-address aabb.cc00.1234

B.  switchport port-security mac-address aabb.cc00.1234 vlan 4

C.  mac-address-table static aabb.cc00.1234 vlan 4 interface fa0/1

D.  switchport port-security mac-address sticky

Answer: B

──────────────────────────────────────────────────

## Question 1162  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. A network engineer is adding another physical interface as a new member to the

existing Port-Channel1 bundle.

Which command set must be configured on the new interface to complete the process?

A.  switchport mode trunk channel-group 1 mode active

B.  no switchport channel-group 1 mode active

C.  no switchport channel-group 1 mode on

D.  switchport switchport mode trunk

Answer: B

──────────────────────────────────────────────────

## Question 1170  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. Inter-VLAN routing is configured on SW1. Client A is running Linux as an OS in VLAN

10 with a default gateway IP 10.0.0.1 but cannot ping client B in VLAN 20 running Windows. What action

must be taken to verify that client A has the correct IP settings?

A.  Run the ipconfig command on client A and ensure that the IP address is within the host range of 10.0.0.1

- 10.0.255.254.

B.  Run the ifconfig command on client A to confirm that its IP and subnet mask fall within 255.254.0.0.

C.  Run the ipconfig command on client A to confirm that the correct 10.0.0.1 default gateway is used.

D.  Run the ifconfig command on client A to confirm that the subnet mask is set to 255.255.128.0.

Answer: B

──────────────────────────────────────────────────

## Question 1221  (Topic 5)  —  Multiple Choice  [V80.2]

Refer to the exhibit. An administrator is configuring a new WLAN for a wireless network that has these

requirements:

Dual-band clients that connect to the WLAN must be directed to the 5-GHz spectrum. Wireless clients on

this WLAN must be able to apply VLAN settings from RADIUS attributes.

Which two actions meet these requirements? (Choose two.)

A.  Enable the Aironet IE option.

B.  Enable the Coverage Hole Detection option.

C.  Set the MFP Client Protection option to Required

D.  Enable the client band select option.

E.  Enable the allow AAA Override option

Answer: D,E

──────────────────────────────────────────────────

## Question 1236  (Topic 5)  —  Multiple Choice  [V80.2]

An engineer requires a switch interface to actively attempt to establish a trunk link with a neighbor switch.

What command must be configured?

A.  switchport mode dynamic desirable

B.  switchport mode trunk

C.  switchport nonegotiate

D.  switchport mode dynamic auto

Answer: A